| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0118-v2 |
| --- | --- |

| Title: | EE2-related: Extension of template matching to Affine, CIIP, GPM merge modes, and boundary sub-blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Vadim Seregin Yao-Jen Chang Zhi Zhang Han Huang Yan Zhang Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1 (858) 845-4257 chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com yjchang@qti.qualcomm.com zhizhang@qti.qualcomm.com hanhuang@qti.qualcomm.com yzh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Template matching refinement applied to regular merge mode as being studied in Exploration Experiments 2 (EE2, JVET-U2024) is extended to other merge modes, namely CIIP, GPM, boundary sub-blocks, and Affine. This extension was implemented on top of the EE2 software and was tested according to the procedure described in the EE2 document. As compared with the EE2 tool-off anchor, the experimental results demonstrate 0.33% Y BD-rate reduction, with 118% encoder and 102% decoder runtime, for random access configuration. It is also reported that the Y BD-rate reduction of Class C/D/E in Low Delay B is 0.48%/0.55%/0.91%, respectively.

# Introduction

Templated matching refinement applied to the regular merge mode was proposed in JVET-U0100 and has been studied in the context of EE2. In this contribution, the template matching refinement is extended to Affine, CIIP, GPM, and boundary sub-blocks of a coding unit. Each modification is described in detail in the subsequent sections.

# CIIP merge mode with template matching refinement

Template matching refinement is applied to the underlying inter-prediction part of CIIP merge mode. When CIIP is enabled for a coding unit (CU), it is signaled to indicate whether template matching refinement is enabled for this CU. When both CIIP and template matching refinement are enabled, the MVs of a merge candidate used in CIIP merge mode is refined by using the same template matching refinement process as in the template matching of EE2 before motion compensation and inter-intra combination.

# GPM merge mode with template matching refinement

Template matching refinement is applied to geometric partitioning merge (GPM) mode. When GPM mode is enabled for a CU, a flag (tm_merge_flag) is signaled to control whether template matching refinement is applied for this CU. If the refinement is utilized, the selected uni-directional MV for each GPM partition is refined by using the same template matching refinement process as in the template matching of EE2 before motion compensation.

# Boundary sub-blocks with template matching refinement

Template matching refinement is applied to boundary 4x4 sub-blocks of a CU, when it is coded using regular merge mode, tm_merge_flag is enabled, and the merge candidate motion is uni-directional. This extension is an additional refinement step applied after the CU-level template matching refinement (EE2, Test-3.3), where the block motion is further refined only for the 4x4 sub-blocks at the top or left boundary in the CU. In this process, since the CU is uni-directional (and so are its sub-blocks), the MV of each aforementioned boundary sub-block is converted to bi-prediction MVs utilizing uni-to-bi conversion similar to the one used in the template matching of EE2 followed by the template matching refinement applied to every boundary sub-block.

# Affine merge mode with template matching refinement

Template matching refinement is applied to the affine merge mode (Affine TM), and the tm_merge_flag is signaled to differentiate whether template matching is applied. In this mode, the template matching search is done around the initially given CPMVs of an affine merge candidate indicated by a merge index. The current block template is the same as in the regular TM applied to translational motion model, but the reference block template comprises multiple 4x4 sub-blocks pointed respectively to by the CPMV-derived MVs of the neighboring sub-blocks (i.e., A0, A1, …, and L0, L1, … as shown on Figure 1) at the CU boundary. The search process of Affine TM starts from the CPMV0, while keeping the other CPMV(s) constant, and the search is performed toward horizontal and vertical directions, followed by diagonal ones only if zero vector is not the best difference vector found from the horizontal/vertical search. Then, Affine TM repeats the same search process for CPMV1, and CPMV2, if 6-parameter model is used. Based on the refined CPMVs, the whole search process restarts from CPMV0, if zero vector is not the best difference vector from the previous iteration and the search process has iterated less than 3 times.

| (a) | (b) |
| --- | --- |

- Current block templates (a) and the reference sub-block based templates (b).

# Experimental results

The described methods are implemented on top EE2 software (https://vcgit.hhi.fraunhofer.de/jvet-u-ee2/VVCSoftware_VTM/-/tree/U0100) and are tested as described in the EE2 document (JVET-U2024). The proposed methods are evaluated together with the EE2 methods, the test results are summarized in the below table.

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.59% | -0.52% | 115% | 100% | x | x | x | x | x |
| Class A2 | -0.42% | -0.39% | -0.51% | 120% | 105% | x | x | x | x | x |
| Class B | -0.29% | -0.40% | -0.44% | 118% | 100% |  |  |  |  |  |
| Class C | -0.32% | -0.44% | -0.52% | 120% | 102% | -0.48% | -0.57% | -0.79% | 130% | 121% |
| Class E | x | x | x | x | x | -0.91% | -0.84% | -0.78% | 127% | 107% |
| Overall | -0.33% | -0.45% | -0.49% | 118% | 102% |  |  |  |  |  |
| Class D | -0.23% | -0.05% | -0.25% | 116% | 101% | -0.55% | -1.05% | -0.90% | 125% | 122% |
| Class F | -0.09% | -0.08% | -0.03% | 112% | 100% | -0.33% | -0.48% | 0.15% | 123% | 103% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).