| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0119-v1 |
| --- | --- |

| Title: | AHG12: LFNST extension with large kernel |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jane Zhao Seung-Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA Address line 2 | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the VVC standard, two types of LFNST kernels, which are commonly called LFNST4 and LFNST8, are incorporated. LFNST4 has a form of 16x16 matrix and is applied to 4xN/Nx4 blocks. LFNST8 has a form of 16x48 matrix and applied to the larger block size which is not coved by LFNST4.  In this contribution, a new LFNST16 kernel is proposed to deal with larger blocks whose width and height are greater than or equal to 16. LFNST16 has a form of 32x96 matrix. In addition, a new LFNST8 architecture with 32x64 matrix is introduced. In Test A, the LFNST16 is tested, and both the LFNST16 and new LFNST8 (32x64) are applied in Test B as follows;

Test A:  (LFNST4, LFNST8, LFNST16*) = (16x16, 16x48, 32x96)

Test B:  (LFSNT4, LFNST8*, LFNST16*) = (16x16, 32x64, 32x96)

VVC:   (LFNST4, LFNST8) = (16x16, 16x48)

In the above tests, the following three aspects are configured in common: 1) the total number of LFNST sets is 36, one of which is dedicated for WAIP extended modes, 2) each LFNST set consists of three kernels, and 3) the worst case multiplication count per sample is 16.

The AI results are reportedly observed that the BD-rates are reduced by 0.86% and 1.04% with 117% and 120% encoding time changes for Test A and B, respectively relative to VTM-12.0 anchor.

The RA results are reportedly observed that the BD-rates are reduced by #.##% and #.##% with ###% and ###% encoding time changes for Test A and B, respectively relative to VTM-12.0 anchor.

Proposed architecture

Basically, the forward LFNST is applied to top-left low frequency region. In this document, this region is called Region-Of-Interest (ROI). As in the VVC standard, primary-transformed coefficients that exist in the region other than ROI are zeroed out, when LFNST is applied.

ROI for LFNST16 and LFNST8

The ROI for LFNST16 is depicted in Figure 1. It consists of six 4x4 sub-blocks, which are consecutive in scan order. Since the number of input samples is 96, transform matrix for forward LFNST16 can be Rx96. R is chosen to be 32 in this contribution, 32 coefficients (two 4x4 sub-blocks) are generated from forward LFNST16 accordingly, which are placed following coefficient scan order.

Figure 1  The ROI for LFNST16

The ROI for LFNST8 in Test B is shown in Figure 2(b). Compared with Figure 2(a) that corresponds to VVC LFNST8 case, the bottom-right 4x4 sub-block is further included. The forward LFNST8 matrix can be Rx64 and R is chosen to be 32 in this contribution. The generated coefficients are located in the same manner as with LFNST16.

Figure 2  The ROI for LFNST8 in the case of (a) Test A and (b) Test B

The configuration of LFNST sets

The total number of LFNST sets is 36 and the mapping from intra prediction modes to these sets is shown in Table 1. The LFNST sets labelled from 2 to 34 are mapped in a symmetric manner, and the 35th set is assigned to all extended wide angle modes. Each LFNST set consists of three kernels, and LFNST4, LFNST8, and LFNST16 have the same number of LFNST sets and kernels per set.

Table 1 The LFNST set mapping in Test A/B

The worst case computational complexity: 16 multiplications per sample

In the VVC standard, the maximum required multiplication count per sample for LFNST is 8. In this contribution, the worst case computational complexity for LFNST is increased to be 16. In the case of LFNST4, the generation of 8 coefficients, i.e. zeroing-out of half coefficients, for a 4x4 TB is disabled. When 16x48 is applied as LFNST8 in Test A, the same kind of zeroing-out is also disabled for an 8x8 TB. But, in Test B, the zeroing-out is still performed in order not to violate 16 multiplications per sample. When it comes to LFNST16, the zeroing-out is not required, because it is applied only to larger blocks.

Experimental results

The proposed methods have been implemented based on VTM 12.0 reference software according to common test conditions defined in JVET-T2010 [1].

Test A:  (LFNST8, LFNST16) = (16x48, 32x96)

Test B:  (LFNST8, LFNST16) = (32x64, 32x96)

Conclusion

In this contribution, a large kernel, i.e. LFNST16, of which dimension is 32x96 was proposed, and two architectures on top of the large kernel were tested. One has a configuration of (16x48, 32x96) as (LFNST8, LFNST16), which showed 0.86% and #.##% BD-rate reductions with 117% and ###% encoding time changes for AI and RA, respectively. The other has a configuration of (32x64, 32x96) as (LFNST8, LFNST16), which showed 1.04% and #.##% BD-rate reductions with 120% and ###% encoding time changes for AI and RA, respectively. It is recommended the proposed large kernel based architecture to be investigated through EE.

References

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, 20th meeting, by teleconference, 7 – 16 Oct. 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).