| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0132-v3 |
| --- | --- |

| Title: | VVC software decoder implementation for mobile devices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | L. Yu, Z. Cui, L. Wang, J. Chen, Z. Huang, L. Xu | Email: | zhuangshu.wlb@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This input document is a follow-up of JVET-U0088 which introduces Ali266, a software VVC decoder optimized specifically for the mobile platform by Alibaba. In JVET-U0088 Ali266 was claimed to occupy only 30 MB of memory when decoding 720p video in real time and to achieve real-time decoding for coded bit rates up to 7 Mbps for the 4K content; and for 1080p content, Ali266 achieves real-time decoding for most cases using only two threads. Ali266 performs and real-time decoding of e-commerce content with one or two threads on a wide variety of phones. However, the claimed performance was achieved by disabling ALF or CCALF and setting internal bit depth to 8.

In this document, additional results are provided with both ALF and CCALF on, the updated decoder requires 33 MB memory occupation and binary size of less than 1 MB, both are slight increases as compared to the decoder reported in JVET-U0088. Meanwhile the decoding speed decreases 33.78% in average for CTC sequences in order to support the ALF and CCALF. The new Ali266 get 15x acceleration ratio compare to VTM-11.0 on Android P40 with 8 threads with CTC classA and class B, and still perform real-time decoding of e-commerce content on most phones with 3 threads.

Introduction

The first version of Ali266 as presented in JVET-U0088 did not support ALF and CCALF due to lack of development time. In this contribution, the support for ALF and CCALF (denoted jointly as ALF for simplicity in the following of this document) is added to Ali266 with Arm Neon optimization; speed tests results on a wide range of mobile devices are reported.

Results

The performance of Ali266 in terms of decoding speed is presented in this section. We used two types of source content: the JVET CTC sequences, and twenty Taobao e-commerce sequences in 720p resolution. The bitstreams used to measure the speed performance are generated using VTM-11.0 with the following configuration modifications:

- Internal bit depth set to 8 bits

For the JVET CTC sequences, the same fixed QP configuration as in JVET CTC, QP = {22, 27, 32 37}, is used. For the Taobao e-commerce 720p sequences, we selected QPs to match three bit rates, 0.6 Mbps, 1.0 Mbps, and 1.5 Mbps. These rates represent target rates that we expect for VVC bitstreams in real-world applications.

Speed test: JVET CTC sequences

The following two devices are used to test Ali266 decoding speed on the JVET CTC sequences:

- Android: Huawei P40 , Oneplus 8T

- iOS: iPhone 12 Pro Max

Note that those devices are the same as used to run the simulations for JVET-U0088 except Oneplus 8T.

Table 1 and Table1.1 shows the average decoding speed of the new Ali266 and VTM-11.0. For Ali266, decoding speed is shown for both platforms; while for VTM-11.0, Android platform was used to collect the speed data. Taking 4K sequences as an example, on Huawei P40, compared to VTM-11.0, Ali266 in single thread mode achieves 4.1x decoding speed-up, and 15x speedup with 8 threads.

Table 1. Average decoding speed (fps) of Ali266 and VTM-11.0 for the JVET CTC sequences on

Android Device Huawei P40 and Oneplus 8T

|  | VTM-11.0 * | Ali266 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Huawei P40 |  |  |  | Oneplus 8T |  |  |  |
| # threads | 1 | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 6 |
| Class A | 1.3 | 5.3 | 10.0 | 17.8 | 19.4 | 6.30 | 11.38 | 20.44 | 21.01 |
| Class B | 5.3 | 22.3 | 42.0 | 74.1 | 75.8 | 26.32 | 46.99 | 83.49 | 78.36 |
| Class C | 24.9 | 101.8 | 187.3 | 309.2 | 273.6 | 122.86 | 210.29 | 340.00 | 277.96 |
| Class D | 93.2 | 365.8 | 610.7 | 850.8 | 595.4 | 435.45 | 674.85 | 886.77 | 585.57 |

* VTM-11.0 speed was tested on Huawei P40

Table 1.1. Average decoding speed (fps) of Ali266 and VTM-11.0 for the JVET CTC sequences on iPhone 12 Pro Max

|  | Ali266 |  |  |  |
| --- | --- | --- | --- | --- |
|  | iPhone 12 Pro Max |  |  |  |
| # threads | 1 | 2 | 4 | 6 |
| Class A | 11.38 | 17.89 | 23.59 | 28.70 |
| Class B | 49.84 | 83.29 | 109.78 | 130.38 |
| Class C | 226.23 | 404.09 | 474.56 | 505.83 |
| Class D | 755.48 | 1295.40 | 1276.04 | 1392.82 |

Table 2 shows the decrease of decoding speed (fps), in percentage, compare to Ali266 V1 without ALF support.

In average, the decoding speed, decreases 36.7% for VTM when ALF is enabled on Huawei P40. Similarly, the decoding speed decreases 32.36% for Ali266 in HuaweiP40, and 33.96% in iPhone 12 Pro Max. So the average decoding speed decrease is 33.78% for Ali266 when ALF is turned on.

Table 2. Average decoding speed (fps) of Ali266 and VTM-11.0 with ALF on/off for the JVET CTC sequences

|  |  | VTM-11.0 * | Ali266 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Huawei P40 |  |  |  | iPhone 12 Pro Max |  |  |  |
| # threads | 1 |  | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 6 |
| Class A | 40.91% |  | 31.15% | 32.13% | 33.58% | 38.29% | 36.78% | 42.29% | 43.83% | 42.60% |
| Class B | 38.90% |  | 28.35% | 30.16% | 32.64% | 40.76% | 35.27% | 40.51% | 38.67% | 38.79% |
| Class C | 35.15% |  | 25.00% | 26.53% | 32.23% | 45.63% | 27.72% | 29.97% | 30.52% | 33.53% |
| Class D | 31.81% |  | 17.90% | 23.34% | 37.43% | 50.78% | 21.79% | 19.89% | 26.07% | 35.10% |
| Avg | 36.69% |  | 25.60% | 28.04% | 33.97% | 43.87% | 30.39% | 33.17% | 34.77% | 37.51% |

Figure 1 shows the CPU percentage with ALF Neon acceleration ON and OFF respectively. The acceleration ratio is roughly 4 on average for the ALF module with NEON optimization.

| ALF with Neon (1080p qp22, RA) | ALF w/o Neon (1080p qp22, RA) |
| --- | --- |

Figure 1 CPU percentage with ALF.

Speed test: E-commerce sequences

Ali266 is also tested on a total of twenty typical e-commerce 720p sequences from Taobao (denoted as TaoSequences); seven of these are in landscape mode (1280x720), and the other thirteen of these are in portrait mode (720x1280). Such choice is to reflect the real-world popularity of taking portrait video on mobile devices.

For the TaoSequence, a device with the latest hardware, a device released a few years ago whose hardware is considered mid-level nowadays, and a device released a few years ago as an affordable model are selected for the test. The speed results on these three device tiers are shown in Table 3, Table 4, and Table 5  respectively. Detailed decoding speed data is provided in the accompanying excel file.

According to the results, for real-world e-commerce applications, Ali266 can achieve real-time decoding using single thread on the latest models, 2 threads on the mid-tier models, and 3 threads on the most affordable models.

Table 3. Decoding speed (fps) of Ali266 on Android device with the latest hardware for TaoSequences

| # threads |  | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- | --- |
| Bit rates | 0.6 Mbps | 73.3 | 132.0 | 221.2 | 205.5 |
|  | 1.0 Mbps | 59.8 | 108.1 | 183.6 | 171.5 |
|  | 1.5 Mbps | 53.8 | 97.5 | 166.7 | 157.0 |
| Average |  | 62.3 | 112.5 | 190.5 | 178.0 |

Table 4. Decoding speed (fps) of Ali266 on Android device with mid-level hardware for TaoSequences

| # threads |  | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- | --- |
| Bit rates | 0.6 Mbps | 32.2 | 57.5 | 98.6 | 113.0 |
|  | 1.0 Mbps | 26.3 | 47.4 | 82.7 | 95.9 |
|  | 1.5 Mbps | 23.7 | 42.9 | 75.1 | 88.2 |
| Average |  | 27.4 | 49.2 | 85.5 | 99.0 |

Table 5. Decoding speed (fps) of Ali266 on Android phone with affordable hardware for TaoSequences

| # threads |  | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- | --- |
| Bit rates | 0.6 Mbps | 14.8 | 26.9 | 48.3 | 75.0 |
|  | 1.0 Mbps | 12.5 | 22.4 | 41.0 | 64.7 |
|  | 1.5 Mbps | 11.3 | 20.3 | 37.3 | 59.2 |
| Average |  | 12.9 | 23.2 | 42.2 | 66.3 |

Conclusions

The VVC decoder implementation in JVET-U0088 did not support ALF and CCALF. In this proposal, we make the implementation more complete by supporting both of them. Although with some sacrifice of decoding speed, test results still show that Ali266 achieves real-time decoding for all mobile device tiers on e-commerce content without needing to run at full capacity of multi-threading.

References

- J. Chen, L. Wang, R.-L. Liao, Y. Ye, “VVC software decoder implementation for mobile devices”, JVET-U0088, Joint Video Experts Team(JVET), January 2021