| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0144 |
| --- | --- |

| Title: | Crosscheck of CE-4.4 from JVET-V0051 (CE-4.3 and CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the cross-check results for CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2 as reported in JVET-V0051. The cross-check results match the coding performance reported by the proponent.

# Results

The source code for CE-4.4 was obtained from the CE git repository. The software is built based on VTM-12.0. The crosscheck results are illustrated as following tables. The detailed results could be seen in the attached excel files.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.23% | -1.78% | -1.92% | -1.23% | -1.81% | -1.94% | 100% | 94% |
| PQ422 | -1.05% | -1.35% | -1.45% | -1.05% | -1.35% | -1.44% | 100% | 94% |
| Overall | -1.14% | -1.57% | -1.68% | -1.14% | -1.58% | -1.69% | 100% | 94% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.67% | -0.72% | -0.80% | -0.68% | -0.72% | -0.80% | 100% | 99% |
| PQ422 | -0.59% | -0.54% | -0.60% | -0.59% | -0.53% | -0.59% | 100% | 99% |
| Overall | -0.63% | -0.63% | -0.70% | -0.64% | -0.63% | -0.69% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.58% | -0.66% | -0.72% | -0.58% | -0.65% | -0.72% | 101% | 98% |
| PQ422 | -0.54% | -0.51% | -0.58% | -0.53% | -0.50% | -0.57% | 100% | 99% |
| Overall | -0.56% | -0.58% | -0.65% | -0.56% | -0.57% | -0.65% | 100% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.50% | -0.86% | -0.87% | 109% | 93% |
| HLG422 | -0.44% | -0.76% | -0.78% | 103% | 95% |
| Overall | -0.47% | -0.81% | -0.83% | 106% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.61% | -0.87% | -0.89% | 102% | 96% |
| HLG422 | -0.53% | -0.63% | -0.69% | 101% | 96% |
| Overall | -0.57% | -0.75% | -0.79% | 102% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.50% | -0.75% | -0.80% | 101% | 97% |
| HLG422 | -0.41% | -0.58% | -0.64% | 100% | 97% |
| Overall | -0.45% | -0.67% | -0.72% | 101% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.80% | -0.54% | -0.54% | 107% | 101% |
| SVT12 | -0.68% | -1.05% | -1.07% | 95% | 90% |
| Overall | -0.74% | -0.80% | -0.80% | 101% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.26% | -0.28% | -0.27% | 103% | 100% |
| SVT12 | -0.70% | -0.58% | -0.59% | 98% | 93% |
| Overall | -0.48% | -0.43% | -0.43% | 101% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.10% | -0.36% | -0.35% | 100% | 101% |
| SVT12 | -0.54% | -0.58% | -0.60% | 97% | 94% |
| Overall | -0.32% | -0.47% | -0.47% | 98% | 97% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.06% | -0.03% | -0.01% | -0.41% | -0.06% | 0.00% | -0.47% | -0.26% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | -0.12% | 0.13% | 100% | 100% |
| Overall | 0.06% | -0.03% | -0.01% | -0.41% | -0.06% | 0.00% | -0.30% | -0.07% | 100% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.08% | -0.01% | -0.01% | 0.14% | -0.02% | -0.01% | 0.15% | -0.07% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | -0.03% | 0.03% | 100% | 100% |
| Overall | -0.08% | -0.01% | -0.01% | 0.14% | -0.02% | -0.01% | 0.06% | -0.02% | 100% | 99% |

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | 0.03% | 0.02% | 0.02% | 102% | 99% |
| RGB 4:4:4 - TGM 720p | -0.24% | 0.03% | -0.08% | 103% | 98% |
| Overall | -0.10% | 0.03% | -0.03% | 102% | 98% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.67% | -0.66% | -0.68% | 103% | 100% |
| RGB 4:4:4 - TGM 720p | -1.87% | -1.51% | -2.31% | 103% | 100% |
| Overall | -1.27% | -1.09% | -1.49% | 103% | 100% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.19% | -0.17% | -0.16% | 102% | 102% |
| RGB 4:4:4 - TGM 720p | 0.20% | -1.97% | -0.07% | 102% | 102% |
| Overall | 0.00% | -1.07% | -0.12% | 102% | 102% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-V0051.