| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural Network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Jianle Chen Franck Galpin Jonathan Pfaff Shanshe Wang Zhao Wang Mathias Wien Ping Wu Jizheng Xu | Email: | elena.alshina@huawei.com shanl@tencent.com asegall@sharplabs.com cjianle@qti.qualcomm.com Franck.Galpin@InterDigital.com firstname.lastname@hhi.fraunhofer.de sswang@pku.edu.cn baixiu.wz@alibaba-inc.com wien@lfb.rwth-aachen.de pwuzte@gmail.com xujizheng@bytedance.com |
| Source: | AHG 11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural Network-based video coding between the 22nd meeting (20 – 28 April 2021) and the 23rd meeting (7 – 16 July 2021), both held by teleconference.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Finalize, conduct and discuss the EE on neural network-based video coding JVET-V2023.

- Solicit input contributions on NN-based video coding technologies.

- Refine the test conditions for neural network-based video coding, and develop supporting software as needed.

- Investigate technical aspects specific to NN-based video coding, such as encoding and decoding complexity of neural networks, design network representation, operation, tensor, on-the-fly network adaption (e.g. updating during encoding) etc;

- Study the impact of training (including the impact of loss function) on the performance of candidate technology, and identify suitable video materials for training.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Refine testing methods for assessment of the effectiveness and complexity of considered technology.

- Study the impact of parameter quantization and fixed-point computations in NN-based video coding.

- Review the outcome of the expert viewing conducted at the meeting, refine the methodology, and prepare viewing for the next meeting.

- Generate and distribute anchor encodings and develop improvements of the JVET-V2016 common test conditions for NNVC technology.

- Coordinate with other relevant groups, including SC29/AG5 on visual quality assessment.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email exchange with AHG11 included in the subject lines. Two emails were exchanged on the reflector.

## EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding. This was accomplished via the reflector. And, the final version of the EE description was announced on May 14, 2021.

A summary report for the EE is available at this meeting as:

| JVET-W0023 | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding | E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang |
| --- | --- | --- |

## CTC Refinement and Support

The AHG refined and released the CTC test conditions on May 27, 2021. (The CTC was updated on June 5, 2021, but the only change was in the document header.)

Additionally, to better support and automate the calculation of complexity metrics, the AHG created a script that computes the number of additions and multiplications from either a pytorch or tensorflow model. The script was made available on the AHG git repository and can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/scripts

Furthermore, to support the decision at the previous meeting to update the NNVC anchor to include JVET-V0056, two steps were taken:

- A repository was created that included VTM-11.0 with the JVET-V0056 backport. This repository was then referenced as the anchor within the CTC document.  The tagged repository can be found at: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- A patch to apply the JVET-V0056 backport to VTM-11.0 was created and also distributed via the AhG git repository. This patch should be useful for participants who have existing software that is based on VTM-11.0 and would like to incorporate the JVET-V0056 technoloyg. The patch can be found at: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

Finally, several bugs in the Excel reporting template were addressed. For example, the rate-distortion curve functionality has been broken in earlier versions but was fixed during the AhG period.

The AHG11 coordinators would like to thank Franck Galpin, Christopher Hollmann, Jacob Ström for their generous contributions.

## Anchor Encoding

Anchors for the NN-based video coding activity were created and released on June 4, 2021, within the EE activity. The anchors were also made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master

It is noted that the anchor data includes performance data for QP points 22, 27, 32, 37, 42 and 47.  The largest QP point was generated to support the EE test on super-resolution methods.

## Coordination with SC29/AG5

The AHG coordinated with SC29/AG5 to prepare a viewing procedure for EE contributions.  Sequences were selected for three proposals included in the EE, with each proponent selecting three sequences/QP pair that illustrated the visual benefit of the proposal. These selections emphaszied the RA configurations.

With close coordination with SC29/AG5, it is intended to perform remote viewing sessions to understand the visual benefit of the approaches. This will tentatively be done by viewing each sequence multiple times, in order to give participants the opportunity to focus and evaluate different aspects of the sequences.

## Technical Evaluation

The AHG made meaningful progress on the mandate to evaluate and quantify potential NN based video coding technologies. A summary of the non-EE contribution provided as input to the 23rd meeting is provided below:

|  | Performance | Dec.T. | Features | #params | kMAC /pixel |
| --- | --- | --- | --- | --- | --- |
| JVET-W0057 | RA(BC classes): 3.0%(Y), 6%(Ch.) | ×500 | fine-tuning per content, CTU level Luma/Chroma independent on/off; input: 128x128 YUV444 + QPmap, FineTune per video sequence (only Bias); LZMA2 for updated weights compression | 0.15M (323 variable) | 151 (SmallNN) 9469(BigNN) |
| JVET-W0100 | RA :12 % (Y) 27% (Ch.) AI: 9 % (Y) 22% (Ch.) | ×1000 (RA) ×545(AI) | Picture on/off control + CTU level model selection (not in AI); Filter dictionary dependes on QP, filter switch granu;larity - on resolution and QP. Number resBlocks 32. New: Attention residual block = Pre & Rec Frames --> CONV; ReLu;CONV | 528M (24*22M) | 1430 |
| JVET-W0101 | RA (BC):10 % (Y) 25% (Ch.) AI: 7 % (Y) 20% (Ch.) | ×225 (RA) ×255 (AI) | JVET-W0100 but on top of ECM-1.0 | 528M (24*22M) | 1430 |
| JVET-W0081 | RA : 1.7 % (Y) --> 1.6% AI: 3.3 % (Y)--> 2.9% | ×64--> 61(AI) | Lower complexity version of JVET-T0073 and JVET-V0105; 16 bits version provided | ~10M | 11~100-->6~11 |
| JVET-W0059 | AI (CD): 5 % (Y) |  | After SAO, before ALF; 5 models, switched based on QP, 16 residual blocks, Luma: 160´160 support (CTU + 16 samples on a border); Chroma up-sampled and used as additional input for Luma filtering Chroma: 80´80 support (CTU + 8 samples on a border); Luma down-sampled and used as additional input for Chroma filtering | 5*(1.4+0.7)M | 380000(?) |
| JVET-W0099 | AI(4K):12%(Y); drop 5% (Ch) |  | Input: Predicted Frame, reconstructed Frame, QP map. 16 ResBlocks; L1 loss; Luma as extra input for Chroma | 4.8(Y)+1.1(Ch) M | 5770 (4400 Y + 1370 Ch) |
| JVET-W0111 | AI: 0.3 % (Y) 0.5% (Ch.) | ×40 (AI) | Luma ¯& 2D Transformation --> 3 channes; Ch 2*1D Tr -->4 channels; 7 channels input --> NN with 8 ResBlocks (3 CONV each); "skip connection " through 5x5 CONV. | 0.15M | 150 |
| JVET-W0113 | RA : 3.5 % (Y) 13% (Ch.)% Al: 4.8%(Y) 11% (Ch.) | ×800 (RA) ×439 (AI) | Replaces Deblock and SAO; 8 ResBlocks; (3 CONV each); "skip connection " through 5x5 CONV. | 8*2.3 M | 37 |
| JVET-W0131 | RA : 5.8 % (Y) 5.9% (Ch.)% Al: 4.8%(Y) 5.7% (Ch.) | x41 (RA) x23 (AI) | Replaces Deblock, 3x3 and 1x1 conv layer and leaky ReLU | 4 * 0.107M | 33.6 |
| JVET-W0132 | RA(4K)´2¯:4%(Y); 4% (Ch) RA(4K)´1.5¯:2.5%(Y); (drop)1% (Ch) |  | Different variats of up-sampling filters of "simlified ESRGAN " architecture: CONV*PixelShufflePReLu and just biCubic --> no big difference in performance | 0.5M (PixelShuffle) 0.3M(BiCubic) |  |

# Input contributions

There are 17 input contriubtions related to the AHG mandates. Seven of the contributions are directly relatd to the EE activity, while the remaining 10 contributions are related to AHG11 but not directly part of the EE. The list of input contributions is provided below.

## EE Related Input Contributions

| JVET-W0062 | EE1: SSIM based CNN model for in-loop filtering | T. Ouyang, Y. Guo, Z. Chen (WHU), X. Xu, S. Liu (Tencent) |
| --- | --- | --- |
| JVET-W0063 | EE1.2.2: NN-based super resolution (JVET-V0073) | T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-W0105 | EE1-2.3: Neural Network-based Super Resolution | A. M. Kotra, K. Reuzé, J. Chen, H. Wang, M. Karczewicz, J. Li (Qualcomm) |
| JVET-W0125 | EE1-related: Tests on Decomposition, Compression and Synthesis (DCS)-based Technology (JVET-V0149) | Ming Lu, Zhan Ma (NJU), Zhenyu Dai, Dong Wang (OPPO) |
| JVET-W0130 | EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer | H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) |
| JVET-W0151 | EE1: neural network based in-loop filter using depthwise separable convolution and regular convolution | L. Wang, S. Lin, X. Xu, S. Liu, C. Auyeung, X. Li (Tencent) |

## Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-W0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall, J. Chen, F. Galpin, J. Pfaff, S. S. Wang, Z. Wang, M. Wien, P. Wu, J. Xu |
| Loop Filtering |  |  |
| JVET-W0057 | AHG11: Content-adaptive neural network post-processing filter | M. Santamaria, Y.-H. Lam, J. Lainema, F. Cricri, R. Ghaznavi-Youvalari, H. Zhang, A. Zare, H. R. Tavakoli, M. Hannuksela (Nokia) |
| JVET-W0059 | AHG11: A Deep In-Loop Filter Method | X. Zhang, C. Fang, D. Jiang, J. Lin (Dahua) |
| JVET-W0100 | AHG11: Deep In-Loop Filter with Adaptive Model Selection and External Attention | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-W0101 | AHG11 & AHG12: Deep In-Loop Filter with Adaptive Model Selection and External Attention for Enhanced Compression Beyond VVC Capability | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-W0113 | AHG11: neural network based in-loop filter | L. Wang, W. Jiang, X. Xu, S. Liu (Tencent) |
| Super Resolution |  |  |
| JVET-W0099 | AHG11: CNN-based Super Resolution for Video Coding Using Decoded Information | C. Lin, L. Zhang, K. Zhang, Y. Li (Bytedance) |
| JVET-W0131 | EE1-related: Neural Network-based in-loop filter with constrained computational complexity | H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) |
| JVET-W0132 | AHG11: 1.5x/2.0x Upsample method for NN-Based Super-Resolution Post-Filters | Y. Yasugi, T. Chujoh, T. Ikai (Sharp) |
| Intra Prediction |  |  |
| JVET-W0081 | AHG11: BD-rate gains vs complexity of NN-based intra prediction | T. Dumas, F. Galpin, P. Bordes, F. Leleannec (InterDigital) |
| JVET-W0111 | AHG11: neural network based cross-component prediction model | L. Wang, S. Lin, R. Chang, X. Xu, S. Liu (Tencent) |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Perform viewing of the neural network-based video coding tools during the meeting, in coordination with SC29/AG5.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.