| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0012-v2 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@tencent.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 22nd meeting by teleconference, 20-28 April 2021, and the 23rd meeting by teleconference, 7-16 July 2021.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Study the performance and complexity tradeoff of these video coding tools.

- Define a common software platform for developing and evaluating video coding tools with promising compression performance.

- Refine test conditions in JVET-V2017, generate anchors, identify new test sequences to be added, especially high resolution ones in 8K, in coordination with AHG4.

- Investigate methods to reduce simulation time.

- Analyse the results of exploration experiments described in JVET-V2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-V2024). The first indication of the achievable improvement over VVC are the results of the EE2 software base (ECM-1.0) as agreed at JVET-V meeting. The combined improvement of the ECM-1.0 over VTM-11.0 with the improved MCTF from JVET-V0056 for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.78% | -9.67% | -10.61% | 219% | 216% |
| Class A2 | -5.47% | -7.85% | -5.90% | 212% | 200% |
| Class B | -5.00% | -7.15% | -7.52% | 246% | 213% |
| Class C | -6.04% | -7.43% | -7.71% | 247% | 198% |
| Class E | -6.20% | -7.84% | -6.72% | 251% | 241% |
| Overall | -5.64% | -7.86% | -7.67% | 236% | 212% |
| Class D | -4.86% | -5.53% | -5.56% | 251% | 200% |
| Class F | -10.03% | -11.63% | -12.07% | 214% | 258% |
| Class TGM | -14.61% | -16.09% | -15.97% | 213% | 282% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -12.21% | -13.33% | -15.84% | 279% | 453% |
| Class A2 | -13.45% | -14.39% | -14.09% | 268% | 546% |
| Class B | -11.52% | -12.24% | -11.79% | 271% | 481% |
| Class C | -13.26% | -13.23% | -13.39% | 266% | 415% |
| Class E |  |  |  |  |  |
| Overall | -12.51% | -13.15% | -13.49% | 271% | 469% |
| Class D | -14.39% | -13.40% | -13.32% | 280% | 449% |
| Class F | -12.13% | -13.26% | -13.60% | 253% | 348% |
| Class TGM | -12.67% | -14.71% | -14.89% | 245% | 255% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -9.55% | -9.45% | -9.76% | 237% | 287% |
| Class C | -10.99% | -9.24% | -10.94% | 251% | 254% |
| Class E | -9.22% | -10.12% | -8.20% | 200% | 241% |
| Overall | -9.95% | -9.55% | -9.76% | 231% | 264% |
| Class D | -13.37% | -11.54% | -12.00% | 257% | 264% |
| Class F | -10.91% | -10.19% | -10.47% | 233% | 232% |
| Class TGM | -10.70% | -12.82% | -13.62% | 191% | 186% |

# Contributions

In addition to twelve EE2 contributions, 19 related technical contributions and 2 contributions on the ECM software were received, which can be subdivided as follows:

## Partitioning

JVET-W0110, “AHG12: GPM with inter and intra prediction”, Y. Kidani, H. Kato, K. Kawamura (KDDI)

## In Loop Filters

JVET-W0073, “ALF parameters for RPR”, P. Bordes, F. Galpin, F. Léannec (InterDigital)

JVET-W0074, “HLS for ALF parameters for RPR”, P. Bordes, F. Galpin, K. Naser, F. Léannec (InterDigital)

JVET-W0079, “AHG12: CTB level filter shape selection of CCALF”, M. G. Sarwer, R. -L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-W0098, “Non-EE2: Bilateral Inloop Filter on Chroma”, W. Yin, K. Zhang, L. Zhang (Bytedance)

JVET-W0101, “AHG11 & AHG12: Deep In-Loop Filter with Adaptive Model Selection and External Attention for Enhanced Compression Beyond VVC Capability”, Y. Li, K. Zhang, L. Zhang (Bytedance)

JVET-W0128, “AHG12: Alternative classifiers for ALF”, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

## Intra

JVET-W0067, “EE2-related: Implicit derivation of DIMD blend modes”, X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-W0069, “[AHG12] On Intra TMP Boundary Conditions”,	K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher (InterDigital)

JVET-W0123, “EE2-related: Fusion for template-based intra mode derivation”, K. Cao, N. Hu, V. Seregin, M. Karczewicz (Qualcomm), Y. Wang, K. Zhang, L. Zhang (Bytedance)

JVET-W0124, “EE2-related: Template based intra most probable modes sorting”, K. Cao, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-W0126, “EE2 Related - DIMD with implicitly derived multiple blending modes” J. Zhao, S. Paluri, S. Kim (LGE)

## Inter

JVET-W0068, “EE2-related: A combination of CIIP and DIMD/TIMD”,	X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-W0097, “EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5”, X. Xiu, C.-W. Kuo, X. Wang (Kwai), R.-L. Liao, Y. Ye, X. Li, J. Chen (Alibaba), Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang (Bytedance), Y.-J. Chang, H. Huang, V. Seregin, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-W0106, “EE2-related: Bilateral matching AMVP-merge mode” Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)

JVET-W0107, “EE2-related: Adaptive decoder side motion vector refinement”, H. Huang, Z. Zhang, V. Seregin, W.-J. Chien, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-W0112, “AHG12: Diagonal MMVD with ARMC”, Y. Kidani, K. Kawamura (KDDI)

JVET-W0122, “EE2-related: On spatial MV propagation and neighboring template block access for template matching and multi-pass DMVR”, C.-C. Chen, C.-T. Hsieh, H. Huang, V. Seregin, W.-J. Chien, Y.-J. Chang, Z. Zhang, Y. Zhang, M. Karczewicz (Qualcomm)

## Coefficient Coding

JVET-W0108, “EE2-related: Low complexity sign prediction” C. Auyeung, X. Li, S. Liu (Tencent)

## ECM software related

JVET-W0049, “HG12: on the status of the ECM software”, Y. Ye (Alibaba), M. Karczewicz (Qualcomm), Y.-W. Huang (MediaTek), P. Yin (Dolby), D. Wang (OPPO), X. Wang (Kwai), J. Ström (Ericsson), F. Le Leannec (InterDigital), L. Zhang (Bytedance), S.-H. Kim (LGE), M. Hannuksela (Nokia), P. Wu (ZTE)

JVET-W0102, “Preliminary draft of algorithm description for Enhanced Compression Model 1 Software (ECM 1)”. M. Coban (Qualcomm), F. Le Léannec (InterDigital), J. Ström (Ericsson), Y. Ye (Alibaba)

# Recommendations

The AHG recommends to:

- To review all the related contributions.

- To encourage contribution of new test sequences, especially 8k resolution, which might be included in future test conditions.