| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0022 |
| --- | --- |

| Title: | CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Karam Naser Mohammed Golam Sarwer Fan Wang | Email: | dmytror@qti.qualcomm.com Karam.Naser@InterDigital.com m.sarwer@alibaba-inc.com wangfan6@oppo.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Core Experiment on entropy coding for high bit depth and high bit rate proposed at V meeting of JVET. A total of 5 tests have been completed within the CE between the JVET V and W meetings to study and evaluate technologies related to entropy coding for High Bitdepth (HBD) and High Bitrate (HBR) coding. The scope of this experiment covered tests on changes to coding of the last significant coeffitient (1 test), tests on content adaptive transform precision (2 tests) and tests on CABAC bypass coding methods (2 tests). The software basis for this CE is VTM-13.0. For the test sequences, configurations and test conditions, the High Bit-depth CTC described in JVET-U2018 is used, unless otherwise specified in the CE description. In this report, coding performance and complexity are reported and analyzed. Test results against anchors are provided to show the coding efficiency and complexity trade-off of each tool. Where appropriate results are also provided for lossless conditions and screen content. For tests CE-3.1 and CE-3.2 targeting high throughput configuration, comparative results against the HM in the corresponding configuration. Cross-check results for the performed tests are submitted as separate documents and their summaries are integrated in this contribution.

# Test conditions and evaluation criteria

In LowQP configuration, extended precision processing is enabled (parameter ExtendedPrecision is set equal to 1), and all results are compared with an anchor with the same setting.

For Normal QP configuration (12 bits 4:2:0 PQ/HLG test sequences), extended precision is disabled (parameter ExtendedPrecision is set equal to 0) and the results are be compared with an anchor with extended precision disabled.

Adaptive transform precision tests (i.e. CE-2) are also report results for extended precision processing being disabled for 12 bits data (HDR PQ/HLG and SVT12) in LowQP configuration for both, the anchor and the test.

For tests targeting high throughput profile ( i.e., CE-3), HM configured in HEVC high throughput profile as an additional reference point to be used. The latest version of HM (HM-16.23) will be used for high throughout profile. In HM-16.23, the High Throughput 4:4:4 16 Intra profile is enabled using encoder_intra_high_throughput_rext.cfg.

# Tests on coding of the last significant coeffitient

In this category, one test on proposal JVET-V0121 has been conducted. Proposed method features encoding method to determine at encoder side the value of sh_reverse_last_sig_coeff_flag for each slice that determines the derivation of LastSignificantCoeffX and LastSignificantCoeffY variables. When sh_reverse_last_sig_coeff_flag is equal to 1, the last coefficient position is coded by using the proposed alternative coordinates for a whole slice. Otherwise, the current coordinates for the last coefficient position are used.

| Test | Description document | Cross-checker document |
| --- | --- | --- |
| CE1.1 | JVET-W0046 | JVET-W0048 |

## Simulation results

Simulation results for the tests in this test category, have been provided for HBD/HBR/HFR CTC in the lowQP, normal QP range and lossless configurations and reported in corresponding JVET input documents. Summary of the results form reporting template is provided below.

Table 2.2. Simulation results for CE1.1 test, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE1.1 | -0.02% | -0.04% | -0.05% | -0.22% | -0.26% | -0.31% | -0.30% | -0.33% | -0.29% | -0.29% |
| LDB | CE1.1 | -0.01% | 0.02% | 0.01% | -0.18% | -0.18% | -0.28% | -0.27% | -0.29% | -0.26% | -0.26% |
| RA | CE1.1 | 0.01% | 0.00% | 0.02% | -0.19% | -0.19% | -0.28% | -0.24% | -0.26% | -0.23% | -0.23% |

Table 2.3. Simulation results for CE1.1 test, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE1.1 | -0.28% | -0.29% | -0.28% | -0.28% |
| LDB | CE1.1 | -0.27% | -0.28% | -0.26% | -0.26% |
| RA | CE1.1 | -0.26% | -0.27% | -0.25% | -0.25% |

Table 2.4. Reported run-time estimates for CE1.1 tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE1.1 | 101% | 99% | 100% | 96% | 100% | 97% | 98% | 95% |
| LDB | CE1.1 | 100% | 100% | 101% | 100% | 103% | 99% | 102% | 99% |
| RA | CE1.1 | 99% | 100% | 101% | 98% | 102% | 99% | 103% | 99% |

Table 2.5. Simulation results for CE1.1 tests, HBD/HBR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | CE1.1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| RA | CE1.1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |  |

Table 2.6. Run-time estimates for CE1.1 tests, HBD/HBR CTC, NormQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec |
| AI | CE1.1 | 99% | 99% | 102% | 100% |
| RA | CE1.1 | 100% | 101% | 100% | 100% |

Table 2.7. Simulation results for CE1.1 tests, HBD/HBR CTC, lossless test configuration.

|  | Test | bit-rate saving |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | HDR PQ | HDR HLG | SVT12 | SVT16 |
| AI | CE1.1 | -0.31% | -0.30% | -0.25% | -0.15% |
| LDB | CE1.1 | -0.42% | -0.45% | -0.26% | -0.20% |
| RA | CE1.1 | -0.41% | -0.44% | -0.26% | -0.19% |

# Tests on Content Adaptive Transform Precision

In this category, one test on proposal JVET-V0133 has been conducted. This contribution proposes to adaptively assign the downshifts in the scaling process for transform coefficients (clause 8.7.3 of VVC) and in the transformation process for scaled transform coefficients (clause 8.7.4 of VVC) by examining the coefficients values of upon dequantization and before inverse transform.

For tests in this category, additional results are provided for 12 bits data (HDR PQ/HLG and SVT12) in LowQP configuration with extended precission flag being disabled for both, the test and the VTM anchor.

Additional results will be provided for 16 bit data (SVT16) being coded in LowQP configuration with extended precission flag being enabled, and Log2TransformRange in the test being set equal to 16. Results will be compared against the VTM anchor coded in LowQP configuration with extended precission flag being enabled, and also against the same anchor with Log2TransformRange being set equal to 16.

| Test | Description document | Cross-checker document |
| --- | --- | --- |
| CE2.1 | JVET-W0050 | JVET-W0056 |
| CE2.2 | JVET-W0050 | JVET-W0094 |

In test CE-2.1, the maximum dequantized coefficient value before applying the downshift is considered for computing the adaptive dequantization downshift. The susbsequent inverse transform shifts (horizontal and vertical) are modified accordingly.

In test CE-2.1, in addition to CE2.1, the maximum coefficient value of the horizontal transform is determined for computing the inverse horizontal transform shift. The subsequent vertical transform shift is modified accordingly.

## Simulation results

Common Test Condition, Anchor is VTM13.0, Extended Precision is ON for Test and Anchor.

Table 3.2. Simulation results for CE2.x tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE2.1 | -0.01% | -0.01% | -0.01% | 0.00% | -0.01% | -0.01% | 0.00% | 0.00% | -0.01% | 0.00% |
|  | CE2.2 | -0.01% | -0.02% | -0.02% | -0.01% | -0.01% | -0.01% | -0.01% | 0.00% | -0.01% | -0.01% |
| LDB | CE2.1 | -0.01% | 0.00% | 0.02% | 0.01% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% |
|  | CE2.2 | -0.01% | -0.01% | 0.01% | 0.00% | -0.02% | -0.01% | 0.00% | -0.01% | 0.00% | 0.00% |
| RA | CE2.1 | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.01% | 0.00% | -0.01% | 0.00% | 0.00% |
|  | CE2.2 | -0.01% | 0.00% | 0.00% | -0.01% | -0.01% | -0.01% | 0.00% | -0.01% | 0.00% | 0.00% |

Table 3.3. Simulation results for CE2.x tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE2.1 | -0.01% | -0.01% | -0.01% | -0.01% |
|  | CE2.2 | -0.02% | -0.01% | -0.02% | -0.02% |
| LDB | CE2.1 | -0.02% | 0.00% | -0.03% | -0.03% |
|  | CE2.2 | -0.03% | -0.01% | -0.04% | -0.04% |
| RA | CE2.1 | -0.01% | 0.00% | -0.02% | -0.02% |
|  | CE2.2 | -0.02% | -0.01% | -0.03% | -0.03% |

Table 3.4. Reported run-time estimates for CE2.x tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE2.1 | 109% | 112% | 108% | 115% | 109% | 116% | 111% | 122% |
|  | CE2.2 | 110% | 114% | 109% | 117% | 111% | 123% | 108% | 118% |
| LDB | CE2.1 | 106% | 109% | 100% | 101% | 101% | 104% | 103% | 105% |
|  | CE2.2 | 107% | 110% | 103% | 108% | 102% | 106% | 102% | 107% |
| RA | CE2.1 | 103% | 106% | 104% | 109% | 104% | 111% | 104% | 114% |
|  | CE2.2 | 107% | 110% | 105% | 107% | 104% | 113% | 104% | 110% |

Additional tests, Anchor is VTM13.0, Extended Precision is OFF for test and Anchor.

Table 3.5. Additional results for CE2.x tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE2.1 | -0.28% | -0.47% | -0.48% | -0.11% | -0.22% | -0.22% | -0.12% | -0.05% | -0.15% | -0.15% |
|  | CE2.2 | -0.49% | -0.76% | -0.82% | -0.18% | -0.36% | -0.38% | -0.20% | -0.11% | -0.24% | -0.24% |
| LDB | CE2.1 | -0.21% | -0.18% | -0.20% | -0.19% | -0.24% | -0.27% | -0.15% | -0.10% | -0.17% | -0.17% |
|  | CE2.2 | -0.38% | -0.35% | -0.38% | -0.29% | -0.38% | -0.42% | -0.21% | -0.17% | -0.23% | -0.23% |
| RA | CE2.1 | -0.12% | -0.13% | -0.14% | -0.12% | -0.18% | -0.18% | -0.14% | -0.05% | -0.18% | -0.18% |
|  | CE2.2 | -0.31% | -0.33% | -0.34% | -0.20% | -0.31% | -0.33% | -0.19% | -0.11% | -0.23% | -0.24% |

Additional tests, Anchor is VTM13.0, Log2Transform is set equal to 16 for Test and Anchor.

Table 3.6. Simulation results for additional CE2.x tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE2.1 | -0.22% | -0.19% | -0.23% | -0.23% |
|  | CE2.2 | -0.62% | -0.63% | -0.61% | -0.62% |
| LDB | CE2.1 | -0.37% | -0.15% | -0.48% | -0.49% |
|  | CE2.2 | -0.68% | -0.46% | -0.78% | -0.79% |
| RA | CE2.1 | -0.32% | -0.11% | -0.43% | -0.43% |
|  | CE2.2 | -0.60% | -0.39% | -0.70% | -0.71% |

Table 3.7. Reported run-time estimates for additional CE2.x tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE2.1 | 107% | 110% | 108% | 113% | 107% | 111% | 108% | 110% |
|  | CE2.2 | 107% | 110% | 110% | 113% | 107% | 113% | 107% | 112% |
| LDB | CE2.1 | 101% | 104% | 101% | 110% | 105% | 108% | 108% | 111% |
|  | CE2.2 | 102% | 109% | 102% | 107% | 107% | 110% | 107% | 108% |
| RA | CE2.1 | 108% | 112% | 106% | 111% | 107% | 106% | 107% | 107% |
|  | CE2.2 | 108% | 113% | 106% | 109% | 107% | 106% | 107% | 110% |

No results have been provided for normal QP configuration.

Tests on of CE2.x are not applicabe for lossless coding configuration.

# Tests on CABAC bypass coding methods

In this category, one test based on proposal JVET-V0059 and another test on JVET-V0122 have been conducted.

## Bypass alignment and bypass coding

In this category, methods targeting high throughput profile and presented in JVET-V0059/JVET-V0178 are being tested.

| Test | Description document | Cross-checker document |
| --- | --- | --- |
| CE3.1 | JVET-W0044 | JVET-W0047 |
| CE3.2 | JVET-W0045 | JVET-W0058 |

In test CE3.1, option 2 presented in JVET-V0059/JVET-V0178 was tested. The CABAC bypass alignment will be done before bypass coding of each coefficient group. After reaching to the maximum number of context coded bins ( defined in existing VVC standard), the bypass alignment will be done only once for that transform block before CABAC engine switched to fully bypass coding of transform coefficients.  If the remaining number of context coded bins < 4, sb_coded_flag (of both RRC and TSRC) will be context coded, Otherwise, sb_coded_flag (of both RRC and TSRC)  will be bypasss coded.

In test CE3.2, the throughput benefits of the method proposed in JVET-V0178 was tested. When sps_cabac_bypass_alignment_enabled_flag is equal to 1:

- the corresponding remBinsPass1 and RemCcbs are set to be 0;

- the coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix remain the same (context coded);

- bypass alignment happens before the first bypass coded bin in both RRC and TSRC;

- all coefficient or residual levels are bypass coded with corresponding syntax elements for both RRC and TSRC;

- all sb_coded_flag are bypass coded as well.

In this test, the VTM throughput will be studied and tested in reference to HEVC. The measurements of bitrate, binrate, bin2bit ratio (weighted, unweighted, peak, average) and other relevant test results will be reported by using the templates in JVET-N0049 and JVET-V0150.

## Simulation results

Table 4.2. Simulation results for CE3.x tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.1 | 1.13% | 1.18% | 1.14% | 0.87% | 0.93% | 0.94% | 0.56% | 0.48% | 0.59% | 0.60% |
|  | CE3.2 | 10.33% | 12.57% | 12.70% | 6.34% | 8.09% | 8.59% | 1.63% | 1.51% | 1.70% | 1.68% |
| LDB | CE3.1 | 1.49% | 1.60% | 1.62% | 1.43% | 1.47% | 1.51% | 0.75% | 0.72% | 0.76% | 0.76% |
|  | CE3.2 | 17.40% | 20.27% | 20.36% | 16.89% | 18.27% | 18.82% | 3.19% | 3.00% | 3.29% | 3.27% |
| RA | CE3.1 | 1.49% | 1.57% | 1.58% | 1.43% | 1.47% | 1.51% | 0.78% | 0.74% | 0.80% | 0.80% |
|  | CE3.2 | 17.11% | 19.76% | 20.20% | 17.19% | 18.16% | 18.61% | 3.59% | 3.40% | 3.69% | 3.67% |

Table 4.3. Simulation results for CE3.x tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.1 | 0.31% | 0.28% | 0.33% | 0.33% |
|  | CE3.2 | 0.07% | 0.06% | 0.09% | 0.07% |
| LDB | CE3.1 | 0.47% | 0.46% | 0.47% | 0.47% |
|  | CE3.2 | 0.34% | 0.32% | 0.36% | 0.35% |
| RA | CE3.1 | 0.48% | 0.46% | 0.49% | 0.49% |
|  | CE3.2 | 0.39% | 0.35% | 0.41% | 0.41% |

Table 4.4. Reported run-time estimates for CE3.x tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE3.1 | 100% | 98% | 100% | 99% | 101% | 99% | 101% | 93% |
|  | CE3.2 | 91% | 88% | 84% | 83% | 86% | 81% | 85% | 76% |
| LDB | CE3.1 | 100% | 98% | 99% | 99% | 102% | 101% | 101% | 96% |
|  | CE3.2 | 86% | 92% | 73% | 81% | 81% | 83% | 86% | 82% |
| RA | CE3.1 | 100% | 98% | 99% | 99% | 102% | 101% | 101% | 96% |
|  | CE3.2 | 86% | 90% | 72% | 84% | 82% | 86% | 87% | 86% |

Table 4.5. Simulation results for CE3.x tests, HBD/HBR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | CE3.1 | 3.06% | 1.97% | 1.91% | 3.26% | 3.29% | 1.90% | 3.31% | 3.42% |
|  | CE3.2 | 11.73% | 8.87% | 9.07% | 12.86% | 13.64% | 15.56% | 8.76% | 9.14% |
| RA | CE3.1 | 2.68% | 1.52% | 1.48% | 2.64% | 2.96% | 1.50% | 2.73% | 3.04% |
|  | CE3.2 | 10.79% | 7.92% | 7.75% | 11.38% | 11.83% | 10.42% | 6.67% | 7.51% |

Table 4.6. Run-time estimates for CE2.x tests, HBD/HBR CTC, NormQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec |
| AI | CE3.1 | 100% | 99% | 99% | 98% |
|  | CE3.2 | 91% | 97% | 91% | 100% |
| RA | CE3.1 | 99% | 99% | 99% | 99% |
|  | CE3.2 | 93% | 101% | 89% | 100% |

Table 4.7. Simulation results for CE3.x tests, HBD/HBR CTC, lossless test configuration.

|  | Test | bit-rate saving |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | HDR PQ | HDR HLG | SVT12 | SVT16 |
| AI | CE3.1 | 0.55% | 0.38% | 0.29% | 0.20% |
|  | CE3.2 | 2.91% | 0.93% | 0.42% | -0.29% |
| LDB | CE3.1 | 0.67% | 0.47% | 0.39% | 0.26% |
|  | CE3.2 | 6.11% | 1.29% | 0.39% | -0.28% |
| RA | CE3.1 | 0.69% | 0.48% | 0.40% | 0.27% |
|  | CE3.2 | 6.10% | 1.30% | 0.41% | -0.25% |

### Throughput related simulation results

Table 4.8. BD-binrate (average, weighted) simulation results for CE3.2 tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.2 | -53.71% | -52.96% | -52.64% | -53.77% | -53.12% | -52.71% | -45.36% | -45.42% | -45.33% | -45.32% |
| LDB | CE3.2 | -54.92% | -53.96% | -53.68% | -61.88% | -61.57% | -61.25% | -50.72% | -50.77% | -50.71% | -50.70% |
| RA | CE3.2 | -55.30% | -54.36% | -53.93% | -61.98% | -61.86% | -61.63% | -51.64% | -51.70% | -51.61% | -51.61% |

Table 4.9. BD-binrate (average, weighted) simulation results for CE3.2 tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.2 | -37.57% | -37.59% | -37.56% | -37.57% |
| LDB | CE3.2 | -39.98% | -40.01% | -39.96% | -39.96% |
| RA | CE3.2 | -40.33% | -40.39% | -40.30% | -40.30% |

Table 4.10. bin to bit ratio (peak, weighted and unweighted) simulation results for CE3.2 tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | unweighted | weighted | unweighted | weighted | unweighted | weighted | unweighted | weighted |
| AI | CE3.2 | -15.85% | -57.49% | -14.55% | -56.41% | -10.40% | -46.46% | -10.57% | -37.45% |
| LDB | CE3.2 | -14.11% | -55.32% | -14.40% | -55.85% | -10.39% | -45.72% | -10.56% | -37.16% |
| RA | CE3.2 | -12.80% | -53.70% | -13.62% | -55.00% | -10.26% | -44.23% | -10.34% | -36.32% |

### Simulation results vs. HM16.23

Table 4.11. Simulation results for CE3.x tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.1 |  |  |  |  |  |  |  |  |  |  |
|  | CE3.2 | 1.95% | 1.30% | 0.58% | 1.28% | 0.89% | 1.14% | -1.72% | -2.52% | -1.27% | -1.36% |
| LDB | CE3.1 |  |  |  |  |  |  |  |  |  |  |
|  | CE3.2 | 8.78% | 10.13% | 8.76% | 10.43% | 7.83% | 9.66% | 0.54% | -1.32% | 1.48% | 1.46% |
| RA | CE3.1 |  |  |  |  |  |  |  |  |  |  |
|  | CE3.2 | 8.27% | 9.35% | 8.81% | 10.04% | 7.19% | 9.08% | 0.69% | -1.22% | 1.66% | 1.64% |

Table 4.12. Simulation results for CE3.x tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.1 |  |  |  |  |
|  | CE3.2 | -1.76% | -2.30% | -1.48% | -1.51% |
| LDB | CE3.1 |  |  |  |  |
|  | CE3.2 | -0.95% | -1.97% | -0.45% | -0.44% |
| RA | CE3.1 |  |  |  |  |
|  | CE3.2 | -1.06% | -2.06% | -0.56% | -0.56% |

Table 4.13. Simulation results for CE3.x tests, 16 bits data, HBD/HBR CTC, LowQP test configuration, HM16.23 with High Throughput 4:4:4 Intra 16 profile.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.1 |  |  |  |  |
|  | CE3.2 | -2.05% | -2.61% | -1.76% | -1.80% |

### Throughput related simulation results vs. HM16.23

Table 4.14. BD-binrate (average, weighted) simulation results for CE3.2 tests, 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.2 | -49.05% | -49.34% | -49.40% | -51.38% | -51.57% | -51.29% | -45.04% | -45.32% | -44.88% | -44.90% |
| LDB | CE3.2 | -51.56% | -51.10% | -51.40% | -60.09% | -60.93% | -60.31% | -51.06% | -51.55% | -50.82% | -50.81% |
| RA | CE3.2 | -51.96% | -51.57% | -51.54% | -60.35% | -61.39% | -60.79% | -51.64% | -52.16% | -51.39% | -51.39% |

Table 4.15. BD-binrate (average, weighted) simulation results for CE3.2 tests, 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.2 | -37.12% | -37.42% | -36.97% | -36.99% |
| LDB | CE3.2 | -39.71% | -40.20% | -39.47% | -39.46% |
| RA | CE3.2 | -39.94% | -40.44% | -39.69% | -39.69% |

Table 4.16. bin to bit ratio (peak, weighted and unweighted) simulation results for CE3.2 tests, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | unweighted | weighted | unweighted | weighted | unweighted | weighted | unweighted | weighted |
| AI | CE3.2 | -10.09% | -49.42% | -11.94% | -51.38% | -13.47% | -44.40% | -13.11% | -35.91% |
| LDB | CE3.2 | -9.23% | -46.53% | -11.04% | -50.08% | -13.64% | -43.83% | -12.90% | -35.37% |
| RA | CE3.2 | -9.21% | -45.33% | -11.27% | -49.42% | -13.76% | -42.39% | -12.75% | -34.83% |

Table 4.17. BD-binrate (average, weighted) simulation results for CE3.2 tests, 16 bits data, HBD/HBR CTC, LowQP test configuration, HM16.23 with High Throughput 4:4:4 Intra 16 profile..

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.2 | -37.86% | -38.16% | -37.71% | -37.72% |

Table 4.18. bin to bit ratio (peak, weighted and unweighted) simulation results for CE3.2 tests, HBD/HBR CTC, LowQP test configuration, HM16.23 with High Throughput 4:4:4 Intra 16 profile..

|  | Test | SVT16 RGB |  |
| --- | --- | --- | --- |
|  |  | unweighted | weighted |
| AI | CE3.2 | -13.28% | -36.50% |

# Crosschecker report

Table 5.1. Summary of cross-check reports:

| Test | Crosschecker | Crosscheck report |
| --- | --- | --- |
| CE-1.1 |  | results: code verification: draft text verification: further comments: |
| CE-2.1 |  | results: code verification: draft text verification: further comments: |
| CE-2.2 | Qualcomm, JVET-W0094. | results: Match in terms of performance and run time estimates. code verification: Match to CE description. draft text verification: N/A. further comments: |
| CE-3.1 |  | results: code verification: draft text verification: further comments: |
| CE-3.2 |  | results: code verification: draft text verification: further comments: |

# Complexity analysis and memory requirements

## CE-1

## CE-2

## CE-3

# Input contributions for CE

- CE3.1: CABAC-bypass alignment for high bit-depth coding, M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba), JVET-W0044.

- CE-3.2: a high throughput mode for high bit depth and high bit rate extensions, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), M. Sarwer, J. Chen, Y. Ye, R. Liao (Alibaba), JVET-W0045.

- CE-1.1: coding of last significant coefficient position for high bit depth and high bit rate extensions, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-W0046.

- [CE2.1][CE2.2] Content Adaptive Transform Precision, K. Naser, F. Galpin, T. Poirier, F. Le Leannec (InterDigital), JVET-W0050.

- Crosscheck of JVET-W0044: CE3.1: CABAC-bypass alignment for high bit-depth coding, A. Browne (Sony), JVET-W0047.

- Cross-check on JVET-W0046: Coding of last significant coefficient position for high bit depth and high bit rate extensions (CE1.1), D.Rusanovskyy (Qualcomm), JVET-W0048.

- Crosscheck of JVET-W0050: [CE2.1][CE2.2] Content Adaptive Transform Precision, A. Browne (Sony), JVET-W0056.

- Cross-check on JVET-W0050: [CE2.1][CE2.2] Content Adaptive Transform Precision (CE2.2), D. Rusanovskyy (Qualcomm), JVET-W0094.

# CE-related contributions

- CE-related: Additional bypass coding for high throughput CABAC, M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao ( Alibaba), JVET-W0051.

- CE-related: CABAC skip mode	K. Abe, T. Toma, V. Drugeon (Panasonic), JVET-W0052.

- CE-related: On constraining inverse transform precision for high bit depth and high bitrate video coding, L. Kerofsky, D. Rusanovskyy, M. Karczewicz (Qualcomm), JVET-W0092.

- CE-related: High throughput mode for high bit-depth coding – harmonization of CE-3.2 and CE-1.1, T. Tsukuba, M. Ikeda, T. Suzuki (Sony), JVET-W0114.

- CE-3 related: a different alignment position for high throughput mode, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-W0116.

- CE-3 related: a slice level high throughput mode	F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-W0117.

- CE-3 related: High throughput 4:4:4 16 intra profile for VVC, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-W0118.

# References

[1] A. Browne, T. Ikai, D.  Rusanovskyy, M.Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, JVET, 21th Meeting: by teleconference, 20-28 April. 2021

[2] AHG8: On coding of last significant coefficient position for high bit depth and high bit rate extensions, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) JVET-V0121.

[3] AHG8: Content Adaptive Transform Precision for High Bit Depth Coding, K. Naser, F. Galpin, F. Le Léannec, P. De Lagrange (InterDigital), JVET-V0133.

[4] AHG8: CABAC-bypass alignment for high bit-depth coding, M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba), JVET-V0059.

[5] AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-V0122.

[6] AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-V0178