| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0041-v1 |
| --- | --- |

| Title: | AHG4: Status Report on HDR Video Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini Kenneth Andersson | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com kenneth.r.andersson@ericsson.com |
| Source: | AHG4 Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the status of test sequence selection and rate point determination for the HDR VVC verification test category since the last JVET meeting.  Areas of progress include:

- Recommendation of QP points and sequences for the HLG category for use in a dry-run.

- Recommendation of sequences for the PQ sequences for use in a dry-run.  QP points are still under evaluation.

- Re-encoding of all sequences using HM-16.23 and VTM-12.0 and using 3840x2160 versions of all sequences is complete.

# Introduction

Preparation for the HDR verification test has focused on finalizing the selection of sequences and candidate QP points for a dry-run.  Five sequences have been selected for both the HLG and PQ categories.  Expert viewing has led to a recommendation for the QP points to be used in a dry-run for the HLG category.    Additionally, HM-16.23 and VTM-12.0 versions of the content has been created.

# Test Sequence Selection

Based on expert viewing, the following set of sequences have been identified and are recommended for further evaluation in a dry-run.

### HDR HLG test sequences

A set of HDR-HLG candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The viewing resulted in the recommendation of using the following sequences for the dry-run:

Table 1 – Recommended HDR HLG test sequences

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) | 3840×2160 | 60 | 600 | HLG |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 3840×2160 | 60 | 600 | HLG |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 3840×2160 | 60 | 600 | HLG |
| C07_DramaSea | 3840x2160 | 60 | 600 | HLG |
| C16_PaddockFollow | 3840x2160 | 60 | 600 | HLG |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 2 below.

Table 2 – Proposed QPs for HDR HLG test sequences

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramOnBridge) | 41, 37, 33, 29, 25 | 43, 39, 35, 31, 27 |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 40, 37, 33, 30, 26 | 42, 39, 35, 32, 28 |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 44, 40, 36, 33, 29 | 47, 43, 39, 35, 31 |
| C07_DramaSea | 39, 35, 31, 26, 21 | 41, 37, 33, 29, 24 |
| C16_PaddockFollow | 42, 37, 31, 27, 23 | 45, 40, 35, 30, 26 |

### The objective performance of each sequence is reported below:

H3_AMS01 (TramOnBridge)

H3_AMS06 (CanalWithTrees)

RiverByBoat

C07_DramaSea

C16_PaddockFollow

### HDR PQ test sequences

A set of HDR-PQ candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The viewing resulted in the recommendation of using the following sequences for the dry-run:

Table 9 – Recommended HDR PQ

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| Chimera HDR5 (DrivingPOV) | 3840×2160 | 60 | 600 |
| Meridian HDR2 (Car) | 3840×2160 | 60 | 600 |
| Meridian HDR5 (Beach) | 3840×2160 | 60 | 600 |
| Sparks DirtLot | 3840x2160 | 60 | 600 |
| Sparks Welding | 3840x2160 | 60 | 600 |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 8 below.

Table 10 – Proposed QPs for HDR PQ test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Chimera HDR5 (DrivingPOV) |  |  |
| Meridian HDR2 (Car) | 35, 31, 27, 24, 20 | 38, 34, 30, 26, 22 |
| Meridian HDR5 (Beach) | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| Sparks DirtLot |  |  |
| Sparks Welding |  |  |

### The objective performance of each sequence is reported below:

Meridian HDR2 (Car)

Meridian HDR5 (Beach)

# Conclusions

It is suggested to finalize the QP selection for the PQ category and then proceed towards a dry-run evaluation for the HLG and PQ test categories when possible due to the constraints of the COVID pandemic.