| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0041-v2 |
| --- | --- |

| Title: | AHG4: Status Report on HDR Video Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini Kenneth Andersson | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com kenneth.r.andersson@ericsson.com |
| Source: | AHG4 Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the status of test sequence selection and rate point determination for the HDR VVC verification test category since the last JVET meeting.  Areas of progress include:

- Recommendation of QP points and sequences for the HLG category for use in a dry-run.

- Recommendation of sequences for the PQ sequences for use in a dry-run.  QP points are still under evaluation.

- Re-encoding of all sequences using HM-16.23 and VTM-12.0 and using 3840x2160 versions of all sequences is complete.

# Introduction

Preparation for the HDR verification test has focused on finalizing the selection of sequences and candidate QP points for a dry-run.  Five sequences have been selected for both the HLG and PQ categories.  Expert viewing has led to a recommendation for the QP points to be used in a dry-run for the HLG category.    Additionally, HM-16.23 and VTM-12.0 versions of the content has been created.

# Test Sequence Selection

Based on expert viewing, the following set of sequences have been identified and are recommended for further evaluation in a dry-run.

### HDR HLG test sequences

A set of HDR-HLG candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The viewing resulted in the recommendation of using the following sequences for the dry-run:

Table 1 – Recommended HDR HLG test sequences

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) | 3840×2160 | 60 | 600 | HLG |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 3840×2160 | 60 | 600 | HLG |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 3840×2160 | 60 | 600 | HLG |
| C07_DramaSea | 3840x2160 | 60 | 600 | HLG |
| C16_PaddockFollow | 3840x2160 | 60 | 600 | HLG |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 2 below.

Table 2 – Proposed QPs for HDR HLG test sequences

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramOnBridge) | 41, 37, 33, 29, 25 | 43, 39, 35, 31, 27 |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 40, 37, 33, 30, 26 | 42, 39, 35, 32, 28 |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 44, 40, 36, 33, 29 | 47, 43, 39, 35, 31 |
| C07_DramaSea | 39, 35, 31, 26, 21 | 41, 37, 33, 29, 24 |
| C16_PaddockFollow | 42, 37, 31, 27, 23 | 45, 40, 35, 30, 26 |

### The objective performance of each sequence is reported below:

H3_AMS01 (TramOnBridge)

H3_AMS06 (CanalWithTrees)

RiverByBoat

C07_DramaSea

C16_PaddockFollow

### HDR PQ test sequences

A set of HDR-PQ candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The viewing resulted in the recommendation of using the following sequences for the dry-run:

Table 9 – Recommended HDR PQ

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| Chimera HDR5 (DrivingPOV) | 3840×2160 | 60 | 600 |
| Meridian HDR2 (Car) | 3840×2160 | 60 | 600 |
| Meridian HDR5 (Beach) | 3840×2160 | 60 | 600 |
| Sparks DirtLot | 3840x2160 | 60 | 600 |
| Sparks Welding | 3840x2160 | 60 | 600 |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 8 below.

Table 10 – Proposed QPs for HDR PQ test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Chimera HDR5 (DrivingPOV) | Under Study | Under Study |
| Meridian HDR2 (Car) | 35, 31, 27, 24, 20 | 38, 34, 30, 26, 22 |
| Meridian HDR5 (Beach) | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| Sparks DirtLot | Under Study | Under Study |
| Sparks Welding | Under Study | Under Study |

### The objective performance of each sequence is reported below:

Meridian HDR2 (Car)

Meridian HDR5 (Beach)

Chimera HDR5 (DrivingPOV)

SparksDirt

SparksWelding

# Content Conversion

Three of the PQ sequences were spatially cropped to simplify the verification test procedures.  These sequence were Chimera HDR5 (Driving POV), SparksDirtLot, and SparksWelding.  In all cases, the sequences were cropped from a spatial resolution of 4096x2160 to 3840x2160 using the following command:

ffmpeg -f rawvideo -s 4096x2160 -pix_fmt yuv420p10le -i <sequence.yuv> -filter:v "crop=3840:2160" <sequence.yuv>

All of the sequences were subsequently re-encoded using VTM-12.0 and HM-16.23 using a dense QP sampling (QP spacing of 1) for both codecs.  The configuration files used for encoding are provided with this document.

# Conclusions

It is suggested to finalize the QP selection for the PQ category and then proceed towards a dry-run evaluation for the HLG and PQ test categories when possible due to the constraints of the COVID pandemic.