| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0042 |
| --- | --- |

| Title: | AHG4: Agenda and report of the AHG meeting on the HDR video verification test preparation on 2021-06-10 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | AHG |  |  |

_____________________________

# Agenda and Notes

The meeting was held online on Thu, 2021-06-10, 19:00h UTC with the connection information provided via the JVET mailing list and in the JVET calendar.

- Review of contribution JVET-W0041

- Discussion on next steps

| JVET-W0041 | m57070 | 2021-06-09 21:11:44 | 2021-06-09 21:14:47 | 2021-06-10 19:55:08 | AHG4: Status Report on HDR Video Verification Test Preparation | A. Segall, M. Wien, V. Baroncini, K. Andersson |
| --- | --- | --- | --- | --- | --- | --- |

This document summarizes the status of test sequence selection and rate point determination for the HDR VVC verification test category since the last JVET meeting.  Areas of progress include:

- Recommendation of QP points and sequences for the HLG category for use in a dry-run.

- Recommendation of sequences for the PQ sequences for use in a dry-run.  QP points are still under evaluation.

- Re-encoding of all sequences using HM-16.23 and VTM-12.0 and using 3840x2160 versions of all sequences is complete.

It was recommended to modify the sequence names to make them unique if used in multiple test categories. This specifically applies to DrivingPOV which was recommended to be renamed to DrivingPOV-HDR.

It was noted that the document reports Y-PSNR curves. It was commented that it might be appropriate to consider WPSNR for initial rate point selection.

It was mentioned that the HLG set is considered to be quite stable in terms of sequences and QP selection. It was recommended to consider propagating the dry run results to verification test results if they are confirmed to be matching the intended quality range. This could also apply to the PQ category.

Meeting ended at approx. 19:40h UTC.

# Participants list

- Andrew Segall (Sharp Labs - US)

- Dmytro Rusanovskyy (Qualcomm- US)

- Gary Sullivan (Microsoft)

- Jens Ohm (RWTH)

- Kenneth Andersson (Ericsson - SE)

- Mathias Wien (RWTH - DE)

- Olena Chubach (MediaTek - US)

- Peng Yin (Dolby - US) (Peng Yin (Dolby))

- Sean McCarthy (Dolby - US)

- Shunsuke Iwamura (NHK - JP)

- Vittorio Baroncini (UK: Vabtech)

- Walt Husak (Dolby - US)

- Yan Ye (Alibaba - US)