| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0043_v2 |
| --- | --- |

| Title: | AHG 10: Alignment of smooth QP control with adaptive QP in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, J. Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The smooth QP control method in VTM do not work properly with adaptive QP because the correct lambda is not used. This contribution suggests fixing the lambda but also change the default settings for smooth QP control to only be applied on intra pictures since that gives most of the visual effect. It could be considered to also add some separate config parameters for inter pictures for additional flexibility.

The BD-rate performance (Y/Cb/Cr) compared to CTC with VTM-13.0 as anchor is as follows:

RA SDR (luma, cb, cr): 0.05%, -0.01%, 0.00%

RA HDR (DE100 PSNR-L100, wPSNR, Y-PSNR): -0.78%, -0.14%, 0.07%, 0.05%

With PerceptQPA enabled in both anchor and proposal:

RA SDR (luma, cb, cr): 0.00%,	0.01%,	-0.02%

RA HDR (DE100 PSNR-L100, wPSNR, Y-PSNR): -0.05%, -0.00%, 0.00%, 0.00%

It is asserted that the method improves visual quality over the percept QPA method for some HDR content.

v2: correction of RA numbers for CatRobot

# Introduction

It was observed that smooth QP in VTM [1] do not work properly together with adaptive QP (PerceptQPA). The reason is that the correct lambda is not used for a block with reduced QP.

# Proposal

It is proposed to use the correct lambda when smooth QP is used. Currently the lambda corresponding to the QP before the smooth QP adjustment is used. It is also proposed to change default settings to only apply the approach on intra pictures since most subjective improvement comes from those pictures. For improved flexibility some separate config parameters could be added for inter coded pictures such that less adjustment can be made for inter pictures than for intra pictures.

# Results

The proposed change is implemented on top of VTM-13.0 [2] and objective results are provided for SDR CTC [3] and HDR CTC [4].

## SDR CTC

### Lambda fix and only application on intra pictures

Random access

|  | Over VTM-13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | -0,08% | -0,03% | 102% | 102% |
| Class A2 | 0,03% | -0,11% | -0,11% | 101% | 103% |
| Class B | 0,05% | 0,04% | 0,04% | 102% | 104% |
| Class C | 0,05% | 0,05% | 0,05% | 111% | 121% |
| Class E |  |  |  |  |  |
| Overall | 0,05% | -0,02% | 0,00% | 104% | 108% |
| Class D | 0,05% | 0,05% | 0,05% | 100% | 110% |
| Class F | 0,05% | 0,00% | -0,12% | 101% | 103% |

### Percept QPA and smooth QP with lambda fix

Random access

|  | Over VTM-13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,02% | -0,07% | 102% | 102% |
| Class A2 | 0,00% | 0,02% | 0,01% | 100% | 104% |
| Class B | 0,00% | -0,01% | -0,01% | 101% | 99% |
| Class C | 0,00% | 0,00% | -0,01% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,01% | -0,02% | 102% | 102% |
| Class D | 0,04% | 0,04% | 0,09% | 99% | 102% |
| Class F | 0,01% | -0,01% | -0,02% | 102% | 103% |

## HDR CTC

### Lambda fix and only application on intra pictures

|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR | PSNR |  |  |  |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V |  | Y | U |  | V | EncT | DecT |  |
| Class H1 | -0,78% | -0,14% | 0,07% | -0,30% |  | 0,07% | 0,07% | -0,30% | 0,23% |  | 107% | 100% |  |
| Class H2 |  |  |  |  |  |  | 0,02% | -0,08% | -0,24% |  | 104% | 94% |  |
| Overall | -0,78% | -0,14% | 0,07% | -0,30% |  | 0,07% | 0,05% | -0,22% | 0,06% |  | 106% | 98% |  |

### Percept QPA and smooth QP with lambda fix

|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR | PSNR |  |  |  |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V |  | Y | U |  | V | EncT | DecT |  |
| Class H1 | -0,05% | 0,00% | 0,00% | 0,00% |  | 0,00% | 0,00% | 0,01% | 0,00% |  | 100% | 105% |  |
| Class H2 |  |  |  |  |  |  | 0,00% | 0,01% | 0,00% |  | 100% | 97% |  |
| Overall | -0,05% | 0,00% | 0,00% | 0,00% |  | 0,00% | 0,00% | 0,01% | 0,00% |  | 100% | 102% |  |

### Subjective example

Subjective video quality is best evaluated by playout on an HDR capable TV. To illustrate the subjective quality improvement by the proposed method we provide a snapshot from the 4K HDR HLG sequence SunsetBeach for VTM-13.0 with PerceptQPA enabled in Figure 1. Even due to imperfect presentation in this document resulting in banding effects it is evident that the sky is enhanced by the proposed method on top of VTM-13.0 with PerceptQPA enabled in Figure 2.

Figure 1: VTM-13.0 with PerceptQPA enabled (QP 42).

Figure 2: VTM-13.0 with PerceptQPA and proposal enabled (QP 42).

# Conclusions

It has been shown that smooth QP reduction can work together with PerceptQPA (adaptive QP). It is therefore suggested to update the VTM software with the lambda fix and modify the default setting to only use it on intra pictures. For increased flexibility some separate parameters could be added to explicitly apply to inter pictures.

# References

[1] K. Andersson, J. Enhorn, J. Ström , “AHG 10: QP control for very smooth blocks”, JVET-V0078, Meeting by Teleconference, April 2021

[2] VTM-13.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-13.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[4] A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Meeting by Teleconference, October 2020.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).