# Specification changes of JVET-W0044

# Changes to the specification text:

# Change clause 7.3.2.22 (Sequence parameter set range extension syntax ) as follows (additions are yellow-highlighted):

#### 7.3.2.22	Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| sps_cabac_bypass_alignment_enabled_flag | u(1) |
| } |  |

# Change clause 7.3.11.11 (Residual coding syntax) as follows (additions are yellow-highlighted):

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth < 6 && log2TbHeight = = 5 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| CABACBypasIntervalAlign = 0 |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| if( log2TbWidth + log2TbHeight > 3 ) |  |
| if( log2TbWidth < 2 ) { |  |
| log2SbW = log2TbWidth |  |
| log2SbH = 4 − log2SbW |  |
| } else if( log2TbHeight < 2 ) { |  |
| log2SbH = log2TbHeight |  |
| log2SbW = 4 − log2SbH |  |
| } |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| HistValue = sps_persistent_rice_adaptation_enabled_flag ? ( 1 << StatCoeff[ cIdx ] ) : 0 |  |
| updateHist = sps_persistent_rice_adaptation_enabled_flag ? 1 : 0 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| if (remBinsPass1 >= 4 && sps_cabac_bypass_alignment_enabled_flag) |  |
| CABACBypasIntervalAlign = 0 |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( i < lastSubBlock && i > 0 ) { |  |
| sb_coded_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( sb_coded_flag[ xS ][ yS ] && ( xS > 3 ¦ ¦ yS > 3 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = firstPosMode0 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( sb_coded_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != LastSignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| abs_level_gtx_flag[ n ][ 1 ] | ae(v) |
| remBinsPass1− − |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( sh_dep_quant_used_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| if (sps_cabac_bypass_alignment_enabled_flag && sb_coded_flag[ xS ][ yS ] && !CABACBypasIntervalAlign) |  |
| CABACBypasIntervalAlign = 1 |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ){ |  |
| abs_remainder[ n ] | ae(v) |
| if( updateHist && abs_remainder[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( abs_remainder[ n ] ) ) + 2 ) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| } |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( sb_coded_flag[ xS ][ yS ] ) |  |
| dec_abs_level[ n ] | ae(v) |
| if( updateHist && dec_abs_level[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( dec_abs_level[ n ] ) ) ) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( sh_dep_quant_used_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| signHiddenFlag = sh_sign_data_hiding_used_flag && ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( AbsLevel[ xC ][ yC ] > 0 ) && ( !signHiddenFlag ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( sh_dep_quant_used_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHiddenFlag ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( n = = firstSigScanPosSb && sumAbsLevel % 2 = = 1 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| if( log2TbWidth + log2TbHeight > 3 ) |  |
| if( log2TbWidth < 2 ) { |  |
| log2SbW = log2TbWidth |  |
| log2SbH = 4 − log2SbW |  |
| } else if( log2TbHeight < 2 ) { |  |
| log2SbH = log2TbHeight |  |
| log2SbW = 4 − log2SbH |  |
| } |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| inferSbCbf = 1 |  |
| RemCcbs = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| CABACBypasIntervalAlign = 0 |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| if (sps_cabac_bypass_alignment_enabled_flag && RemCcbs >= 4) |  |
| CABACBypasIntervalAlign = 0 |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ][ i ][ 1 ] |  |
| if( i != lastSubBlock ¦ ¦ !inferSbCbf ) |  |
| sb_coded_flag[ xS ][ yS ] | ae(v) |
| if( sb_coded_flag[ xS ][ yS ] && i < lastSubBlock ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| lastScanPosPass1 = −1 |  |
| for( n = 0; n <= numSbCoeff − 1 && RemCcbs >= 4; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| lastScanPosPass1 = n |  |
| if( sb_coded_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| RemCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| CoeffSignLevel[ xC ][ yC ] = 0 |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| RemCcbs− − |  |
| CoeffSignLevel[ xC ][ yC ] = ( coeff_sign_flag[ n ] > 0 ? −1 : 1 ) |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| RemCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| RemCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| } |  |
| /* Greater than X scan pass (numGtXFlags=5) */ |  |
| lastScanPosPass2 = −1 |  |
| for( n = 0; n <= numSbCoeff − 1 && RemCcbs >= 4; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| AbsLevelPass2[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] |  |
| for( j = 1; j < 5; j++ ) { |  |
| if( abs_level_gtx_flag[ n ][ j − 1 ] ) { |  |
| abs_level_gtx_flag[ n ][ j ] | ae(v) |
| RemCcbs− − |  |
| } |  |
| AbsLevelPass2[ xC ][ yC ] += 2 * abs_level_gtx_flag[ n ][ j ] |  |
| } |  |
| lastScanPosPass2 = n |  |
| } |  |
| if (sps_cabac_bypass_alignment_enabled_flag && sb_coded_flag[ xS ][ yS ] && !CABACBypasIntervalAlign) |  |
| CABACBypasIntervalAlign = 1 |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && sb_coded_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass */ |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ]− − |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |

# In clause 7.4.3.22, add the following after the semantics of sps_persistent_rice_adaptation_enabled_flag :

#### 7.4.3.22 Sequence parameter set range extension semantics

…

sps_cabac_bypass_alignment_enabled_flag equal to 1 specifies that the value of ivlCurrRange may be aligned prior to bypass decoding of the syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[]. sps_cabac_bypass_alignment_enabled_flag equal to 0 specifies that the value of ivlCurrRange is not aligned prior to bypass decoding. When not present, the value of sps_cabac_bypass_alignment_enabled_flag is inferred to be equal to 0.

# In clause 9.3.1, make the following changes (additions are highlighted in yellow):

## 9.3 CABAC parsing process for slice data

### 9.3.1General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.11.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

When sps_cabac_bypass_alignment_flag is equal to 1, the request for a value of a syntax element is for either the syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[ ], and CABACBypasIntervalAlign is equal to 1, the alignment process prior to bypass decoding as specified in subclause 9.3.4.3.6 is invoked.

In clause 9.3.4.2.1, make the following changes (additions are highlighted in yellow):

#### 9.3.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

##### 9.3.4.2.1 General

Table 132 – Assignment of ctxInc to syntax elements with context coded bins

| last_sig_coeff_x_prefix | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| last_sig_coeff_y_prefix | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| sb_coded_flag[ ][ ] remBinsPass1 >= 4 ¦ ¦ sps_cabac_bypass_alignment_flag == 0 | 0..6 (clause 9.3.4.2.6) | na | na | na | na | na |
| sb_coded_flag[ ][ ] remBinsPass1 < 4 && sps_cabac_bypass_alignment_flag | bypass | na | na | na | na | na |

Add new clause 9.3.4.3.6 as follows:

##### 9.3.4.3.6 Alignment process prior to aligned bypass decoding

Input to this process is the variable ivlCurrRange.

Output of this process is the updated variable ivlCurrRange.

This process applies prior to the decoding of syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[ ].

ivlCurrRange is set equal to 256.