| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0047-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-W0044: CE3.1: CABAC-bypass alignment for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-3.1 described in JVET-W0044 [1], a proposed modification to improve the throughput of the CABAC. The code and the descriptions in both the contribution document and the CE description [2] matched. The simulation results gave an exact match with those supplied by the proponents. The specification text given in the second version of JVET-W0044 matched the software and description.

# Introduction

CE-3.1 aims to improve the throughput of CABAC by introducing alignment of the CABAC process before the coding of bypass data in RRC and TSRC. This simplifies the decoding of multiple bypass bins simultaneously and thus improves the throughput. This is a technique which was included in the high throughput profile of HEVC range extensions.

# Analysis

The code was found to be in agreement with the description in the CE. The specification changes included in the version 2 of the contribution document were in agreement with both the software and the description.

# Results

The code[3] was compiled using gcc 8.2.1 and tested according to the high bit depth / high bit rate CTC[4] on Linux clusters running openSUSE Leap 15.2. The results were compared with a similarly prepared CE anchor of VTM13.0.

Results for CE-3.1 were found to be in exact agreement with the proponent’s across the low QP, standard QP and lossless test sets.

## Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.17% | 1.24% | 1.20% | 1.16% | 1.22% | 1.19% | 100% | 99% |
| PQ422 | 1.08% | 1.12% | 1.09% | 1.07% | 1.11% | 1.08% | 100% | 99% |
| Overall | 1.13% | 1.18% | 1.14% | 1.12% | 1.17% | 1.14% | 100% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.54% | 1.67% | 1.64% | 1.51% | 1.63% | 1.61% | 99% | 99% |
| PQ422 | 1.44% | 1.52% | 1.61% | 1.40% | 1.49% | 1.57% | 98% | 97% |
| Overall | 1.49% | 1.60% | 1.62% | 1.46% | 1.56% | 1.59% | 99% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.55% | 1.63% | 1.61% | 1.51% | 1.60% | 1.59% | 99% | 99% |
| PQ422 | 1.44% | 1.50% | 1.55% | 1.41% | 1.47% | 1.54% | 100% | 99% |
| Overall | 1.49% | 1.57% | 1.58% | 1.46% | 1.54% | 1.57% | 100% | 99% |
| Overall PQ | 1.37% | 1.45% | 1.45% | 1.34% | 1.42% | 1.43% | 99% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.90% | 0.97% | 0.98% | 100% | 99% |
| HLG422 | 0.83% | 0.89% | 0.91% | 101% | 99% |
| Overall | 0.87% | 0.93% | 0.94% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.46% | 1.50% | 100% | 101% |
| HLG422 | 1.39% | 1.47% | 1.51% | 99% | 98% |
| Overall | 1.43% | 1.47% | 1.51% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.46% | 1.52% | 98% | 98% |
| HLG422 | 1.39% | 1.48% | 1.49% | 99% | 99% |
| Overall | 1.43% | 1.47% | 1.51% | 99% | 98% |
| Overall HLG | 1.24% | 1.29% | 1.32% | 99% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.28% | 0.33% | 0.33% | 100% | 92% |
| SVT12 | 0.48% | 0.59% | 0.60% | 99% | 98% |
| Overall | 0.38% | 0.46% | 0.46% | 99% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.47% | 0.47% | 100% | 94% |
| SVT12 | 0.72% | 0.76% | 0.76% | 102% | 102% |
| Overall | 0.59% | 0.62% | 0.62% | 101% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.49% | 0.49% | 103% | 97% |
| SVT12 | 0.74% | 0.80% | 0.80% | 101% | 99% |
| Overall | 0.60% | 0.64% | 0.64% | 102% | 98% |
| Overall RGB | 0.52% | 0.57% | 0.57% | 101% | 97% |

## Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 2.68% | 1.52% | 1.48% | 2.64% | 2.96% | 1.50% | 2.73% | 3.04% | 102% | 102% |
| Class H2 |  |  |  |  |  | 2.02% | 2.50% | 2.40% | 101% | 101% |
| Overall | 2.68% | 1.52% | 1.48% | 2.64% | 2.96% | 1.76% | 2.62% | 2.72% | 102% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 3.06% | 1.97% | 1.91% | 3.26% | 3.29% | 1.90% | 3.31% | 3.42% | 101% | 100% |
| Class H2 |  |  |  |  |  | 2.15% | 2.56% | 3.12% | 99% | 97% |
| Overall | 3.06% | 1.97% | 1.91% | 3.26% | 3.29% | 2.03% | 2.94% | 3.27% | 100% | 99% |

## Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  |
| PQ444 | 2.6 | 2.6 | 0.56% | 3.1 | 3.1 | 0.69% | 3.1 | 3.1 | 0.72% |
| PQ422 | 2.4 | 2.4 | 0.54% | 2.9 | 2.8 | 0.65% | 2.9 | 2.9 | 0.66% |
| Overall | 2.5 | 2.5 | 0.55% | 3.0 | 3.0 | 0.67% | 3.0 | 3.0 | 0.69% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 97% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  |
| HLG444 | 1.8 | 1.8 | 0.37% | 2.0 | 1.9 | 0.50% | 2.0 | 1.9 | 0.51% |
| HLG422 | 1.7 | 1.7 | 0.38% | 1.9 | 1.9 | 0.45% | 1.9 | 1.9 | 0.46% |
| Overall | 1.7 | 1.7 | 0.38% | 1.9 | 1.9 | 0.47% | 1.9 | 1.9 | 0.48% |
| Enc Time[%] | 98% |  |  | 97% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  | 98% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  | VTM13.0 | VTM_V_CE_3_1 |  |
| SVT16 | 1.2 | 1.2 | 0.20% | 1.2 | 1.2 | 0.26% | 1.2 | 1.2 | 0.27% |
| SVT12 | 1.3 | 1.3 | 0.29% | 1.3 | 1.3 | 0.39% | 1.3 | 1.3 | 0.40% |
| Overall | 1.2 | 1.2 | 0.25% | 1.3 | 1.3 | 0.33% | 1.3 | 1.3 | 0.34% |
| Enc Time[%] | 96% |  |  | 96% |  |  | 99% |  |  |
| Dec Time[%] | 88% |  |  | 87% |  |  | 90% |  |  |

# References

[1] “CE3.1: CABAC-bypass alignment for high bit-depth coding”, JVET-W0044, 	M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao

[2] “Core Experiment on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V2022, K. Naser, D. Rusanovskyy, M.Sarwer, F. Wang

[3] https://vcgit.hhi.fraunhofer.de/jvet-v-ce/VVCSoftware_VTM/-/tree/CE3.1

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu