| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0048-v1 |
| --- | --- |

| Title: | Cross-check on JVET-W0046: Coding of last significant coefficient position for high bit depth and high bit rate extensions (CE1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE1.1 test on coding of last significant coefficient position for HBD/HBR coding which is described in JVET-W0046. Proponents proposes the method of coding the relative coordinates of last significant coefficient in reference to the bottom-right corner of a zero-out transform block. Simulation results in low QP, normal QP and lossless configuration have been cross-checked. In terms of coding performance, results, reported by proponent were replicated during the cross-check. Provided software match to the CE description.

# Introduction

This document reports results of the cross-check for JVET-W0046 Coding of last significant coefficient position for high bit depth and high bit rate extensions (CE1.1) [2].  The following work was conducted:

- Cross-check of simulation results for LowQP, Normal QP and lossless.

- Cross-check of software implementation.

# Cross-check results

In terms of coding performance, results, reported by proponent were replicated during the cross-check.

Tables 1 present simulation results HBD/HBR CTC in Low QP coding configuration.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | -0.03% | 101% | 98% |
| PQ422 | -0.03% | -0.06% | -0.07% | -0.03% | -0.05% | -0.06% | 100% | 101% |
| Overall | -0.02% | -0.04% | -0.05% | -0.02% | -0.04% | -0.05% | 100% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.04% | 0.02% | 0.00% | 0.03% | 0.01% | 101% | 100% |
| PQ422 | -0.02% | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Overall | -0.01% | 0.02% | 0.01% | -0.01% | 0.02% | 0.01% | 101% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | -0.02% | 0.01% | 0.02% | -0.02% | 0.01% | 100% | 99% |
| PQ422 | 0.00% | 0.01% | 0.03% | 0.00% | 0.01% | 0.03% | 100% | 97% |
| Overall | 0.01% | 0.00% | 0.02% | 0.01% | 0.00% | 0.02% | 100% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.21% | -0.24% | -0.30% | 102% | 98% |
| HLG422 | -0.23% | -0.27% | -0.32% | 101% | 99% |
| Overall | -0.22% | -0.26% | -0.31% | 102% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.17% | -0.16% | -0.24% | 102% | 97% |
| HLG422 | -0.20% | -0.20% | -0.32% | 101% | 99% |
| Overall | -0.18% | -0.18% | -0.28% | 102% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.17% | -0.16% | -0.23% | 101% | 97% |
| HLG422 | -0.21% | -0.22% | -0.34% | 102% | 99% |
| Overall | -0.19% | -0.19% | -0.28% | 102% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.29% | -0.28% | -0.28% | 99% | 97% |
| SVT12 | -0.33% | -0.29% | -0.29% | 100% | 97% |
| Overall | -0.31% | -0.28% | -0.29% | 100% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.28% | -0.26% | -0.26% | 103% | 98% |
| SVT12 | -0.29% | -0.26% | -0.26% | 102% | 96% |
| Overall | -0.28% | -0.26% | -0.26% | 103% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.27% | -0.25% | -0.25% | 101% | 101% |
| SVT12 | -0.26% | -0.23% | -0.23% | 102% | 96% |
| Overall | -0.27% | -0.24% | -0.24% | 102% | 98% |

Tables 2 present simulation results HBD/HBR CTC in Normal QP coding configuration.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |

Tables 3 present simulation results HBD/HBR CTC in lossless coding configuration.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  |
| PQ444 | 2.6 | 2.6 | -0.32% | 3.1 | 3.1 | -0.41% | 3.1 | 3.1 | -0.39% |
| PQ422 | 2.4 | 2.4 | -0.30% | 2.9 | 2.9 | -0.44% | 2.9 | 2.9 | -0.42% |
| Overall | 2.5 | 2.5 | -0.31% | 3.0 | 3.0 | -0.42% | 3.0 | 3.0 | -0.41% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  |
| HLG444 | 1.8 | 1.8 | -0.31% | 2.0 | 2.0 | -0.43% | 2.0 | 2.0 | -0.42% |
| HLG422 | 1.7 | 1.7 | -0.29% | 1.9 | 1.9 | -0.47% | 1.9 | 1.9 | -0.46% |
| Overall | 1.7 | 1.7 | -0.30% | 1.9 | 2.0 | -0.45% | 1.9 | 2.0 | -0.44% |
| Enc Time[%] | 99% |  |  | 96% |  |  | 102% |  |  |
| Dec Time[%] | 87% |  |  | 100% |  |  | 91% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  | VTM13.0 | QC-Xcheck |  |
| SVT16 | 1.2 | 1.2 | -0.15% | 1.2 | 1.2 | -0.20% | 1.2 | 1.2 | -0.19% |
| SVT12 | 1.3 | 1.3 | -0.25% | 1.3 | 1.3 | -0.26% | 1.3 | 1.3 | -0.26% |
| Overall | 1.2 | 1.2 | -0.20% | 1.3 | 1.3 | -0.23% | 1.3 | 1.3 | -0.22% |
| Enc Time[%] | 94% |  |  | 115% |  |  | 84% |  |  |
| Dec Time[%] | 91% |  |  | 113% |  |  | 91% |  |  |

# References

[1] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu.

[2] JVET-W0046: Coding of last significant coefficient position for high bit depth and high bit rate extensions (CE1.1), F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO).