| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0053-v1 |
| --- | --- |

| Title: | EE2-2.1: Results for template-based intra mode derivation using MPMs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Li Zhang2, Kai Zhang2 Zhipin Deng1, Na Zhang1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports results for EE2-2.1 on template-based intra mode derivation using MPMs (TIMD). On top of the ECM-1.0, simulation results of EE2-2.1 are reported as below:

Tool-on test (Anchor: VTM-11.0 with new MCTF): AI: {-0.57%, 130%, 113%}; RA: {-0.26%, 110%, 107%}.

Tool-off test (Anchor: ECM-1.0): AI: {-0.35%, 123%, 111%}; RA: {-0.17%, 104%, 102%}.

Introduction

Template-based intra mode derivation using MPMs (TIMD) was proposed in JVET-V0098 [1] and established as Test-2.1 in EE2 [2]. A TIMD mode is derived from MPMs using the neighbouring template. The TIMD mode is used as an additional intra prediction method for a CU. As shown in Figure 1, the prediction samples of the template are generated using the reference samples of the template for each candidate mode. A cost is calculated as the sum of absolute transformed differences (SATD) between the prediction and the reconstruction samples of the template. The intra prediction mode with the minimum cost is selected as the TIMD mode and used for intra prediction of the CU.

Figure 1. Template and its reference samples used in TIMD

Technical description of TIMD

TIMD mode derivation

For each intra prediction mode in MPMs, The SATD between the prediction and reconstruction samples of the template is calculated. The intra prediction mode with the minimum SATD is selected as the TIMD mode and used for intra prediction of current CU. Position dependent intra prediction combination (PDPC) and gradient PDPC are supported in the derivation of the TIMD mode.

TIMD signalling

A flag is signalled in sequence parameter set (SPS) to enable/disable TIMD. When the flag is true, a CU level flag is signalled to indicate whether TIMD is used for the CU. The TIMD flag is signalled right after the MIP flag. If the TIMD flag is equal to true, the remaining syntax elements related to luma intra prediction mode, is skipped.

Interaction with new coding tools in ECM-1.0

When DIMD flag or MIP flag is equal to true, the TIMD flag is not signalled and set equal to false.

TIMD is allowed to be combined with ISP and MRL. When TIMD is combined with ISP or MRL and the TIMD flag is equal to true, the derived TIMD mode is used as the intra prediction mode for ISP or MRL.

When the secondary MPM is enabled, both the primary MPMs and the secondary MPMs are used to derive the TIMD mode.

6-tap interpolation filter is not used in the derivation of the TIMD mode.

Modification of MPM list construction in the derivation of TIMD mode

During the construction of MPM list, intra prediction mode of a neighbouring block is derived as Planar when it is inter-coded. To improve the accuracy of MPM list, when a neighbouring block is inter-coded, a propagated intra prediction mode [3] is derived using the motion vector and reference picture and used in the construction of MPM list.

Simulation results

TIMD was implemented on top of ECM-1.0. Simulations were performed following the common test conditions for EE2 [4]. More detailed results could be found in the attached excel sheets.

The results of tool-on test compared to VTM-11.0 with new MCTF [5] and tool-off test compared to ECM-1.0 are shown in Table 1 and Table 2, respectively.

Table 1: Coding performance compared to VTM-11.0 with new MCTF

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.16% | -0.15% | 127% | 105% | -0.17% | -0.14% | -0.30% | 110% | 106% |
| Class A2 | -0.45% | -0.40% | -0.44% | 128% | 109% | -0.29% | -0.24% | -0.28% | 109% | 106% |
| Class B | -0.58% | -0.41% | -0.44% | 130% | 112% | -0.33% | -0.33% | -0.24% | 110% | 107% |
| Class C | -0.57% | -0.37% | -0.45% | 131% | 119% | -0.22% | -0.21% | -0.26% | 110% | 109% |
| Class E | -1.01% | -0.85% | -0.92% | 131% | 117% |  |  |  |  |  |
| Overall | -0.57% | -0.43% | -0.47% | 130% | 113% | -0.26% | -0.24% | -0.27% | 110% | 107% |
| Class D | -0.41% | -0.34% | -0.23% | 130% | 120% | -0.17% | 0.15% | 0.08% | 110% | 108% |
| Class F (Optional) | -0.46% | -0.43% | -0.32% | 115% | 111% | -0.29% | -0.36% | -0.21% | 109% | 110% |

Table 2: Coding performance compared to ECM-1.0

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.08% | -0.12% | 121% | 103% | -0.12% | -0.03% | -0.09% | 104% | 101% |
| Class A2 | -0.27% | -0.24% | -0.29% | 122% | 109% | -0.15% | -0.05% | -0.21% | 104% | 101% |
| Class B | -0.36% | -0.27% | -0.31% | 124% | 112% | -0.18% | -0.18% | -0.15% | 104% | 101% |
| Class C | -0.35% | -0.23% | -0.30% | 123% | 117% | -0.19% | -0.04% | -0.25% | 104% | 102% |
| Class E | -0.59% | -0.59% | -0.47% | 123% | 112% |  |  |  |  |  |
| Overall | -0.35% | -0.28% | -0.30% | 123% | 111% | -0.17% | -0.09% | -0.18% | 104% | 101% |
| Class D | -0.32% | -0.39% | -0.21% | 122% | 117% | -0.17% | -0.01% | -0.07% | 105% | 103% |
| Class F (Optional) | -0.20% | -0.14% | -0.14% | 113% | 108% | -0.10% | -0.26% | -0.05% | 104% | 102% |

Conclusions

In this contribution, results of EE2-2.1 on TIMD are reported. It is recommended to adopt TIMD in ECM.

References

- Y. Wang, L. Zhang, K. Zhang, Z. Deng, and N. Zhang, “EE2-related: Template-based intra mode derivation using MPMs,” Document of Joint Video Experts Team, JVET-V0098, by teleconference, April 2021.

- V. Seregin, J. Chen, S. Esenlik, F. L. Leannec, L. Li, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-V2024, by teleconference, April 2021.

- K. Zhang, L. Zhang, W. Chien and M. Karczewicz, "Intra-Prediction Mode Propagation for Video Coding," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 9, no. 1, pp. 110-121, March 2019.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-V2017, by teleconference, April 2021.

- P. Wennersten, C. Hollmann, and J. Ström, “[AHG10] GOP-based temporal filter improvements,” Document of Joint Video Experts Team, JVET-V0056, by teleconference, April 2021.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).