| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0054 |
| --- | --- |

| Title: |  | EE2 Test 2.2: DIMD with multiple blending modes |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seethal Paluri Seung-Hwan Kim |  | Tel: Email: | +1 (858) 635-5300 jie.zhao@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: |  | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of DIMD with multiple blending modes in Exploration Experiment (EE) 2 test 2.2. In this test, DIMD with multiple blending modes are introduced. An index is signalled to indicate which DIMD blend mode is to be used. The performance is reported to be:

Tool-On test: (The numbers below are additional gain relative to ECM 1.0 DIMD Tool-on results)

AI: -0.20% (Y) -0.13% (U) -0.10% (V), runtimes 119% (enc) 100% (dec)
RA: -0.09%	 (Y) 0.03% (U) -0.03% (V), runtimes 103% (enc) 99% (dec)

Tool-Off Test:

AI: -0.10% (Y) 0.02% (U) -0.04% (V), runtimes 119% (enc) 100% (dec)
RA: -0.05%	 (Y) 0.04% (U) -0.02% (V), runtimes 104% (enc) 100% (dec)

# Introduction

In EE2 common software ECM1.0 [1], one of the intra tools is Decoder Side Intra Mode Derivation (DIMD) [4]. When DIMD is applied, intra modes are derived at decoder side instead of being signalled. Two intra modes are derived from the reconstructed neighboring samples based on Histogram of Gradients (HoG). The DIMD intra prediction is computed by blending the predictions from the two intra modes and planar mode. Planar prediction is assigned a weight of (1/3) and the remaining (2/3) is split between the first mode and second mode in proportion to their HoG amplitudes. A flag dimd_flag is signalled to indicate if a block is coded in the DIMD mode or not.

In April 2021 a technique allowing multiple DIMD blending modes was proposed in JVET-V0087 [2] and included in the EE2 test. In this document, algorithm of DIMD with multiple blending modes is firstly briefly described, then the tool on and tool off result on the top of ECM1.0 are reported.

# Algorithm Description

In ECM-1.0, when DIMD is applied, there is only one way to blend the predictions from the two intra modes obtained from Histogram of Gradients (HoG) analysis and the planar mode.

In the proposed method, HoG analysis is unchanged, it further allows multiple ways to blend among the first mode, the second mode and planar. These multiple ways to blend are referred as blending modes. The blending mode is selected at encoder based on RD decision and signaled in the bitstream.

In this EE test, we allow 3 blend modes. The first blend mode is to blend the first mode and the second mode only. Weights are based on their HoG amplitudes. The second blend mode is to blend the second mode and planar. Here weight of planar is 1/3 and weight of the second mode is 2/3. The third blend mode is to blend the first mode, the second mode and planar. Here, Planar prediction has a weight of (5/9) and the remaining (4/9) is split between the first mode and second mode in proportion to their HoG amplitude. These modes and weights are picked based on our experiments. This proposed method shows that coding efficiency of DIMD can be improved by allowing flexible ways to blend.

In terms of signalling, a DIMD index instead of dimd_flag is signalled in the bitstream. DIMD index of 0 means DIMD is not applied, and DIMD indices of 1, 2, 3 means DIMD is applied and corresponds to blend mode 1, 2, 3.

| DIMD Index | DIMD blend mode |
| --- | --- |
| 0 | No DIMD |
| 1 | Blend predictions of Mode1 and Mode2 |
| 2 | Blend predictions of Mode2 and Planar (wP = 1/3) |
| 3 | Blend predictions of Mode1, Mode2 and Planar(wP = 5/9) |

Table 1.0 shows the DIMD index and corresponding blending modes.

# Results

The proposed change is implemented on top of ECM1.0.

## Tool On Tests

### ECM1.0 DIMD Tool On Test

The table below shows the ECM 1.0 tool on test result. It is to be used as reference for our EE2.2 tool-on test. Macro settings of the tool on test are BASE_ENCODER=BASE_NORMATIVE=TOOLS=0, ENABLE_DIMD=JVET_V0087_DIMD_NO_ISP=1. Anchor is VTM11+MCTF as specified in EE2 description [3].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM11+MCTF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.55% | -0.64% | 114% | 105% |
| Class A2 | -0.39% | -0.24% | -0.30% | 114% | 106% |
| Class B | -0.46% | -0.23% | -0.30% | 116% | 107% |
| Class C | -0.47% | -0.21% | -0.29% | 114% | 104% |
| Class E | -0.58% | -0.57% | -0.46% | 115% | 107% |
| Overall | -0.51% | -0.34% | -0.38% | 115% | 106% |
| Class D | -0.43% | -0.29% | -0.28% | 114% | 111% |
| Class F | -0.32% | -0.22% | -0.21% | 108% | 107% |
| Class TGM | -0.02% | 0.03% | 0.03% | 109% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM11+MCTF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.35% | -0.48% | 101% | 102% |
| Class A2 | -0.19% | -0.24% | -0.24% | 102% | 101% |
| Class B | -0.30% | -0.24% | -0.24% | 102% | 102% |
| Class C | -0.23% | -0.30% | -0.29% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.28% | -0.30% | 102% | 102% |
| Class D | -0.20% | -0.11% | -0.07% | 102% | 103% |
| Class F | -0.18% | -0.29% | 0.04% | 103% | 103% |
| Class TGM | 0.00% | 0.05% | 0.07% | 103% | 102% |

### EE2.2 Tool On Test

The table below shows the ECM1.0 tool on result with DIMD and the proposed EE2.2 Tool. As shown in the table, gain of DIMD in AI configuration is now increased from -0.51% to -0.71%.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM11+MCTF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.64% | -0.74% | 134% | 105% |
| Class A2 | -0.58% | -0.40% | -0.50% | 135% | 106% |
| Class B | -0.63% | -0.32% | -0.35% | 138% | 107% |
| Class C | -0.61% | -0.36% | -0.30% | 134% | 104% |
| Class E | -0.92% | -0.75% | -0.69% | 138% | 110% |
| Overall | -0.71% | -0.47% | -0.49% | 136% | 106% |
| Class D | -0.55% | -0.39% | -0.20% | 134% | 112% |
| Class F | -0.45% | -0.14% | -0.16% | 117% | 106% |
| Class TGM | -0.10% | -0.04% | -0.08% | 120% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM11+MCTF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.32% | -0.59% | 104% | 102% |
| Class A2 | -0.31% | -0.23% | -0.29% | 104% | 101% |
| Class B | -0.37% | -0.34% | -0.24% | 106% | 101% |
| Class C | -0.30% | -0.08% | -0.27% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.25% | -0.33% | 105% | 101% |
| Class D | -0.20% | -0.27% | -0.11% | 106% | 101% |
| Class F | -0.27% | -0.27% | -0.17% | 106% | 101% |
| Class TGM | -0.02% | -0.05% | -0.06% | 106% | 101% |

### Difference of EE2.2 Tool-on Test and DIMD Tool-on Test

The table below shows the gain of the EE2.2 Tool-on test relative to the ECM1.0 DIMD tool-on test.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Diff to ECM DIMD |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.10% | -0.10% | 118% | 100% |
| Class A2 | -0.19% | -0.16% | -0.20% | 119% | 100% |
| Class B | -0.17% | -0.09% | -0.04% | 119% | 100% |
| Class C | -0.14% | -0.15% | -0.01% | 118% | 100% |
| Class E | -0.35% | -0.18% | -0.24% | 120% | 102% |
| Overall | -0.20% | -0.13% | -0.10% | 119% | 100% |
| Class D | -0.13% | -0.09% | 0.08% | 117% | 101% |
| Class F | -0.13% | 0.08% | 0.05% | 109% | 99% |
| Class TGM | -0.08% | -0.07% | -0.11% | 110% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Diff to ECM DIMD |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.03% | -0.11% | 103% | 100% |
| Class A2 | -0.12% | 0.01% | -0.04% | 102% | 99% |
| Class B | -0.07% | -0.11% | 0.00% | 103% | 99% |
| Class C | -0.07% | 0.22% | 0.02% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.03% | -0.03% | 103% | 99% |
| Class D | 0.00% | -0.16% | -0.04% | 103% | 97% |
| Class F | -0.09% | 0.03% | -0.20% | 103% | 98% |
| Class TGM | -0.02% | -0.10% | -0.13% | 103% | 99% |

## EE2.2 Tool Off Tests

Table below shows the tool-off test of the EE2.2 tool. Its gain in AI is 0.1% now. It is smaller than the gain in tool-on test probably due to overlapping with other tools in ECM1.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.04% | -0.06% | 119% | 99% |
| Class A2 | -0.09% | -0.01% | -0.01% | 118% | 99% |
| Class B | -0.08% | 0.02% | -0.05% | 121% | 100% |
| Class C | -0.07% | 0.10% | -0.02% | 118% | 100% |
| Class E | -0.17% | -0.11% | -0.06% | 119% | 100% |
| Overall | -0.10% | 0.02% | -0.04% | 119% | 100% |
| Class D | -0.06% | 0.07% | 0.13% | 118% | 100% |
| Class F | -0.06% | 0.08% | 0.01% | 111% | 100% |
| Class TGM | -0.03% | -0.01% | 0.02% | 111% | 96% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.04% | -0.14% | 103% | 99% |
| Class A2 | -0.03% | 0.15% | -0.15% | 103% | 99% |
| Class B | -0.06% | 0.06% | 0.13% | 104% | 100% |
| Class C | -0.04% | -0.02% | -0.04% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.04% | -0.02% | 104% | 100% |
| Class D | -0.04% | -0.02% | -0.03% | 103% | 99% |
| Class F | -0.03% | -0.24% | 0.17% | 103% | 100% |
| Class TGM | -0.05% | -0.13% | -0.05% | 105% | 100% |

# Conclusions

In this document, results of EE2.2 DIMD with multiple blending modes are reported. In tool-on test, it has a gain of 0.2% in AI and 0.09% in RA, and in tool-off test, it has a gain of 0.1% in AI and 0.05% in RA.

# References

[1] ECM1.0, available at https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-1.0

[2] J. Zhao, “EE2-Related: Improvements of Decoder-Side Intra Mode Derivation,” JVET-V0087, Meeting by Teleconference, April 2021.

[3] V. Serrgin, J. Chen, S. Esenlik, et. al, “Exploration Experiment on Enhanced Compression beyond VVC capability”, JVET-V2024, Meeting by Teleconference, April 2021.

[4] M. Abdoli, T. Guionnet, E. Mora, et. al, “Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar”, JVET-O0449, Gothenburg, July 2019.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).