| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0055-v3 |
| --- | --- |

| Title: | AHG8: Signaling TSRC rice parameter in slice header extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

The slice header of VVC version 1 contains an extension field, called slice header extension, as a hook for the future extension; It makes it possible to signal new syntax elements to the slice header that are ignored by a version 1 decoder, but can be used by a decoder conforming to a future version. In the latest WD for VVC operation range extension, a rice parameter index for transform-skip residual coding can be signaled in the slice header but not in the slice header extension. This contribution proposes to relocate it to the slice header extension. A patch for the proposed changes relative to VTM-13.0 is attached.

In v2, presentation deck was attached.

In v3, presentation deck was revised.

Introduction

The slice header of VVC version 1 contains an extension filed, i.e. slice header extension, which makes it possible to add new syntax elements to the slice header that are ignored by a version 1 decoder, but can be used by a decoder that conforms to a future version [1] .

At the last JVET-V meeting, rice parameter extension for transform-skip residual coding (TSRC) [1] was adopted, where the rice parameter used for binarizing bin strings of abs_reminder is explicitly signaled in the slice header of each slice. Currently, this syntax element is located before the slice header extension. Related syntax tables and semantics are as below:

Parts for rice parameter extension for TSRC are highlighted in yellow while those for slice header extension are highlighted in green.

# Change clause 7.3.7 (Slice header syntax) as follows (additions are yellow-highlighted):

| … |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |
| for( i = 0; i < sh_slice_header_extension_length; i++) |  |
| sh_slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| … |  |

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx_minus1 may be present in slice_header( ) syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx_minus1 is not present in slice_header( ) syntax structures referring to the SPS. When not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

// cited from JVET-T2001v2 [4]

sh_slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling sh_slice_header_extension_length itself. When not present, the value of sh_slice_header_extension_length is inferred to be equal to 0. Although sh_slice_header_extension_length is not present in bitstreams conforming to this version of this Specification, some use of sh_slice_header_extension_length could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow sh_slice_header_extension_length to be present and in the range of 0 to 256, inclusive.

sh_slice_header_extension_data_byte[ i ] could have any value. Its presence and value do not affect the decoding process specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore the values of all the sh_slice_header_extension_data_byte[ i ] syntax elements. Its value does not affect the decoding process specified in this version of specification.

Considering the motivation of slice header extension, it is natural to signal a new syntax element sh_ts_residual_coding_rice_idx_minus1 in the slice header extension.

Proposal

This contribution proposes to relocate sh_ts_resiudal_coding_rice_idx_minus1 to the slice header extension. Newly added syntax slice_header_extension_data_bit and a function more_data_in_slice_header_extension( ) are inherited from HEVC.

Proposed text changes relative to JVET-V2005 [3] are as below:

# Change clause 7.3.7 (Slice header syntax) as follows (additions are yellow-highlighted):

| … |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |
| for( i = 0; i < sh_slice_header_extension_length; i++) |  |
| sh_slice_header_extension_data_byte[ i ] | u(8) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| while ( more_data_in_slice_header_extension() ) |  |
| slice_header_extension_data_bit | u(1) |
| } |  |
| … |  |

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx_minus1 may be present in slice_header( ) syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx_minus1 is not present in slice_header( ) syntax structures referring to the SPS. When not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

slice_header_extension_data_bit could have any value. Its presence and value do not affect the decoding process specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore the values of all the slice_header_extension_data_bit syntax elements. Its value does not affect the decoding process specified in this version of specification.

more_data_in_slice_header_extension( ) is specified as follows:

– If ( the current position in the slice_header( ) syntax structure ) − ( the position immediately following slice_header_extension_length ) is less than ( slice_header_extension_length * 8 ), the return value of more_data_in_slice_header_extension( ) is equal to TRUE.

– Otherwise, the return value of more_data_in_slice_header_extension( ) is equal to FALSE

Conclusion

This contribution proposes to relocate sh_ts_resiudal_coding_rice_idx_minus1 to the slice header extension. It is recommended to adopt the proposed cleanup to the next VTM.

Reference

- R. Sjöberg, M. Damghanian, M. Pettersson, “AHG17: Slice header extension”, JVET-O0234, Jun. 2019

- H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding,” JVET-V0054, Apr. 2021

- F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, Y.-K. Wang, “VVC operation range extensions (Draft 3)”, JVET-V2005, Apr. 2021

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, Nov. 2020

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).