| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0047-v1 |
| --- | --- |

| Title: | Crosscheck of CE2.1 from JVET-W0050: [CE2.1][CE2.2] Content Adaptive Transform Precision |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-2.1 described in JVET-W0050 [1], a proposed modification to dynamically assign the downshift upon dequantization and inverse transform in contrast to VTM’s fixed downshifts. No discrepancies between the code and the descriptions in both the contribution document and the CE description [2] were identified. The simulation results gave an exact match for the tested software with only a minor mismatch in an anchor.

# Introduction

CE-2.1 dynamically assigns the downshift upon dequantization and inverse transform. This requires an examination of all coefficients to determine the optimum size of the downshift.

# Analysis

No discrepancies were found between the code and the description in the CE, or between the code and the specification changes included with the contribution.

# Results

The code[3] was compiled using gcc 8.2.1 and tested according to the high bit depth / high bit rate CTC[4] on Linux clusters running openSUSE Leap 15.2. The results were compared with a similarly prepared CE anchor of VTM13.0.

Results for CE-2.1 were found to be in exact agreement with the proponent’s across the low QP range. The proponent did not provide standard QP or lossless test sets, but results are included in this cross-check.

Additional results were provided by the proponent for which the transform length was set to 16. These have also been checked and a small mismatch was found for RA in which it appears the test values have been used for the anchor in addition to be used for the tested values.

## Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.01% | -0.01% | -0.01% | 0.00% | -0.01% | 109% | 113% |
| PQ422 | 0.00% | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% | 107% | 112% |
| Overall | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | 108% | 112% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | 0.00% | 0.01% | -0.02% | -0.01% | 0.01% | 106% | 110% |
| PQ422 | -0.01% | 0.00% | 0.03% | -0.01% | 0.00% | 0.02% | 106% | 112% |
| Overall | -0.01% | 0.00% | 0.02% | -0.01% | 0.00% | 0.02% | 106% | 111% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 111% |
| PQ422 | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 106% | 111% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 111% |
| Overall PQ | -0.01% | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 107% | 112% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | -0.01% | 108% | 114% |
| HLG422 | -0.01% | -0.01% | -0.01% | 109% | 114% |
| Overall | 0.00% | -0.01% | -0.01% | 108% | 114% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.00% | 0.00% | 106% | 113% |
| HLG422 | 0.01% | -0.01% | -0.01% | 105% | 111% |
| Overall | 0.01% | 0.00% | 0.00% | 106% | 112% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | -0.01% | 0.01% | 105% | 110% |
| HLG422 | 0.00% | 0.00% | 0.02% | 106% | 111% |
| Overall | 0.00% | -0.01% | 0.01% | 106% | 111% |
| Overall HLG | 0.00% | 0.00% | 0.00% | 107% | 112% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.01% | -0.01% | -0.01% | 109% | 115% |
| SVT12 | 0.00% | -0.01% | 0.00% | 108% | 114% |
| Overall | 0.00% | -0.01% | -0.01% | 109% | 115% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | -0.03% | -0.03% | 104% | 108% |
| SVT12 | -0.01% | 0.00% | 0.00% | 105% | 110% |
| Overall | -0.01% | -0.02% | -0.02% | 105% | 109% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | -0.02% | -0.02% | 106% | 108% |
| SVT12 | -0.01% | 0.00% | 0.00% | 105% | 109% |
| Overall | 0.00% | -0.01% | -0.01% | 105% | 108% |
| Overall RGB | 0.00% | -0.01% | -0.01% | 106% | 111% |

## Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.20% | 0.03% | 0.02% | 0.19% | 0.26% | 0.01% | 0.06% | 0.20% | 104% | 102% |
| Class H2 |  |  |  |  |  | 0.05% | 0.27% | 0.19% | 103% | 102% |
| Overall | 0.20% | 0.03% | 0.02% | 0.19% | 0.26% | 0.03% | 0.17% | 0.20% | 104% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.11% | 0.03% | 0.03% | 0.10% | 0.00% | 0.02% | 0.13% | 0.00% | 107% | 105% |
| Class H2 |  |  |  |  |  | 0.04% | -0.10% | 0.02% | 107% | 106% |
| Overall | 0.11% | 0.03% | 0.03% | 0.10% | 0.00% | 0.03% | 0.01% | 0.01% | 107% | 105% |

## Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  |
| PQ444 | 2.6 | 2.6 | 0.00% | 3.1 | 3.1 | 0.00% | 3.1 | 3.1 | 0.00% |
| PQ422 | 2.4 | 2.4 | 0.00% | 2.9 | 2.9 | 0.00% | 2.9 | 2.9 | 0.00% |
| Overall | 2.5 | 2.5 | 0.00% | 3.0 | 3.0 | 0.00% | 3.0 | 3.0 | 0.00% |
| Enc Time[%] | 95% |  |  | 97% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 100% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  |
| HLG444 | 1.8 | 1.8 | 0.00% | 2.0 | 2.0 | 0.00% | 2.0 | 2.0 | 0.00% |
| HLG422 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Overall | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Enc Time[%] | 95% |  |  | 94% |  |  | 94% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  | VTM13.0 | VTM_V_CE_2_1 |  |
| SVT16 | 1.2 | 1.2 | 0.00% | 1.2 | 1.2 | 0.00% | 1.2 | 1.2 | 0.00% |
| SVT12 | 1.3 | 1.3 | 0.00% | 1.3 | 1.3 | 0.00% | 1.3 | 1.3 | 0.00% |
| Overall | 1.2 | 1.2 | 0.00% | 1.3 | 1.3 | 0.00% | 1.3 | 1.3 | 0.00% |
| Enc Time[%] | 96% |  |  | 94% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  | 103% |  |  |

# References

[1] “[CE2.1][CE2.2] Content Adaptive Transform Precision”, JVET-W0050, K. Naser, F. Galpin, T. Poirier, F. Le Leannec

[2] “Core Experiment on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V2022, K. Naser, D. Rusanovskyy, M.Sarwer, F. Wang

[3] https://vcgit.hhi.fraunhofer.de/jvet-v-ce/VVCSoftware_VTM/-/tree/CE2.1

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu