| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23nd Meeting, by teleconference, 7–16 Jul. 2021 | Document: JVET-W0059-v5 |
| --- | --- |

| Title: | AHG11: A Deep In-Loop Filter Method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Zhang, Cheng Fang Dong Jiang, Jucai Lin | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based in-loop filtering method with QP based models. Compared with VTM-11.0-NNVC, the proposed method reportedly shows on average {%}, {%}, and {%} BD-rate reductions for {Y, Cb, Cr}, under AI configuration.

Introduction

In this contribution, several convolutional neural networks are trained for reducing the distortion incurred during compression. The luma and chroma components both have 5 different CNN models, corresponding to 5 QPs. The CNN model is selected according to the colour component and QP. The CNN in-loop filter tool is applied after SAO and before ALF. An enable flag in SPS is set to control CNN in-loop filter tool.

Proposed method

Architecture

The architecture of the proposed CNN filtering method is shown in Fig. 1. The network contains N residual blocks, several convolutional layers and a shortcut connection from input to output. Each convolution layer except the last one consists of a 3x3 convolutional layer and a ReLU activation layer.

The input size of the luma NN filtering process is 160x160, including the current CTU and 16 neighboring samples to each side of the current CTU. The chroma samples are up-sampled to 160x160, as an additional channel of input tensor.

The input size for chroma is 80x80, including 8 neighboring samples to each side of the chroma block. The luma samples are down-sampled to 80x80, as an additional channel of input tensor.

Other details related to the network architecture are listed as below:

- For all of convolutional layers, kernel size of 3x3 is used. For internal convolutional layers in residual blocks, the number of feature maps is N (N=64). ReLU is used as the activation function.

- The number of residual blocks is M for luma and M_C for chroma (M = 16, M_C = 10).

- Different groups of models are trained for luma and chroma components.

(a)

(b)

(c)

Fig. 1. (a) Architecture of the proposed luma CNN filtering method. M denotes the number of residual blocks. N stands for the number of feature maps. (b) Architecture of the proposed chroma CNN filtering method. M_C denotes the number of residual blocks. N stands for the number of feature maps. (c) Architecture of the residual block.

Model Selection

In the proposed technique, individual models are trained with QPs in {22, 27, 32, 37, 42}. The corresponding CNN model is selected for the test sequence according to the given QP.

Training and Inference

PyTorch is used for performing the training and the inference process of the proposed CNN filters in VTM. The network information in the training stage is provided in Table 2 as suggested by [1].

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce GTX 1080 Ti |
|  | Framework: | PyTorch v1.3.0 |
|  | Number of GPUs per Task | 1 GPU was used for training |
|  | Epoch: | 1000 |
|  | Batch size: | 32 |
|  | Training time: | 17 h/model |
|  | Training data information: | DIV2K still images [2] |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional | Learning rate | 1e-4 |
|  | Number of iterations |  |
|  | Patch size | 160 x 160 (Luma) 80 x 80 (Chroma) |
|  | Learning rate: | 1.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | L2 MSE |
|  | Preprocessing: | randomly cropped |

The network information in the inference stage is provided in Table 2 as suggested by [1].

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1,218,816 /model (luma) 1.2M/model (luma) 775,872 /model (chroma) 0.78M/model (chroma) |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | 4.67 (per Luma model) 2.97 (per Chroma model) |
|  | kMAC / pixel | 3808.8 (Luma) 2424.6 (Chroma) |
| Optional | Total Conv. Layers | 35 (Luma) 23 (Chroma) |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0-NNVC [3] according to the common test conditions defined in [1]. The proposed CNN in-loop filter method is applied after SAO and before ALF. Test results are shown in Table 3. Under AI configurations, the proposed method can bring about 5%, 13%, 15% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM11.0-NNVC (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | #NUM! | #NUM! |
| Class A2 |  |  |  | #NUM! | #NUM! |
| Class B |  |  |  | #NUM! | #NUM! |
| Class C | -4.55% | -13.21% | -15.06% | 1731% | 412921% |
| Class E | -5.64% | -14.76% | -15.05% | 4044% | 618997% |
| Overall |  |  |  | #NUM! | #NUM! |
| Class D | -5.23% | -12.27% | -15.56% | 2751% | 303132% |
| Class F | 1.45% | -2.37% | 2.54% | 2956% | 609187% |

Conclusions

This contribution presents a CNN-based in-loop filtering method. It is proposed to further study CNN-based in-loop filters in the Ad-Hoc group.

References

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).