| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0060-v1 |
| --- | --- |

| Title: | AHG8: A constraint of max CTU size and tile on WPP for high bit rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan A | Tel: Email: | +81-70-7598-7407 Kenji.Kondo@sony.com Masaru.Ikeda@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a constraint for max CTU size to 64 for high bit rate coding. When high throughput is required, wavefront parallel processing can be used by setting sps_entropy_coding_sync_enabled_flag to 1. In VVC version 1, max CTU size was extended from 64 (HEVC) to 128. It might make max delay of CABAC of a CTU row increased. This proposed constraint can decrease the max delay. Also, the other solution, CTU row splitting using tile, is proposed. To investigate the impact of coding efficiency, max CTU size 64 and CTU row split by tiling were tested. In the experiment, an impact of coding efficiency is reported as below. The numbers in the bracket mean BD rates for Y/G, U/B and V/R.

- Test-1: sps_entropy_coding_sync_enabled_flag: 1 (CTU size: 128)

  - {0.08%, 0.07%, 0.02%} for wPSNR and PQ contents

  - {0.08%, 0.07% ,0.02%} for PSNR and PQ contents

  - {0.06%, 0.03%, 0.01%} for HLG contents

  - {0.01%, 0.01%,0.01%} for SVT contents.

- Test-2: sps_entropy_coding_sync_enabled_flag: 1 and max CTU size: 64

  - {0.15%, 0.01%, -0.17%} for wPSNR and PQ contents

  - {0.15%, 0.02%, -0.14%} for PSNR and PQ contents

  - {0.15%, 0.02%, -0.08%} for HLG contents

  - {0.05%, 0.03%, 0.02%} for SVT contents.

- Test-3: sps_entropy_coding_sync_enabled_flag: 1 and Tile 2x1

  - {0.19%, 0.18%, 0.11%} for wPSNR and PQ contents

  - {0.18%, 0.17%, 0.10%} for PSNR and PQ contents

  - {0.10%, 0.08%, 0.03%} for HLG contents

  - {0.04%, 0.03%, 0.02%} for SVT contents.

Introduction

Generally, if we encode high bit depth contents, lower QP setting will basically be used to keep high accuracy of input signal. Consequently, they are coded at high bit rate. Thus, high throughput CABAC is required.

To get high throughput of CABAC, a technique the so-called wavefront parallel processing (WPP) can be used. When WPP is enabled, CABAC can start for each first CTB of a row of CTBs. It allows a decoder to parse bitstream in parallel, and it can achieve high throughput.

WPP is enabled by setting a flag, sps_entropy_coding_sync_enabled_flag is equal to 1. This framework of WPP is adopted in HEVC and VVC.

In HEVC specification [1], profiles to handle high throughput are specified in the section A.3.6 “High throughput profiles”. In the profile, it is required to set entropy_coding_sync_enabled_flag equal to 1 for High Throughput 4:4:4, High Throughput 4:4:4 10, or High Throughput 4:4:4 14.

Throughput of bin parsing in low QPs condition

Figure 1 shows max numbers of bins in AI setting. The vertical axis means the max number of bins foreach QP in a frame. Those sequences are HLG 4K 60P contents that were used in previous common test condition [4]. The orange and blue of the bar mean EP bins and context bins respectively.

Figure 1: Max number of bins for HLG 4K contents

As we can see, for the test sequence “NightStreet_444”, the number of bins is highest compared with other test sequences at the same QP. To check detail numbers, Table 1 shows the max number of bins for the “NighStreet_444”. In this table, the columns of “ctxbins+epbins/2” and “ctxbins+epbins/4” mean a summation of context bins and EP bins, and EP bins are divided by 2 and 4 respectively. If the CABAC engine can decode multi-bins (e.g. 2 or 4) simultaneously, the actual number can be estimated less.

Table 1: Max numbers of bins for NightStreet_444

| QP | EP bins | Context bins | ctxbins+epbins/2 | ctxbins+epbins/4 |
| --- | --- | --- | --- | --- |
| -13 | 98649680 | 52236665 | 101554671 | 76892251 |
| -8 | 75615675 | 50843571 | 88642450 | 69743010.5 |
| -3 | 54646068 | 47561968 | 74882491.5 | 61222229.75 |
| 2 | 40957725 | 41768709 | 62237056.5 | 52002882.75 |
| 7 | 27320531 | 35501232 | 49161497.5 | 42331364.75 |
| 12 | 16103422 | 30186968 | 38222286.5 | 34204627.25 |

Here, a case analysis is considered for a CABAC throughput. The implementation design is assumed as below.

- The system clock: 500 MHz

- The throughput of CABAC: 1 bin / clock and 4 bins / clock for context bins and EP bins.

These assumptions depend on LSI technology. Although the 500 MHz of clock frequency might be slightly lower for high performance LSI, it is used for low power LSI design. With this assumptions, the time to parse bins in Table 1 is estimated as shown in Table 2. In this table, the unit of number is millisecond (ms).

Table 2 The time to parse bins in milliseconds

|  | Parse time [ms] |  |
| --- | --- | --- |
| QP | ctxbins+epbins/2 | ctxbins+epbins/4 |
| -13 | 203.1 | 153.8 |
| -8 | 177.3 | 139.5 |
| -3 | 149.8 | 122.4 |
| 2 | 124.5 | 104.0 |
| 7 | 98.3 | 84.7 |
| 12 | 76.4 | 68.4 |

The frame rate of this test sequence “NightStreet_444” is 60 fps, and the time per 1 frame is about 16.7 ms. The numbers in Table 2 are larger than 16.7 ms. This result means difficult to decode in real-time.

Parallel processing with WPP

VVC has a framework to allow parallel processing. With WPP, CABAC can work in parallel for each CTU row. Consequently, the number of bins of 1 parsing can be decreased. Figure 2 shows the max number of bins that in a frame and in a CTU row.

Figure 2: Max number of bins

As shown in the graph, with WPP, the max number of bins of 1 parsing can be decreased. The time to parse bins in a CTU row is shown in Table 3. The numbers are less than 16.7 ms (time per frame at 60fps).

Table 3: The time to parse bins in milliseconds

|  | Parse time with WPP [ms] |  |
| --- | --- | --- |
| QP | ctxbins+epbins/2 | ctxbins+epbins/4 |
| -13 | 14.2 | 10.3 |
| -8 | 12.6 | 9.5 |
| -3 | 10.9 | 8.5 |
| 2 | 9.6 | 7.7 |
| 7 | 8.3 | 7.0 |
| 12 | 6.9 | 6.1 |

An issue for WPP and the proposed solution

In the previous JVET meeting, we proposed a constraint for max CTU size in [5]. As mentioned above, WPP allows a decoder to have parallel processing. However, it sometimes increases max delay in following case. The example is shown in Figure 3. In the figure, the rectangular with gray and orange colored shows CTU rows. If WPP is enabled, those CTU rows can be decoded in parallel. However, when one CTU row has too many bits at the rectangular in orange, the delay to parse becomes large in the CTU row.

Figure 3: An example in WPP

In VVC, this problem becomes larger than HEVC because the max CTU size is larger than HEVC. Therefore, we propose a constraint of max CTU size for high bit depth or high bit rate coding. Generally, large CTU can be efficient to improve coding efficiency at lower bit rate. On the other hand, at high bit rate coding, it’s expected that the coding efficiency loss is small.

The other proposed solution using Tile.

The issue of large CTU row as mentioned above, it can be solved also by tiling. It is illustrated in Figure 4. The CTU size is 128 and the CTU rows are divided by tiling. The size of each CTU row becomes half as same as above solution (max CTU size constraint). When WPP is used, the probability model of CABAC is reset at first CTU in CTU rows for each tile. Therefore, CABAC can be processed in smaller CTU row.

Figure 4: CTU row splitting using tile

Experiment

To investigate an impact of coding efficiency in above solutions, we tested three cases as below based on HBD CTC in [2].

- Test-1

  - WaveFrontSyncro: 1 (sps_entropy_coding_sync_enabled_flag = 1)

- Test-2

  - WaveFrontSyncro: 1 (sps_entropy_coding_sync_enabled_flag = 1) and to restrict max CTU size to 64.

- Test-3

  - WaveFrontSyncro: 1 (sps_entropy_coding_sync_enabled_flag = 1) and tiling 2x1.

Experimental results

The test results are shown in Table 4, Table 5 and Table 6 for three test cases.

WaveFrontSynchro: 1 (CTU: 128)

Table 4: Summary of results for Test-1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.04% | 0.02% | 0.03% | 0.04% | 0.02% | 100% | 100% |
| PQ422 | 0.04% | 0.04% | 0.01% | 0.04% | 0.04% | 0.01% | 103% | 100% |
| Overall | 0.04% | 0.04% | 0.02% | 0.04% | 0.04% | 0.02% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.08% | 0.10% | 0.03% | 0.07% | 0.10% | 0.03% | 103% | 101% |
| PQ422 | 0.11% | 0.09% | 0.03% | 0.10% | 0.10% | 0.04% | 100% | 99% |
| Overall | 0.09% | 0.09% | 0.03% | 0.09% | 0.10% | 0.03% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.10% | 0.08% | 0.02% | 0.09% | 0.08% | 0.03% | 104% | 98% |
| PQ422 | 0.12% | 0.07% | 0.02% | 0.12% | 0.07% | 0.02% | 101% | 100% |
| Overall | 0.11% | 0.08% | 0.02% | 0.10% | 0.07% | 0.02% | 103% | 99% |
| Overall PQ | 0.08% | 0.07% | 0.02% | 0.08% | 0.07% | 0.02% | 102% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.00% | 0.01% | 102% | 100% |
| HLG422 | 0.01% | 0.00% | 0.01% | 102% | 99% |
| Overall | 0.01% | 0.00% | 0.01% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.08% | 0.05% | 0.02% | 108% | 100% |
| HLG422 | 0.09% | 0.04% | -0.01% | 107% | 98% |
| Overall | 0.08% | 0.04% | 0.00% | 107% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.07% | 0.05% | 0.01% | 104% | 99% |
| HLG422 | 0.09% | 0.04% | 0.01% | 105% | 100% |
| Overall | 0.08% | 0.05% | 0.01% | 105% | 99% |
| Overall HLG | 0.06% | 0.03% | 0.01% | 105% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.02% | 0.01% | 106% | 100% |
| SVT12 | 0.01% | 0.01% | 0.01% | 104% | 100% |
| Overall | 0.01% | 0.01% | 0.01% | 105% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.02% | 0.02% | 106% | 100% |
| SVT12 | 0.02% | 0.01% | 0.01% | 108% | 101% |
| Overall | 0.01% | 0.02% | 0.01% | 107% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.02% | 0.01% | 0.01% | 98% | 98% |
| SVT12 | 0.02% | 0.01% | 0.01% | 103% | 99% |
| Overall | 0.02% | 0.01% | 0.01% | 101% | 99% |
| Overall RGB | 0.01% | 0.01% | 0.01% | 104% | 100% |

WaveFrontSynchro: 1 and max CTU size: 64

Table 5: Summary of test results for Test-2

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.11% | 0.00% | -0.07% | 0.10% | 0.01% | -0.05% | 99% | 98% |
| PQ422 | 0.13% | 0.04% | -0.04% | 0.12% | 0.06% | -0.03% | 102% | 99% |
| Overall | 0.12% | 0.02% | -0.06% | 0.11% | 0.03% | -0.04% | 101% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.10% | 0.04% | -0.18% | 0.11% | 0.05% | -0.14% | 96% | 95% |
| PQ422 | 0.17% | -0.04% | -0.28% | 0.18% | -0.01% | -0.25% | 94% | 97% |
| Overall | 0.13% | 0.00% | -0.23% | 0.14% | 0.02% | -0.19% | 95% | 96% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.15% | 0.04% | -0.19% | 0.16% | 0.05% | -0.15% | 102% | 97% |
| PQ422 | 0.22% | -0.05% | -0.23% | 0.23% | -0.01% | -0.20% | 99% | 98% |
| Overall | 0.19% | 0.00% | -0.21% | 0.19% | 0.02% | -0.18% | 101% | 97% |
| Overall PQ | 0.15% | 0.01% | -0.17% | 0.15% | 0.02% | -0.14% | 99% | 97% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.07% | 0.02% | 0.03% | 104% | 98% |
| HLG422 | 0.08% | 0.03% | 0.02% | 102% | 98% |
| Overall | 0.07% | 0.02% | 0.02% | 103% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.15% | 0.02% | -0.09% | 103% | 97% |
| HLG422 | 0.19% | 0.02% | -0.16% | 100% | 96% |
| Overall | 0.17% | 0.02% | -0.12% | 102% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.17% | 0.04% | -0.12% | 105% | 98% |
| HLG422 | 0.22% | 0.01% | -0.19% | 103% | 99% |
| Overall | 0.20% | 0.03% | -0.15% | 104% | 98% |
| Overall HLG | 0.15% | 0.02% | -0.08% | 103% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.03% | 0.02% | 106% | 99% |
| SVT12 | 0.03% | 0.02% | 0.01% | 105% | 99% |
| Overall | 0.03% | 0.03% | 0.02% | 105% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.04% | 0.04% | 0.04% | 100% | 98% |
| SVT12 | 0.07% | 0.03% | 0.02% | 100% | 99% |
| Overall | 0.06% | 0.04% | 0.03% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.06% | 0.03% | 0.03% | 100% | 97% |
| SVT12 | 0.07% | 0.01% | 0.01% | 99% | 98% |
| Overall | 0.07% | 0.02% | 0.02% | 99% | 98% |
| Overall RGB | 0.05% | 0.03% | 0.02% | 102% | 98% |

WaveFronSynchro: 1 and Tiling 2x1

Table 6: Summary of test results for Test-3

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.10% | 0.10% | 0.08% | 0.09% | 0.09% | 0.07% | 97% | 99% |
| PQ422 | 0.11% | 0.13% | 0.10% | 0.10% | 0.13% | 0.09% | 101% | 99% |
| Overall | 0.11% | 0.11% | 0.09% | 0.10% | 0.11% | 0.08% | 99% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.20% | 0.22% | 0.13% | 0.19% | 0.20% | 0.12% | 95% | 100% |
| PQ422 | 0.24% | 0.21% | 0.11% | 0.23% | 0.20% | 0.11% | 94% | 99% |
| Overall | 0.22% | 0.21% | 0.12% | 0.21% | 0.20% | 0.11% | 95% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.22% | 0.22% | 0.10% | 0.21% | 0.21% | 0.09% | 98% | 100% |
| PQ422 | 0.26% | 0.20% | 0.13% | 0.25% | 0.20% | 0.11% | 95% | 99% |
| Overall | 0.24% | 0.21% | 0.11% | 0.23% | 0.20% | 0.10% | 96% | 100% |
| Overall PQ | 0.19% | 0.18% | 0.11% | 0.18% | 0.17% | 0.10% | 97% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.03% | 0.03% | 99% | 100% |
| HLG422 | 0.04% | 0.04% | 0.03% | 98% | 99% |
| Overall | 0.04% | 0.03% | 0.03% | 98% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.12% | 0.10% | 0.04% | 100% | 101% |
| HLG422 | 0.12% | 0.11% | -0.01% | 99% | 96% |
| Overall | 0.12% | 0.10% | 0.01% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.11% | 0.11% | 0.02% | 101% | 100% |
| HLG422 | 0.14% | 0.12% | 0.05% | 100% | 99% |
| Overall | 0.13% | 0.11% | 0.04% | 100% | 100% |
| Overall HLG | 0.10% | 0.08% | 0.03% | 99% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.03% | 0.02% | 102% | 100% |
| SVT12 | 0.03% | 0.02% | 0.02% | 100% | 99% |
| Overall | 0.03% | 0.03% | 0.02% | 101% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.03% | 0.03% | 102% | 99% |
| SVT12 | 0.04% | 0.02% | 0.02% | 101% | 99% |
| Overall | 0.04% | 0.03% | 0.03% | 102% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.04% | 0.03% | 0.03% | 101% | 98% |
| SVT12 | 0.04% | 0.03% | 0.03% | 101% | 99% |
| Overall | 0.04% | 0.03% | 0.03% | 101% | 99% |
| Overall RGB | 0.04% | 0.03% | 0.02% | 101% | 99% |

Consideration

The overall test results are shown as below. The numbers in the bracket mean BD rates for Y/G, U/B and V/R.

- Test-1: sps_entropy_coding_sync_enabled_flag: 1 (CTU size: 128)

  - {0.08%, 0.07%, 0.02%} for wPSNR and PQ contents

  - {0.08%, 0.07% ,0.02%} for PSNR and PQ contents

  - {0.06%, 0.03%, 0.01%} for HLG contents

  - {0.01%, 0.01%,0.01%} for SVT contents.

- Test-2: sps_entropy_coding_sync_enabled_flag: 1 and max CTU size: 64

  - {0.15%, 0.01%, -0.17%} for wPSNR and PQ contents

  - {0.15%, 0.02%, -0.14%} for PSNR and PQ contents

  - {0.15%, 0.02%, -0.08%} for HLG contents

  - {0.05%, 0.03%, 0.02%} for SVT contents.

- Test-3: sps_entropy_coding_sync_enabled_flag: 1 and Tile 2x1

  - {0.19%, 0.18%, 0.11%} for wPSNR and PQ contents

  - {0.18%, 0.17%, 0.10%} for PSNR and PQ contents

  - {0.10%, 0.08%, 0.03%} for HLG contents

  - {0.04%, 0.03%, 0.02%} for SVT contents.

For Test-2 and Test-3, small coding efficiency loss (0.1% ~ 0.2%) are confirmed. The reasons are expected that the probability model is frequently reset and CABAC entry points increased. However, the difference between Test-1 and Test-2 or Test-3 is small.

As shown in above results, with proposed constraint, the coding efficiency loss is small, and it’s beneficial to decrease max delay in WPP. If a profile for high bit rate or high throughput was defined, this proposed constraint works well to limit max delay in parallel CABAC.

Conclusion

In this contribution, an issue about throughput was pointed out that max delay increases in parallel CABAC with large CTU size. To solve the problem, two solutions were proposed that a constraint (max CTU size: 64) and CTU row splitting using tile. In the experiments, it was confirmed that the impact of coding efficiency is small. It’s recommended that sps_entropy_coding_sync_enabled_flag sets to 1 and those two constraints.

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Recommendation ITU-T H.265 / High efficiency video coding, SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS Infrastructure of audiovisual services – Coding of moving video, ITU-T H.265 (V7), 2019-11-29

- Adrian Browne, Tomohiro Ikai, Dmytro Rusanovskyy, Mohammed Golam Sarwer, Xiaoyu Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, Jan 2021

- Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001-v2, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020

- Adrian Browne, Tomohiro Ikai, D. Rusanovskyy, Xiaoyu Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020

- Kenji Kondo, Masaru Ikeda, “AHG8: A constraint of max CTU size for high bit depth and high bit rate coding”, JVET-V0068, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, Apr. 2021