| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0061-v1 |
| --- | --- |

| Title: | AHG10: Encoder MV selections and DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Kenneth Andersson Jacob Ström | Email: | ruoyang.yu@ericsson.com kenneth.r.andersson@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

VVC includes a tool called decoder side motion vector refinement (DMVR) to refine motion vectors for inter blocks that are coded using merge mode. The refinement is performed on a subblock basis. It is asserted that the refined MVs for the subblocks may sometimes have poor correlation since the refinement is carried out for each subblock independently. It is further asserted that the poorly correlated subblock MVs are likely to introduce noticeable subblock boundary artifacts, especially for low spatial activity regions.

The contribution proposes an encoder-only method to reduce the risk of subblock boundary artifacts being introduced by DMVR. The method restricts the encoder from choosing a pair of DMVR-applicable bi-MVs when a current block together with the corresponding DMVR-refined MVs are determined to be prone to give the subblock boundary artifacts.

The method has been implemented in the VTM-13.0. The BD-rate PSNR impact is reported to be 0.19% for SDR RA CTC and 0.20% for HDR RA CTC. It is asserted that the proposed method can improve subjective quality when refinement is based on low spatial activity regions, especially when the source block to be predicted happens to contain an edge and the residual coding is too coarse to take care of the artifact. It is proposed to include the method in VTM to enable improved subjective robustness of DMVR.

# Introduction

The DMVR tool in VVC refines motion vectors for inter blocks coded using merge mode. The refinement is carried out for each subblock independently. It is asserted that the refined MVs for the subblocks may have poor correlation between each other. It has been observed that a poor correlation of refined MVs can result in noticeable subblock boundary artifacts for low activity regions coded with high QPs.

# Proposal

This contribution proposes the following encoder-only method: When evaluating a pair of DMVR-applicable bi-MVs for an inter block coded using merge mode, the method first determines whether a current subblock within the inter block as well as its above neighboring subblock and the left neighboring subblock all have low spatial activity. The subblock spatial activity is measured as the averaged spatial activity of the samples of the subblock. The spatial activity of each sample is measured as the sum of the absolute difference between the current sample and the right sample and the absolute difference between the current sample and the left sample. The subblock spatial activity is then compared to a predefined threshold. If the subblock spatial activity is not greater than the threshold, the subblock is determined to be a low activity subblock.

When a current subblock and its above neighboring subblock and left neighboring subblock are all determined to be low activity, the method further compares the DMVR-refined MVs for the subblocks. When the MV difference is above a threshold, the method determines that the area that the subblocks belong to has a high risk of DMVR-introduced subblock boundary artifacts. When this happens, the method prevents the encoder from selecting DVMR refinement by avoiding selecting the bi-MVs for the encoding of the current inter block.

The following configurable parameters are added for controlling the enabling of the method as well as the minimum base QP when the method becomes activated in VTM:

| DMVREncMvSelect | The enabling flag for the proposed method (default is 0/OFF) |
| --- | --- |
| DMVREncMvSelectBaseQpTh | The parameter controls the base QP threshold for the proposed method. The proposed method is activated when the baseQP >= DMVREncMvSelectBaseQpTh (default is 33). |

# Results

The method has been implemented in VTM-13.0 [1] and tested for SDR RA CTC [2], HDR RA CTC [3]. The method has also been tested on non-CTC HLG content [4] under RA configuration. The settings used are DMVREncMvSelect = 1 and DMVREncMvSelectBaseQpTh = 33.

## Subjective results

The subjective problem arises as flickering typically around lines and can appear from QP around 32 and higher. The subjective problem is typically more frequently appearing for even higher QPs.

Figure 1 shows an example area at POC18 of the H3_AMS01 sequence [4] coded with VTM-13.0 at QP 38. Figure 2 shows the same area as in Figure 1 but with the proposed method enabled. It can be noticed that the subblock boundaries (highlighted with a red circle) are removed.

Figure 1. VTM-13.0: H3_AMS01 at QP 38, POC 18

Figure 2. Proposed method on top of VTM-13.0: H3_AMS01 at QP 38, POC 18

The method has also been tested for the BQTerrace SDR sequence in CTC [4] with QP 42. Figure 3 shows an example area at POC 401 of the BQTerrace sequence coded with VTM-13.0. Figure 4 shows the same area as in Figure 3 but with the proposed method enabled. It can be noticed that the subblock boundaries (highlighted with a red circle) are removed.

Figure 3. VTM-13.0: BQTerrace at QP 42, POC 401

Figure 4. Proposed method on top of VTM-13.0: BQTerrace at QP 42, POC 401

## Objective results

Table1 and Table 2 show the BD-rate results for the proposed method under SDR RA CTC [2] and HDR RA CTC [3]. The encoding and decoding run times are not reliable. It can be seen that the method has a BD-rate penalty since it restricts the usage of DMVR.

Table 1. BD-rate performance of the proposed method under SDR RA CTC

|  | Over VTM-13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.35% | 0.31% | 0.28% | 98% | 90% |
| Class A2 | 0.32% | 0.31% | 0.34% | 97% | 93% |
| Class B | 0.12% | 0.16% | 0.14% | 98% | 102% |
| Class C | 0.04% | 0.05% | 0.04% | 93% | 108% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.19% | 0.18% | 96% | 99% |
| Class D | 0.01% | 0.02% | 0.01% | 94% | 92% |
| Class F | 0.03% | 0.05% | 0.05% | 93% | 95% |

Table 2. BD-rate performance of the proposed method under HDR RA CTC

|  | Over VTM-13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0.19% | 0.18% | 0.28% | 94% | 99% |
| Class H2 | 0.23% | 0.28% | 0.27% | 100% | 96% |
| Overall | 0.20% | 0.22% | 0.28% | 96% | 98% |

Table 3 and Table 4 shows the BD-rate results for VTM-13.0 with DMVR turned off. Comparing to Table1 and Table 2, it can be seen that the proposed method maintains the majority of the DMVR objective gains.

Table 3. BD-rate performance of VTM with DMVR OFF under SDR RA CTC

|  | Over VTM-13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.87% | 1.05% | 1.22% | 90% | 84% |
| Class A2 | 1.62% | 1.97% | 1.91% | 89% | 86% |
| Class B | 0.74% | 1.03% | 1.00% | 88% | 89% |
| Class C | 0.73% | 1.00% | 0.96% | 105% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.94% | 1.22% | 1.21% | 93% | 87% |
| Class D | 0.74% | 1.04% | 0.87% | 87% | 73% |
| Class F | 0.33% | 0.44% | 0.39% | 87% | 87% |

Table 4. BD-rate performance of VTM with DMVR OFF under HDR RA CTC

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | 1.11% | 1.34% | 1.60% | 91% | 86% |
| Class H2 | 0.93% | 1.12% | 1.28% | 102% | 90% |
| Overall | 1.05% | 1.26% | 1.48% | 95% | 88% |

# Conclusion

It has been reported that DMVR can introduce noticeable subblock boundary artifacts for low spatial activity regions coded at high QPs. The proposed method restricts the DMVR usage for blocks that are prone to give those DMVR artifacts. It has been shown that the subjective quality for the low spatial activity regions can be improved with the proposed method being enabled. It is proposed to include the proposed method in VTM reference software to enable improved robustness of DMVR.

# References

[1] VTM-13.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-13.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, October 2020.

[3] A. Segall, E. François, W. Husak, S. Iwam, ura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Meeting by Teleconference, October 2020.

[4] S. Iwamura, S. Nemoto, A. Ichigaya, “4K HLG test sequences for HDR verification test”, JVET-S0218, Meeting by Teleconference, June 2020.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).