| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting: by teleconference, 7 – 16 July 2021 | Document: JVET-W0062-v1 |
| --- | --- |

| Title: | EE1: SSIM based CNN model for in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Ouyang Yu Guo Zhenzhong Chen Wuhan University Xiaozhong Xu Shan Liu Tencent | Tel: Email: | oyjiyu@whu.edu.cn zzchen@whu.edu.cn xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Exploration Experiment 1 on Neural Network based filtering for video coding of JVET-U0074. In the test, the impact of loss function adopted during the training is investigated via the MS-SSIM results.

Two sets of experiments are set up, which are implemented on VTM-12.0. The experimental results under All Intra (AI) configuration with 5 QPs recommended by AHG11 are reported in this contribution. From which the inconsistency between the MS-SSIM and PSNR is observed. The model trained with MS-SSIM loss function is proven to have more BD-rate savings in MS-SSIM, while the L1 loss function has more gain over PSNR. The experimental results show that the model trained with MS-SSIM based loss function shows superiority in preserving subjective quality, while the model trained with L1 loss performs better in saving more BD-Rate in PSNR.

Introduction

The architecture of the proposed network is shown in Fig. 1. The network mainly consists of 8 residual units with some 3x3 convolutional layers. Apart from the last convolution layer that generates a 3-channel output, other convolutional layers generate the same amount of feature maps (denoted as M=64). A 3×3 convolution layer and two SE blocks are appended as the last components of the network to adaptively fuse the preceding feature maps and generate the final reconstructed images. Note that 1x1 convolutional layer is not employed in the first unit. The global identity skip connection further facilitates the flow of shallow features.

Fig. 1 The architecture of proposed network.

The architecture of SE block is shown in Fig.2. The feature maps are squeezed into channel-wise statistics, which is produced by aggregating feature maps across their spatial dimensions (H×W). The aggregation is followed by an excitation operation, which takes the form of a simple self-gating mechanism. It takes the channel descriptor as input and produces a collection of per-channel modulation weights. These weights are applied to the feature maps U the generate the output of the SE block which can be fed directly into subsequent layers of the network. SE block adaptively recalibrates channel-wise feature responses by explicitly modeling interdependencies between channels, which benefits the model in learning the compressed information.

Fig. 2 The architecture of SE block.

To keep the metrics adopted in training and testing consistent, the loss function in the contribution JVET-U0074 is changed from the Eq. (1) to Eq. (2), for SSIM oriented testing. In this change, in order to eliminate the influence of other loss functions on the performance, L1 loss function is not added into the final loss function, the influence of MS-SSIM loss function over BD-rate savings is explored through such adjustment.

| LsΘOld=i=1n0.8×SSIMFXi;Θ,Yi+0.2×L1(FXi;Θ,Yi), | (1) |
| --- | --- |
| LsΘTest1=i=1nMS_SSIMFXi;Θ,Yi, | (2) |
| LsΘTest2=i=1nL1FXi;Θ,Yi, | (3) |

Where Θ denotes the parameters of proposed network, Yi indicates the ground truth of the compressed image patch Xi. As a comparison, in a separate test, Eq. (3) is used with pure L1 loss function.

Test Descriptions

Tests

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | The proposed network trained with MS-SSIM loss function. | Tong Ouyang, Yu Guo |  |
| 2 | The proposed network trained with L1 loss function. | Tong Ouyang, Yu Guo |  |

In the two sets of experiments, the influence of different loss functions is discussed via the results of MS-SSIM. In such cases, the network structure is the same in two tests, while MS-SSIM loss function and L1 loss are used for training respectively. Two models are tested under the updated NN CTC, which QPs = {22, 27, 32, 37, 42} under AI and RA configuration. The specific settings of each group of experiments are shown below.

- Test 1: The proposed scheme. MS-SSIM is adopted as a loss function during training. (Results in Section 3.1)

- Test 2: L1 loss is adopted as a loss function during training. (Results in Section 3.2)

Simulation results

Considering the NN-based in-loop filtering is constructed as {DBF, NN, SAO, ALF} in JVET-U0074, SAO and ALF is disabled on all tests to avoid the unnecessary influence of SAO and ALF on the results of NN, and to fairly compare the results from L1 and MS-SSIM model.

Test 1 (Trained with MS-SSIM)

The experimental results of improving the subjective as well as the objective quality are reported in the flowing tables.

Table 1 Experimental results (All Intra)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -3.31% | -0.91% | 1.28% | -5.37% | -5.83% | -4.20% | 225% | #DIV/0! | 0% |
| Class A2 | -4.30% | -9.16% | -10.89% | -4.09% | -10.27% | -8.54% | 155% | #DIV/0! | 0% |
| Class B | -2.92% | -8.71% | -3.96% | -2.95% | -9.52% | -7.52% | 140% | #DIV/0! | 0% |
| Class C | -5.13% | -10.23% | -8.96% | -3.68% | -9.96% | -9.20% | 121% | #DIV/0! | 0% |
| Class E | -6.22% | 0.26% | -0.95% | -5.65% | -7.65% | -4.65% | 149% | #DIV/0! | 0% |
| Overall | -4.26% | -6.33% | -4.85% | -4.16% | -8.82% | -7.03% | 151% | #DIV/0! | 0% |
| Class D | -5.00% | -9.78% | -11.37% | -3.28% | -10.66% | -11.96% | 117% | #DIV/0! | 0% |

Test 2 (Trained with L1)

Table 9 Experimental results (All Intra)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -4.30% | -8.98% | -10.15% | -3.79% | -11.27% | -10.33% | 196% | #DIV/0! | 0% |
| Class A2 | -5.82% | -15.10% | -17.17% | -4.47% | -14.37% | -12.99% | 148% | #DIV/0! | 0% |
| Class B | -4.84% | -14.60% | -12.41% | -3.03% | -13.53% | -11.88% | 135% | #DIV/0! | 0% |
| Class C | -6.34% | -14.26% | -13.27% | -3.43% | -13.27% | -12.50% | 118% | #DIV/0! | 0% |
| Class E | -8.03% | -9.64% | -9.20% | -6.23% | -11.39% | -10.52% | 142% | #DIV/0! | 0% |
| Overall | -5.78% | -12.85% | -12.48% | -4.02% | -12.88% | -11.72% | 143% | #DIV/0! | 0% |
| Class D | -6.21% | -12.91% | -15.77% | -2.94% | -12.72% | -15.05% | 117% | #DIV/0! | 0% |

Sequence wise difference

|  | MS-SSIM |  |  |  |  |  | L1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Campfire | -1.75% | 0.85% | 0.18% | -8.45% | 0.57% | -0.90% | -1.61% | -1.45% | -5.84% | -3.45% | -5.98% | -5.25% |
| MarketPlace | -3.65% | -12.48% | -8.11% | -3.19% | -15.98% | -10.47% | -4.92% | -17.76% | -12.09% | -2.83% | -18.20% | -11.74% |
| PartyScene | -3.41% | -0.62% | -0.50% | -3.36% | -1.24% | -0.86% | -4.33% | -8.56% | -6.25% | -2.84% | -8.62% | -6.07% |
| RaceHorses | -3.44% | -12.99% | -13.95% | -3.16% | -11.92% | -11.94% | -4.56% | -15.47% | -18.21% | -2.34% | -12.89% | -14.94% |
| BlowingBubbles | -4.00% | -6.28% | 0.13% | -4.24% | -8.88% | -0.94% | -4.98% | -10.06% | -7.45% | -3.73% | -11.13% | -6.57% |
| RaceHorses | -6.22% | -17.19% | -16.15% | -2.83% | -14.77% | -14.42% | -7.24% | -20.26% | -20.43% | -2.17% | -15.92% | -17.25% |
| Overall | -3.75% | -8.12% | -6.40% | -4.21% | -8.71% | -6.59% | -4.61% | -12.26% | -11.71% | -2.89% | -12.12% | -10.30% |

Training stage

The proposed network is trained with DIV2K dataset, there are 800 images in its training set and 100 images in its validation set. All the images are compressed under the common test conditions (AI configuration and QP= {22, 27, 32, 37, 42}) with VTM-12.0. More information related with the network architecture are reported in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: GTX 1080ti x8 x 11GB |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: CUDA 9.0, cuDNN 7.1.4, nvidia driver 396.37 |
|  | Framework: | Pytorch v0.4 |
|  | Epoch: | 300 |
|  | Batch size: | 2Kx16 |
|  | Training time: | 57h |
|  | Training data information: | DIV2K (800 images for training and 100 for validation) |
|  | Configurations for generating compressed training data (if different to VTM CTC): | QP= {22,27,32,37,42}, AI. |
| Optional | Patch size | 128x128 |
|  | Learning rate: | 5e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | 0.8×SSIM+0.2×L1 |
|  | Preprocessing: | Mean shift |
|  | Other information: | NO |

Inference stage

The proposed model is tested with 5 different QPs= {22, 27, 32, 37, 42} under AI and RA configuration.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: NO |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: NO |
|  | Framework: | Pytorch v0.4 |
|  | Total Conv. Layers | 45 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 131299 |
|  | Parameter Precision | 32(F) |
|  | Memory Parameter (MB) | 0.50 |
|  | Memory Temp (MB) | 32399.36 (input size: 3840x2160) |
|  | MAC (Giga) | 988 Giga (input size: 3840x2160) |
| Optional | Preprocessing: | Mean shift |
|  | Other information: | NO |

Conclusion

By analyzing the results in Section 3, it can be found that, under the same network structure and test conditions, the MS-SSIM-based training model has better performance in maintaining subjective quality (through MS-SSIM evaluation), while L1 model has better performance in objective quality.

References

- Vmaf, https://github.com/Netflix/vmaf.

- J. Hu, L. Shen and G. Sun, “Squeeze-and-Excitation Networks,” 2018 IEEE/CVF Conference on Computer Vision and Pattern Recognition, Salt Lake City, UT, 2018, pp. 7132-7141.

- JVET-K0391, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Ljubljana, SI, 10–18 July 2018.

- JVET-L0242, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Macao, CN, 03–12 Oct. 2018.

- JVET-M0508, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Marrakech, MA, 9–18 Jan. 2019.

- JVET-N0254, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, Geneva CH, 19–27 March 2019.

- JVET-O0101, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7)”, Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Gothenburg, SE, 3–12 July 2019.

- JVET-T0041, “Methodology and reporting template for neural network coding tool testing”, S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, teleconference, 7-16 Oct. 2020.

- JVET-T0069, “AHG 11: SSIM based CNN model for in-loop filtering”, T. Ouyang, F. Liu, H. Zhu, Z. Chen, X. Xu, S. Liu, teleconference, 7-16 Oct. 2020.

- JVET-U0074, “EE: SSIM based CNN model for in-loop filtering”, T. Ouyang, H. Zhu, Z. Chen, X. Xu, S. Liu, teleconference, 6-15 Jan. 2021.

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

The L1 loss is known as the least absolute error (LAE), which minimizes the sum of the absolute difference.