| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0063-v1 |
| --- | --- |

| Title: | EE1.2.2: NN-based super resolution (JVET-V0073) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, updated experimental results of EE1.2.2 NN-based super-resolution (JVET-V0073) based on the new anchor of EE1.2 are reported. JVET-V0073 was based on a framework for introducing a super-resolution post-filter using CNN (Neural Network). In this framework, the network parameters as network updated can be signalled to the decoder in bit-stream. As experimental results, by sending partial network parameters, the coding efficiency has been improved obviously compared to the fixed network.

EE1.2.2 NN-based super resolution (JVET-V0073) [1]

We proposed a framework of super-resolution post-filter using a neural network [2]. In that framework, an SEI which includes network parameters was proposed.  In this contribution, a super-resolution network using the above experiments is compressed by using NCTM-5.0 [3] which is the reference software of MPEG NNR. Both the configuration parameters, approx_method and enc_method are set to baseline.

Figure 1 A framework of super-resolution neural network

- Test 1: Constant NN parameter version (same NN parameters for all sequences, no signalling)

- Test 2: NN parameters are partially updated by each sequence and each QP (signalled to the decoder in bit-stream)

Figure 2  Network structure of super-resolution neural network

Table 1  Information of training stage

| HW environment: | CPU: Intel(R) Core i7-9700K CPU @ 3.60GHz GPU: GeForce RTX 2080Ti |
| --- | --- |
| SW environment: | Ubuntu 18.04.4 LTS, CUDA 10.2, Nvidia Driver 440.100 |
| Framework: | PyTorch v1.6.0 |
| Epoch: | 8 (100k iteration) for base network 1 (40k iteration) for updated network |
| Batch size: | 32 |
| Training time: | About 5.5h for base network About 0.4h for updated network |
| Training data information: | BVI-DVC 4K |
| Configurations for generating compressed training data | QP values: 22, 27, 32, 37, 42 |
| Patch size | 96x96 |
| Learning rate: | 2e-4 |
| Optimizer: | ADAM |
| Loss function: | L1 |

Table 2  Information of inference stage

| HW environment: | CPU: Intel(R) Core i7-9700K CPU @ 3.60GHz GPU: GeForce RTX 2080Ti |
| --- | --- |
| SW environment: | Ubuntu 18.04.4 LTS, CUDA 10.2, Nvidia Driver 440.100 |
| Framework: | PyTorch v1.6.0 |
| Total Conv. Layers | 12 |
| Total FC Layers | 0 |
| Total Parameter Number | About 500k |

As shown in Figure 2, the neural network structure on 12 layers of network based on simplified ESRGAN. The loss function is the L1 norm and part of the BVI-DVC test set is used as training sequences. The network structure of Test2 is the same as that of Test 1. Except for the last convolutional layer's network parameter, the same network parameters as Test 1 are used. The last convolutional layer parameters are re-trained by each sequence and each QP, and those parameters are encoded using NCTM. The bit rate of the encoded parameters is included in the resulting bit rate. Table 1 shows the information of the training stage and Table 2 shows the information of the inference stage.

Table 3 shows the experimental results of RPR with half-resolution compared to VTM-11.0_NNVC (QP=27, …, 47). This is the second anchor (VVC RPR2 cfg #1) [1]. In several 4K sequences with low bitrate, it is shown that there are some coding gains by changing the resolution of the whole sequence. Table 4 shows the results of Test 1 compared to VTM-11.0_NNVC. Table 5 shows the results of Test 1 compared to RPR2, which means the gains of super-resolution compared to that of up-sampling filtering of RPR. Table 6 shows the results of Test 2 compared to VTM-11.0_NNVC. Table 7 shows the results of Test 1 compared to RPR2. Finally, Table 8 shows Test 2 compared to Test 1, which means the gain of sending partial network parameters as network updated by each sequence and QP. All results have been improved even if the bitrate of the network parameters is included.

Table 3  Experimental results of VVC RPR2 cfg＃1

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC QP=27, ... ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -7.98% | -3.73% | 3.21% | -9.29% | -3.35% | 2.00% | 52% | 30% |
|  | FoodMarket4 | -7.03% | 4.25% | 4.51% | -9.77% | 2.69% | 3.37% |  |  |
|  | Campfire | -10.14% | 36.00% | 14.56% | -12.67% | 15.86% | 7.86% |  |  |
| Class A2 4K | CatRobot1 | 5.41% | 27.95% | 38.98% | -0.49% | 22.71% | 30.13% | 45% | 30% |
|  | DaylightRoad2 | 11.80% | 6.97% | 13.35% | 4.43% | 1.63% | 3.11% |  |  |
|  | ParkRunning3 | -6.12% | 156.68% | 65.53% | -8.86% | 121.23% | 45.52% |  |  |

Table 4  Experimental results of Test1 (QP=22, … ,42)

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC QP=27, ... ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -8.32% | -7.55% | 0.83% | -9.65% | -8.10% | -0.48% | 52% | 238% |
|  | FoodMarket4 | -8.12% | 5.74% | 3.65% | -10.66% | 2.59% | 1.56% |  |  |
|  | Campfire | -12.88% | 35.76% | 5.81% | -14.79% | 15.01% | -0.10% |  |  |
| Class A2 4K | CatRobot1 | -0.15% | 24.60% | 31.37% | -4.63% | 18.34% | 22.93% | 45% | 220% |
|  | DaylightRoad2 | 5.38% | 4.81% | 9.41% | -0.35% | -0.64% | -0.46% |  |  |
|  | ParkRunning3 | -9.38% | 147.06% | 68.01% | -11.08% | 112.54% | 47.50% |  |  |

Table 5  Experimental results of Test1 compared to VVC RPR2 cfg＃1

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC RPR with half resolution QP=22, ... ,42 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -0.36% | -3.97% | -2.32% | -0.40% | -4.92% | -2.47% | 100% | 795% |
|  | FoodMarket4 | -1.27% | 1.50% | -0.83% | -1.05% | -0.25% | -1.82% |  |  |
|  | Campfire | -3.71% | -0.88% | -8.43% | -2.48% | -0.81% | -7.84% |  |  |
| Class A2 4K | CatRobot1 | -6.69% | -2.47% | -6.11% | -4.81% | -3.85% | -5.89% | 100% | 733% |
|  | DaylightRoad2 | -8.29% | -1.98% | -4.51% | -5.82% | -2.28% | -3.58% |  |  |
|  | ParkRunning3 | -3.60% | -2.81% | 3.52% | -2.43% | -4.57% | 1.55% |  |  |

Table 6  Experimental results of Test2

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC QP=27, … ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 | Tango2 | -8.59% | -9.95% | -0.75% | -10.01% | -11.34% | -2.72% | 52% | 237% |
| 4K | FoodMarket4 | -8.47% | -0.49% | -1.32% | -10.55% | -2.96% | -2.50% |  |  |
|  | Campfire | -14.41% | 29.83% | -5.58% | -16.10% | 10.72% | -9.79% |  |  |
| Class A2 | CatRobot1 | -0.54% | 19.57% | 27.35% | -4.75% | 14.02% | 19.41% | 46% | 219% |
| 4K | DaylightRoad2 | 5.17% | -7.38% | 6.68% | -0.39% | -12.17% | -2.89% |  |  |
|  | ParkRunning3 | -9.92% | 29.71% | 38.28% | -11.20% | 18.03% | 21.65% |  |  |

Table 7  Experimental results of Test2 compared to VVC RPR2 cfg＃1

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC RPR with half resolution QP=22, ... ,42 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -0.36% | -3.97% | -2.32% | -0.40% | -4.92% | -2.47% | 100% | 795% |
|  | FoodMarket4 | -1.27% | 1.50% | -0.83% | -1.05% | -0.25% | -1.82% |  |  |
|  | Campfire | -3.71% | -0.88% | -8.43% | -2.48% | -0.81% | -7.84% |  |  |
| Class A2 4K | CatRobot1 | -6.69% | -2.47% | -6.11% | -4.81% | -3.85% | -5.89% | 100% | 733% |
|  | DaylightRoad2 | -8.29% | -1.98% | -4.51% | -5.82% | -2.28% | -3.58% |  |  |
|  | ParkRunning3 | -3.60% | -2.81% | 3.52% | -2.43% | -4.57% | 1.55% |  |  |

Table 8 Experimental results of Test2 compared to Test 1

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over Test 1 QP=22, ... ,42 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 | Tango2 | -0.26% | -2.90% | -1.58% | -0.36% | -3.87% | -2.23% | 101% | 100% |
| 4K | FoodMarket4 | -0.54% | -6.28% | -4.98% | 0.06% | -5.46% | -4.00% |  |  |
|  | Campfire | -2.05% | -7.42% | -12.42% | -1.55% | -5.80% | -10.36% |  |  |
| Class A2 | CatRobot1 | -0.61% | -4.76% | -3.98% | -0.22% | -3.87% | -3.13% | 100% | 100% |
| 4K | DaylightRoad2 | -0.48% | -12.68% | -3.56% | -0.14% | -12.11% | -2.61% |  |  |
|  | ParkRunning3 | -0.61% | -53.68% | -25.13% | -0.10% | -48.34% | -19.24% |  |  |

Reference

[1]	E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma and H. Wang, “Exploration Experiments on Neural Network-based Video Coding”, Joint Video Experts Team Document, JVET-V2023, teleconference, April 2021.

[2]	T. Chujoh, E. Sasaki and T. Ikai, “AHG9/AHG11: Neural network based super resolution SEI”, Joint Video Experts Team Document, JVET-T0092, teleconference, Oct. 2020.

[3]	“Test Model 5 of Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, N 19500, Online, July 2020.

[4]	“Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29 Documents, MDS19764_WG04_N00016, Online, Oct. 2020.

[5]	T. Suzuki, E. Sasaki, T. Chujoh, T. Ikai and H. Watanabe, “Coded Video Transmission using Super-Resolution and MPEG-NNR Deep Learning Metadata”, the 35th Picture Coding Symposium of Japan/the 25th Image Media Processing Symposium (PCSJ/IMPS 2020), Nov. 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).