| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0066 |
| --- | --- |

| Title: | EE2-5.1: Cross-component Sample Adaptive Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Wei Chen, Xianglin Wang | Email: | {cheweikuo, xiaoyuxiu, yiwenchen, jhuhong-jheng, chenwei06, xianglinwang} @kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of the Cross-component Sample Adaptive Offset (CCSAO) in EE2. CCSAO operates as part of SAO process and utilizes the correlation between three components as guidance to further enhance the reconstructive quality of the current sample. Compared to the EE software version that performs an additional bitdepth clipping for CCSAO after the SAO/BIF joint clipping, one harmonized design that performs the clipping after all SAO/BIF/CCSAO offsets addition is also studied. Compared to ECM-1.0 anchors, the BD-rate impact is summarized as below.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| SAO/BIF joint clip +one additional CCSAO clip | AI | 0.10% | -1.54% | -1.74% | 103% | 101% |
|  | RA | 0.00% | -3.03% | -3.32% | 103% | 100% |
|  | LB | -0.13% | -7.62% | -8.15% | 103% | 101% |
| SAO/BIF/CCSAO joint clip | AI | 0.10% | -1.54% | -1.74% | 103% | 102% |
|  | RA | -0.01% | -3.03% | -3.32% | 104% | 101% |
|  | LB | -0.13% | -7.61% | -8.16% | 104% | 102% |

Introduction

In the VVC standard [1], Sample Adaptive Offset (SAO) consists of edge offset (EO) and band offset (BO). EO can effectively remove ringing artifacts near sharp transitions in reconstructed video signal while BO aims at alleviating the DC distortion that is caused by the coding process. In the BO, the reconstructed samples are firstly classified into multiple bands, and one offset is derived for each band which is added to the corresponding reconstructed samples that fall into the same band. Meanwhile, in the existing SAO design, the BO classifier only uses the reconstruction of one single luma/chroma sample as input to enhance itself, without taking the strong correlation between different components into consideration.

Proposal

Cross-component Sample Adaptive Offset (CCSAO) is proposed in [2] for further compression efficiency exploration beyond VVC. Specifically, similar to SAO, the CCSAO classifies the reconstructed samples into different categories, properly derives one offset for each category and adds the offset to the reconstructed samples in that category. However, different from SAO which only uses one single luma/chroma component of current sample as input, the CCSAO utilizes all three components to classify the current sample into different categories. To facilitate the parallel processing, the output samples from the de-blocking filter are used as the input of the CCSAO. Figure 1 shows the diagram of the decoding workflow when the CCSAO is applied.

Figure 1 Modified SAO process when the proposed CCSAO is applied.

In the current CCSAO design, to achieve one better complexity/performance trade-off, only BO is used to enhance the quality of the reconstructed samples. For a given luma/chroma sample, three candidate samples are selected to classify the given sample into different categories: one collocated Y sample, one collocated U sample, and one collocated V sample. The sample values of these three selected samples are then classified into three different {bandY, bandU, bandV} bands, and a joint index i is used to indicate the category of the given sample. One offset is signaled and added to the reconstructed samples that fall into that category, which can be formulated as

bandY=Ycol∙NY≫BDbandU=Ucol∙NU≫BDbandV=Vcol∙NV≫BDi=bandY∙NU∙NV+bandU∙NV+bandVC'rec=Clip1Crec+σCCSAOi                       (1)

In equation (1), {Ycol, Ucol, Vcol} are the three selected collocated samples that are used to classify the current sample; {NY, NU, NV} are the numbers of equally divided bands applied to {Ycol, Ucol, Vcol} full range respectively; BD is the internal coding bit-depth; Crec and Crec' are the reconstructed samples before and after the CCSAO is applied; σCCSAOi is the value of CCSAO offset that is applied to the i-th BO category. In the current design, the collocated luma sample can be chosen from 9 candidate positions, while the collocated chroma sample positions are fixed, as depicted in Figure 2.

Similar to SAO, different classifiers can be applied to different local region to further enhance the whole picture quality. The parameters for each classifier (i.e., the position of Ycol, NY, NU, NV, and offsets) are signaled in frame level, and which classifier to be used is explicitly signaled and switched in CTB level. For each classifier, the maximum of {NY, NU, NV} is set to {16, 4, 4}, and offsets are constrained to be within the range [-15, 15]. The maximum classifiers per frame is constrained to be 4.

Figure 2 Illustration of the candidate positions used for the CCSAO classifier.

Bilateral filter (BIF) was proposed in [3] and was incorporated into ECM-1.0 [4]. Figure 3 (a) shows the current BIF design when it interacts with SAO. Offsets from SAO and BIF are added to the input sample, followed by performing one bitdepth clipping. However, when CCSAO is also joined in the SAO stage, two possible clipping designs can be selected: (1) adding one additional bitdepth clipping for CCSAO, and (2) one harmonized design that performs a joint clipping after adding SAO/BIF/CCSAO offsets to the input sample, as shown in Figure 3 (b) and Figure 3 (c). Both methods are tested in this contribution. Note the abovementioned clipping designs only differ in luma samples since BIF only applies on them.

Figure 3 (a) SAO/BIF offsets clipping. (b) Adding one additional bitdepth clipping for CCSAO. (c) Jointly clipping after adding SAO/BIF/CCSAO offsets to the input sample.

Simulation results

The proposed technique was implemented based on ECM-1.0 exploration software [4] and tested by following the common test condition (CTC) as specified in [5]. Note that SIMD optimization is not applied in the current implementation.

SAO/BIF joint clip + one additional CCSAO clip

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.89% | -1.43% | 104% | 102% |
| Class A2 | 0.05% | -1.69% | -0.28% | 108% | 102% |
| Class B | 0.14% | -2.30% | -3.16% | 103% | 101% |
| Class C | 0.12% | -1.27% | -1.24% | 101% | 100% |
| Class E | 0.11% | -1.12% | -1.82% | 102% | 101% |
| Overall | 0.10% | -1.54% | -1.74% | 103% | 101% |
| Class D | 0.06% | -0.93% | -0.59% | 101% | 101% |
| Class F | -0.11% | -1.97% | -2.37% | 103% | 102% |
| Class TGM | -0.76% | -1.45% | -1.44% | 106% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -2.15% | -4.25% | 104% | 101% |
| Class A2 | 0.09% | -2.52% | -1.36% | 105% | 100% |
| Class B | 0.12% | -4.49% | -5.02% | 103% | 100% |
| Class C | 0.06% | -2.23% | -1.98% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -3.03% | -3.32% | 103% | 100% |
| Class D | 0.06% | -1.87% | -1.45% | 101% | 100% |
| Class F | -0.23% | -2.90% | -2.95% | 106% | 100% |
| Class TGM | -1.15% | -3.03% | -3.24% | 116% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -11.76% | -11.37% | 105% | 101% |
| Class C | 0.06% | -5.96% | -5.11% | 101% | 101% |
| Class E | -0.18% | -2.94% | -6.82% | 104% | 101% |
| Overall | -0.13% | -7.62% | -8.15% | 103% | 101% |
| Class D | 0.16% | -6.25% | -5.04% | 101% | 101% |
| Class F | -0.05% | -7.17% | -6.64% | 108% | 101% |
| Class TGM | -2.23% | -7.59% | -7.60% | 121% | 97% |

SAO/BIF/CCSAO joint clip

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.89% | -1.43% | 104% | 103% |
| Class A2 | 0.05% | -1.69% | -0.28% | 108% | 102% |
| Class B | 0.14% | -2.30% | -3.16% | 102% | 102% |
| Class C | 0.12% | -1.27% | -1.24% | 100% | 101% |
| Class E | 0.11% | -1.12% | -1.82% | 101% | 103% |
| Overall | 0.10% | -1.54% | -1.74% | 103% | 102% |
| Class D | 0.06% | -0.93% | -0.59% | 100% | 101% |
| Class F | -0.11% | -1.97% | -2.37% | 102% | 102% |
| Class TGM | -0.76% | -1.45% | -1.44% | 105% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -2.15% | -4.25% | 105% | 102% |
| Class A2 | 0.09% | -2.52% | -1.36% | 105% | 101% |
| Class B | 0.11% | -4.51% | -5.01% | 104% | 101% |
| Class C | 0.05% | -2.22% | -2.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -3.03% | -3.32% | 104% | 101% |
| Class D | 0.05% | -1.87% | -1.45% | 101% | 100% |
| Class F | -0.23% | -2.87% | -2.95% | 106% | 101% |
| Class TGM | -1.15% | -3.03% | -3.24% | 117% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -11.74% | -11.40% | 106% | 102% |
| Class C | 0.06% | -5.96% | -5.11% | 102% | 102% |
| Class E | -0.18% | -2.94% | -6.82% | 104% | 102% |
| Overall | -0.13% | -7.61% | -8.16% | 104% | 102% |
| Class D | 0.16% | -6.25% | -5.04% | 101% | 100% |
| Class F | 0.00% | -7.46% | -7.04% | 109% | 102% |
| Class TGM | -2.23% | -7.59% | -7.60% | 123% | 98% |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, Jun. 2020, Teleconference.

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “AHG12: Cross-component Sample Adaptive Offset”, JVET-V0153, May 2021, Teleconference.

- J. Ström, P. Wennersten, K. Andersson, R. Sjöberg, S. Ikonin, E. Alshina, “EE2: Bilateral filter in VTM, EE2 and VVenC”, JVET-V0094, May 2021, Teleconference.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, May 2021, Teleconference.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).