| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0067-v1 |
| --- | --- |

| Title: | EE2-related: Implicit derivation of DIMD blend modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, a method of implicitly deriving the DIMD blend mode is proposed. In the multiple DIMD blend modes method which is studied in EE2 test 2.2, three DIMD blend modes are allowed and a DIMD index is signaled in the bitstream to indicate which DIMD blend mode is to be used. It is proposed to use the DIMD histogram information to implicitly select two of the three DIMD blend modes. It is reported that on top of ECM-1.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.10%, 0.00%, -0.03%, 107%, 101%} for AI and {-0.05%, -0.05%, -0.06%, 101%, 100%} for RA.

Introduction

The decoder-side intra mode derivation (DIMD) is proposed in JVET-U0100 [1] and is included in the ECM-1.0 [2]. First, a horizontal gradient and a vertical gradient are calculated for each reconstructed sample of the L-shaped template of the second neighboring row and column of the current block to build a Histogram of Gradients (HoG). Then, the two intra prediction modes with the largest and the second largest histogram amplitude values, M1 and M2, are blended with the Planar mode to generate the final prediction values of the current block. A DIMD flag is signaled to indicate whether to use DIMD for a block.

In JVET-V0087 [3], a multiple DIMD blend modes method is proposed to improve the coding efficiency of DIMD and is studied in EE2 test 2.2 [4]. It allows 3 different DIMD blend modes to be selected at encoder based on RD decision. A DIMD index binarized using truncated unary instead of the DIMD flag is signaled in the bitstream to indicate which DIMD blend mode is selected as shown in Table 1.

Table 1. The DIMD index and the corresponding DIMD blend mode in the multiple DIMD blend modes method.

| DIMD Index | DIMD blend patterns |
| --- | --- |
| 0 | DIMD is not applied |
| 1 | M1 + M2 |
| 2 | M2 + Planar |
| 3 | M1 + M2 + Planar |

Proposed method

In this contribution, it is proposed to use the DIMD histogram information to implicitly derive two of the three DIMD blend modes at both encoder and decoder. Specifically, when DIMD index is equal to 1, the proportion of the sum of the amplitude values of M1 and M2 (called R), is used to derive which of the two DIMD blend modes, M1 + M2 and M1 + M2 + Planar, is selected for the current block as shown in Table 2.

Table 2. The DIMD index and the corresponding DIMD blend mode in the proposed method.

| DIMD Index | DIMD blend patterns |  |
| --- | --- | --- |
| 0 | DIMD is not applied |  |
| 1 | if(R<3/8) | M1 + M2 + Planar |
|  | else | M1 + M2 |
| 2 | M2 + Planar |  |

In addition, the comparison of R in the Table 2 can be converted into the following form to avoid the division operation:

amplM1+amplM2<3*i=066ampl(i)≫3

, where ampli represents the amplitude value of mode i in the histogram. The proposed method can reduce the bit overhead required to signal the DIMD index and the time spent on encoder RD decision in the multiple DIMD blend modes method.

Simulation results

The proposed method is implemented on top of ECM-1.0. The simulation results are tested under ECM-1.0 test condition [5] with all intra and random access configurations. The encoder fast algorithm provided in the EE2 test 2.2 software is used for the proposed method to reduce the encoding time.

Table 3. Simulation results of the proposed method.

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-1.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.10% |  | 0.14% |  | -0.13% | 107% |  | 101% |  |
| Class A2 | -0.08% |  | -0.02% |  | 0.00% | 108% |  | 101% |  |
| Class B | -0.09% |  | 0.01% |  | -0.03% | 107% |  | 101% |  |
| Class C | -0.06% |  | 0.04% |  | -0.07% | 107% |  | 100% |  |
| Class E | -0.18% |  | -0.21% |  | 0.06% | 107% |  | 100% |  |
| Overall | -0.10% |  | 0.00% |  | -0.03% | 107% |  | 101% |  |
| Class D | -0.09% |  | 0.01% |  | 0.14% | 109% |  | 98% |  |
| Class F | 0.07% |  | 0.06% |  | 0.07% | 102% |  | 98% |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-1.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.07% |  | -0.20% |  | -0.11% | 102% |  | 100% |  |
| Class A2 | -0.05% |  | 0.09% |  | -0.02% | 102% |  | 100% |  |
| Class B | -0.03% |  | -0.06% |  | 0.00% | 101% |  | 100% |  |
| Class C | -0.06% |  | -0.02% |  | -0.12% | 101% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% |  | -0.05% |  | -0.06% | 101% |  | 100% |  |
| Class D | -0.05% |  | 0.27% |  | 0.11% | 101% |  | 101% |  |
| Class F | 0.01% |  | -0.17% |  | -0.02% | 100% |  | 99% |  |

For convenience, the simulation results of EE2 test2.2 [6] are shown in Table 4.

Table 4. Simulation results of EE2 test2.2.

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-1.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.11% |  | 0.04% |  | -0.06% | 120% |  | 101% |  |
| Class A2 | -0.09% |  | -0.01% |  | -0.01% | 119% |  | 101% |  |
| Class B | -0.08% |  | 0.02% |  | -0.05% | 121% |  | 101% |  |
| Class C | -0.07% |  | 0.10% |  | -0.02% | 120% |  | 102% |  |
| Class E | -0.17% |  | -0.11% |  | -0.06% | 121% |  | 101% |  |
| Overall | -0.10% |  | 0.02% |  | -0.04% | 120% |  | 101% |  |
| Class D | -0.06% |  | 0.07% |  | 0.13% | 119% |  | 101% |  |
| Class F | -0.06% |  | 0.08% |  | 0.01% | 111% |  | 100% |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-1.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.07% |  | -0.04% |  | -0.14% | 102% |  | 100% |  |
| Class A2 | -0.03% |  | 0.15% |  | -0.15% | 102% |  | 100% |  |
| Class B | -0.06% |  | 0.06% |  | 0.13% | 103% |  | 100% |  |
| Class C | -0.04% |  | -0.02% |  | -0.04% | 103% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% |  | 0.04% |  | -0.02% | 103% |  | 100% |  |
| Class D | -0.04% |  | -0.02% |  | -0.03% | 103% |  | 100% |  |
| Class F | -0.03% |  | -0.24% |  | 0.17% | 103% |  | 100% |  |

Conclusion

This contribution proposes an implicit derivation method of the multiple DIMD blend modes which achieves 0.10% and 0.05% gain in AI and RA configurations, respectively. Compared with EE2 test 2.2, the proposed method can effectively reduce the encoding time with similar coding performance. It is suggested to adopt in the next version of ECM software.

References

- Y.-J. Chang, et. al., “Compression efficiency methods beyond VVC,” JVET-U0100, Jan. 2021.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM.

- J. Zhao, et. al., “EE2-Related: Improvements of Decoder-Side Intra Mode Derivation,” JVET-V0087, Apr. 2021.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-V2024, Apr. 2021.

- M.Karczewicz, Y.Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

- J. Zhao, et. al., “EE2 Test 2.2: DIMD with multiple blending modes,” JVET-W0054, Jun. 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).