| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0070 |
| --- | --- |

| Title: | [AHG8] SPS Cleanup for VVC operation range extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Franck Galpin Tangi Poirier Fabrice Le Leannec | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

# Abstract

This contribution proposes a syntax cleanup of SPS flags related to VVC range extension. Specifically, it proposes signaling all/some of the range extension SPS flags only when BitDepth is higher than 10, otherwise it is inferred to 0. In addition, it proposes to signal SPS for TSRC rice parameter only when transform skip is enabled. The goal is to remove the redundant signaling and improve the readability of the specification.

# Introduction

The range extension of VVC targets improved coding for contents with high bit-depth, high bit-rate and high frame-rate. Currently, there are 4 new SPS parameters [1]:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

It is noted that both the extended precision and the persistent rice adaptation target high bit-depth coding only, i.e., sequences with bit-depth higher than 10. In essence, both tools are deactivating during the decoding process for bitDepths less than 10.

For extended_precision_processing_flag:

#### 7.4.3.22 Sequence parameter set range extension semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

The variable Log2TransformRange is derived as follows:

For sps_persistent_rice_adaptation_enabled_flag:

# Clause 9.3.2.1

The context variables of the arithmetic decoding engine are initialized as follows:

- If the CTU is the first CTU in a slice or tile, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0, and the following applies:

–	If sps_persistent_rice_adaptation_enabled_flag is equal to 0, the values of StatCoeff[ idx ] for idx ranging from 0 to 2, inclusive, are initialized to be equal to 0.

–	Otherwise the following applies:

StatCoeff[ idx ] = ( bitDepth > 10 ) ? ( 2 * Floor( Log2( bitDepth − 10 ) ) : 0	(X)

That is, regardless of their SPS flags values, the two tools are deactivated during the decoding process, which leads to both redundant signaling and misleading encoder configuration. On the other hand, the other tools are independent from the input bitDepth despite being developed and tested only for high bitDepth data.

# Proposed draft change

## Proposal #1

In this proposal, the SPS flag sps_range_extension_flag is conditionally signaled according to the value of bitDepth. That is, it is signaled only if (bitDepth > 10), otherwise it is inferred as zero. By doing so, the 4 SPS flags of the operation range extension will only be signaled when (bitDepth > 10). The corresponding specification change is highlighted below (blue for added part and red for removed part):

| if( sps_extension_flag ) { |  |
| --- | --- |
| if (bitDepth > 10){ |  |
| sps_range_extension_flag | u(1) |
| } |  |
| sps_extension_7bits | u(7) |
| if( sps_range_extension_flag ) |  |
| sps_range_extension( ) |  |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |

## Proposal #2

In this proposal, only the SPS related to extended precision flag and persistent rice parameter adaptation are conditionally coded. The advantage is that these tools would be restricted to only high bit-depth coding, while the rest are also allowed for high bit-rate and high frame-rate coding, even for low bit-depth input. The corresponding changes are:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| if (bitDepth > 10){ |  |
| extended_precision_processing_flag | u(1) |
| } |  |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| if (bitDepth > 10){ |  |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |
| } |  |

## Proposal #3

Independent from proposal #1 and proposal #2, it is proposed here to condition the signaling of sps_ts_residual_coding_rice_present_in_sh_flag on the TrSkip SPS flag (sps_transform_skip_enabled_flag). This is to avoid the unnecessary signaling when transform skip is deactivated.

The corresponding change is:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| if (sps_transform_skip_enabled_flag){ |  |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| } |  |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

# Conclusion

This contribution proposes three methods to clean up the SPS design of the specification of VVC operation range extension. All methods assert harmonized design with other SPS flags where redundant signaling is removed and improves the readability of the specification.

# Reference

[1] Frank Bossen et al., “VVC operation range extensions (Draft 3)”, JVET-V2005. Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).