| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0073-v3 |
| --- | --- |

| Title: | ALF parameters for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Galpin, F.Leleannec | Tel: Email: | philippe.bordes@interdigital.com franck.galpin@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VVC, RPR allows coding some frames at lower resolution to dynamically adapt to the available transport bit rate. For low bit rates, RPR allows competing with VVC at full resolution while reducing encoder and decoder complexity. However, its performance may depend on the video content and the chroma distortion may be significant compared to full-resolution coding. It is proposed to add an ALF post-filtering stage applied on the reconstructed up-sampled pictures when RPR is enabled. The BD-rate performance of RPR is improved in chroma by about 15% in RA, LDB and AI.

# Introduction

In VVC, RPR allows coding some frames at lower resolution to dynamically adapt to the available transport bit rate. In Table 1 and Table 2 one depicts the RPR performance of VVC using AHG-11 CTCs [1] with VTM-11-NNVC and ECM-1.0 software [2][3]. The cfg-1 stands for RPR with all frames down-sampled by 2 horizontally and vertically (quarter size). The cfg-3 is a combination of full size and quarter size coding (cfg-1) adaptively per sequences and target rate, keeping monotonous BD-rate curves. For low bit rates, RPR allows competing with VVC at full resolution while reducing encoder and decoder complexity. However, its performance may depend on the video content and the chroma distortion may be significant.

Table 1: VTM-11 NNVC and ECM-1.0 coding performance with RPR enabled (cfg-1) compared with full size coding.

|  | VTM-11 with RPR, cfg-1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main10 |  |  | Low delay B Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -8,38% | 12,18% | 7,42% | -10,68% | 10,84% | 3,22% | -6,20% | 62,58% | 12,66% |
| Class A2 | 3,69% | 63,84% | 39,30% | -1,98% | 64,92% | 44,10% | -2,36% | 36,72% | 25,07% |
| Overall | -2,35% | 38,01% | 23,36% | -6,33% | 37,88% | 23,66% | -4,28% | 49,65% | 18,87% |

|  | ECM-1.0 with RPR, cfg-1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main10 |  |  | Low delay B Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 |  | -6,44% |  | 18,53% |  | 13,13% |  |  |  |  |  |  |  | -5,24% |  | 74,48% |  | 18,06% |  |
| Class A2 |  | 4,92% |  | 70,94% |  | 44,33% |  |  |  |  |  |  |  | -1,87% |  | 42,00% |  | 27,86% |  |
| Overall | -0,76% | 44,73% | 28,73% |  |  |  | -3,56% | 58,24% | 22,96% |

Table 2: VTM-11 NNVC and ECM-1.0 coding performance with RPR enabled (cfg-3) compared with full size coding.

|  | VTM-11 with RPR, cfg-3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main10 |  |  | Low delay B Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -8,28% | 2,25% | 2,90% | -11,37% | 8,65% | 1,84% | -6,50% | 62,57% | 12,50% |
| Class A2 | -3,97% | 56,17% | 26,28% | -7,12% | 55,82% | 30,05% | -6,33% | 33,51% | 22,68% |
| Overall | -6,13% | 29,21% | 14,59% | -9,66% | 35,60% | 12,62% | -6,42% | 48,04% | 17,59% |

|  | ECM-1.0 with RPR, cfg-3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main10 |  |  | Low delay B Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -7,97% | 21,82% | 24,87% |  |  |  | -5,61% | 20,88% | 11,34% |
| Class A2 |  | -3,92% |  | 58,79% |  | -4,87% |  |  |  |  |  |  |  | -6,30% |  | 37,17% |  | 24,73% |  |
| Overall | -5,95% | 40,31% | 10,00% |  |  |  | -5,95% | 29,02% | 18,03% |

# Proposal

The current design of VVC allows post-filtering reconstructed pictures with ALF to reduce distortion with original pictures. However, in case RPR is enabled, ALF reduces distortion of the low-res reconstructed pictures with the low-res (down-sampled) original pictures.

When RPR is enabled, it is proposed to use an additional ALF process stage (ALF-2, CC-ALF-2) to reduce the distortion between the up-sampled reconstructed pictures and the original full-size pictures. We investigated two methods for applying ALF:

Method 1: ALF-2 and CC-ALF-2 are applied on the re-scaled pictures (Figure 1). The coefficients of ALF-2 and CC-ALF-2 are derived so that to minimize the distortion between the up-sampled reconstructed samples and the original full-size pictures (V1 in the figure)

Method 2: ALF-2 is applied on the reconstructed low-res pictures and CC-ALF-2 is applied on the re-scaled pictures. In this case, the coefficients of ALF-2 are derived so that to minimize the distortion between the low-res reconstructed samples and the corresponding samples in the original full-size pictures (phase-0) (V2 in the figure).

For the two methods, a variant is tested where ALF-1 and CC-ALF-1 are disabled (disable regular ALF).

Figure 1: method-1: ALF-2 and CC-ALF-2 are applied on the re-scaled pictures, psnr measured at V1 (V0 corresponds to the regular RPR).

Figure 2: method-2: ALF-2 applied on low-res pictures, next CC-ALF-2 applied on re-scaled pictures, psnr measured at V2.

# Experimental results

The proposed methods 1 and 2 have been implemented in VTM-11-NNVC software [2] with cfg-1 (all frames down-sampled quarter size) and cfg-3 (combination of full size and quarter size coding adaptively per sequences and target rate, keeping monotonous BD-rate curves).

## CFG-1

Random Access

Table 3: VTM-11 NNVC with RPR enabled (cfg-1) compared with full size coding (reference RPR).

|  | Random Access Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,38% | 12,18% | 7,42% |  |  |  | 53% | 31% |
| Class A2 | 3,69% | 63,84% | 39,30% |  |  |  | 46% | 31% |
| Overall | -2,35% | 38,01% | 23,36% |  |  |  | 49% | 31% |

Table 4: method-1.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,41% | 11,12% | 2,40% | 0,97% | -1,05% | -5,02% | 59% | 87% |
| Class A2 | 3,73% | 35,25% | 21,19% | 0,04% | -28,59% | -18,11% | 52% | 87% |
| Overall | -1,84% | 23,18% | 11,80% | 0,50% | -14,82% | -11,56% | 55% | 87% |

Table 3: method-1 with regular ALF disabled.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,35% | 10,00% | 1,32% | 1,03% | -2,18% | -6,10% | 57% | 82% |
| Class A2 | 3,42% | 30,10% | 15,40% | -0,27% | -33,74% | -23,90% | 50% | 80% |
| Overall | -1,97% | 20,05% | 8,36% | 0,38% | -17,96% | -15,00% | 53% | 81% |

Table 4: method-2.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,93% | 9,65% | 1,38% | 0,46% | -2,52% | -6,04% | 54% |  |
| Class A2 | 2,67% | 30,31% | 17,17% | -1,02% | -33,53% | -22,14% | 44% |  |
| Overall | -2,63% | 19,98% | 9,27% | -0,28% | -18,03% | -14,09% | 49% |  |

Table 4: method-2 with regular ALF disabled.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,79% | 9,72% | 1,31% | 0,59% | -2,46% | -6,11% | 55% |  |
| Class A2 | 2,47% | 27,29% | 14,42% | -1,22% | -36,55% | -24,88% | 47% |  |
| Overall | -2,66% | 18,50% | 7,87% | -0,32% | -19,51% | -15,49% | 51% |  |

From the results above, the best method is method-2 with regular ALF disabled.

LDB

Table 5: VTM-11 NNVC with RPR enabled (cfg-1) compared with full size coding (reference RPR).

|  | Low delay B Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -10,68% | 10,84% | 3,22% |  |  |  | 49% | 32% |
| Class A2 | -1,98% | 64,92% | 44,10% |  |  |  | 43% | 32% |
| Overall | -6,33% | 37,88% | 23,66% |  |  |  | 44% | 35% |

Table 6: method-1.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -9,00% | 11,00% | -0,16% | 1,68% | 0,16% | -3,38% | 53% | 86% |
| Class A2 |  | 0,42% |  | 42,80% |  | 29,18% |  | 2,40% |  | -22,12% |  | -14,92% |  | 47% |  | 89% |  |
| Overall | -4,29% | 26,90% | 14,51% | 2,04% | -10,98% | -9,15% | 50% | 87% |

Table 6: method-1 with regular ALF disabled.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,53% | 10,25% | -0,28% | 2,15% | -0,58% | -3,50% | 54% | 86% |
| Class A2 |  | 1,78% |  | 37,57% |  | 19,93% |  | 3,76% |  | -27,35% |  | -24,17% |  | 48% |  | 88% |  |
| Overall | -3,37% | 23,91% | 9,83% | 2,95% | -13,97% | -13,83% | 51% | 87% |

Table 7: method-2 with regular ALF disabled.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -9,46% | 9,44% | -1,19% | 1,21% | -1,40% | -4,41% | 49% |  |
| Class A2 | -0,42% | 33,83% | 17,53% | 1,56% | -31,08% | -26,57% | 44% |  |
| Overall | -4,94% | 21,64% | 8,17% | 1,39% | -16,24% | -15,49% | 46% |  |

From the results, the best method is method-2.

All Intra

Table 8: VTM-11 NNVC with RPR enabled (cfg-1) compared with full size coding(reference RPR).

|  | All Intra Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -6,20% | 62,58% | 12,66% |  |  |  | 83% | 38% |
| Class A2 | -2,36% | 36,72% | 25,07% |  |  |  | 72% | 40% |
| Overall | -4,28% | 49,65% | 18,87% |  |  |  | 63% | 39% |

Table 9: method-1.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,29% | 55,16% | 5,22% | -1,09% | -7,42% | -7,44% | 97% | 84% |
| Class A2 | -4,87% | 16,30% | 6,92% | -2,51% | -20,42% | -18,15% | 79% | 79% |
| Overall | -6,08% | 35,73% | 6,07% | -1,80% | -13,92% | -12,79% | 88% | 81% |

Table 9: method-1 with regular ALF disabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,15% | 52,75% | 4,57% | -0,94% | -9,83% | -8,09% | 93% | 78% |
| Class A2 | -4,69% | 14,15% | 4,18% | -2,33% | -22,57% | -20,89% | 77% | 74% |
| Overall | -5,92% | 33,45% | 4,38% | -1,64% | -16,20% | -14,49% | 84% | 76% |

Table 14: method-2 with regular ALF disabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -6,80% | 53,75% | 5,07% | -0,94% | -9,83% | -8,09% | 93% | 78% |
| Class A2 | -4,54% | 13,31% | 4,20% | -2,33% | -22,57% | -20,89% | 77% | 74% |
| Overall | -5,67% | 33,53% | 4,64% | -1,39% | -16,12% | -14,23% | 79% | 76% |

From the results, the best method is method-1.

## CFG-3

Random Access

Table 1: VTM-11 NNVC with RPR enabled (cfg-3) compared with full size coding (reference RPR).

|  | Random Access Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,28% | 2,25% | 2,90% |  |  |  |  |  |
| Class A2 | -3,97% | 56,17% | 26,28% |  |  |  |  |  |
| Overall | -6,13% | 29,21% | 14,59% |  |  |  |  |  |

Table 1: method-1.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,62% | 11,06% | 2,07% | 0,66% | 8,81% | -0,83% | 67% | 91% |
| Class A2 | -3,64% | 30,87% | 13,92% | 0,33% | -25,30% | -12,36% | 70% | 91% |
| Overall | -5,63% | 20,97% | 7,99% | 0,50% | -8,25% | -6,60% | 69% | 91% |

Table 1: method-1 with regular ALF disabled.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -7,61% | 9,80% | 0,94% | 0,67% | 7,55% | -1,96% | 66% | 88% |
| Class A2 | -3,88% | 26,53% | 10,43% | 0,09% | -29,64% | -15,85% | 68% | 87% |
| Overall | -5,74% | 18,17% | 5,68% | 0,38% | -11,04% | -8,91% | 67% | 87% |

Table 1: method-2.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,46% | 14,38% | 0,24% | -0,18% | 12,13% | -2,66% | 66% |  |
| Class A2 | -4,92% | 27,01% | 10,20% | -0,95% | -29,16% | -16,08% | 59% |  |
| Overall | -6,69% | 20,70% | 5,22% | -0,57% | -8,52% | -9,37% | 62% |  |

Table 1: method-2 with regular ALF disabled.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,33% | 14,25% | -0,03% | -0,05% | 12,00% | -2,93% | 67% |  |
| Class A2 | -5,03% | 24,28% | 8,90% | -1,06% | -31,89% | -17,38% | 62% |  |
| Overall | -6,68% | 19,27% | 4,44% | -0,56% | -9,95% | -10,16% | 64% |  |

From the results, the best method is method-2 with regular ALF disabled

LDB

Table 5: VTM-11 NNVC with RPR enabled (cfg-3) compared with full size coding (reference RPR).

|  | Low delay B Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -11,37% | 8,65% | 1,84% |  |  |  | 65% | 42% |
| Class A2 | -7,12% | 55,82% | 30,05% |  |  |  | 64% | 48% |
| Overall | -9,66% | 35,60% | 12,62% |  |  |  | 68% | 55% |

Table 5: method-1.

|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Psnr 5 QPs |  |  |  |  | Diff vs reference RPR |  |  |  |  | Time |  |  |
|  |  | Y |  | U | V |  | Y |  | U | V |  | EncT |  | DecT |
| Class A1 |  | -9,00% |  | 11,00% | -0,16% |  | 2,37% |  | 2,35% | -2,00% |  | 60% |  | 90% |
| Class A2 |  | -5,52% |  | 32,74% | 15,87% |  | 1,60% |  | -23,08% | -14,18% |  | 61% |  | 93% |
| Overall |  | -7,26% |  | 21,87% | 7,86% |  | 1,99% |  | -10,37% | -8,09% |  | 46% |  | 78% |

Table 5: method-1 with regular ALF disabled.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -8,57% | 10,30% | -0,22% | 2,80% | 1,65% | -2,06% | 56% | 88% |
| Class A2 | -4,71% | 27,28% | 10,43% | 2,41% | -28,54% | -19,62% | 65% | 95% |
| Overall | -6,64% | 18,79% | 5,10% | 2,61% | -13,45% | -10,84% | 47% | 82% |

Table 5: method-2 with regular ALF disabled.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 |  | -10,11% |  | 15,46% |  | -1,47% |  | 1,26% |  | 6,81% |  | -3,31% |  | 55% |  |  |  |
| Class A2 | -6,47% | 24,05% | 8,34% | 0,65% | -31,77% | -21,71% | 61% |  |
| Overall | -8,29% | 19,76% | 3,43% | 0,96% | -12,48% | -12,51% | 58% |  |

From the results, the best method is method-2.

All Intra

Table 8: VTM-11 NNVC with RPR enabled (cfg-3) compared with full size coding (reference RPR).

|  | All Intra Main10 (reference RPR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -6,50% | 62,57% | 12,50% |  |  |  | 88% | 45% |
| Class A2 | -6,33% | 33,51% | 22,68% |  |  |  | 88% | 53% |
| Overall | -6,42% | 48,04% | 17,59% |  |  |  | 88% | 49% |

Table 8: method-1.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -9,50% | 50,80% | 4,53% | -3,00% | -11,77% | -7,97% | 106% | 90% |
| Class A2 | -7,42% | 16,98% | 8,18% | -1,09% | -16,53% | -14,50% | 86% | 82% |
| Overall | -8,46% | 33,89% | 6,36% | -2,05% | -14,15% | -11,24% | 95% | 86% |

Table 8: method-1 with regular ALF disabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -9,20% | 49,63% | 4,53% | -2,70% | -12,94% | -7,97% | 102% | 83% |
| Class A2 | -7,11% | 14,95% | 6,06% | -0,78% | -18,56% | -16,62% | 84% | 78% |
| Overall | -8,16% | 32,29% | 5,30% | -1,74% | -15,75% | -12,30% | 93% | 81% |

Table 8: method-2 with regular ALF disabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Diff vs reference RPR |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -6,96% | 54,37% | 5,54% | -0,46% | -8,20% | -6,96% | 92% | 82% |
| Class A2 | -7,55% | 13,81% | 5,46% | -1,22% | -19,70% | -17,22% | 80% | 78% |
| Overall | -7,26% | 34,09% | 5,50% | -0,84% | -13,95% | -12,09% | 86% | 80% |

From the results, the best method is method-1.

# Conclusion

It is proposed to add a ALF post-filtering stage applied on the reconstructed up-sampled pictures when RPR is enabled. The BD-rate performance of RPR is improved in chroma by about 15% in RA, LDB and AI, and also provides some gains in luma components. It is proposed to include RPR in EE2 CTCs.

# References

- S. Liu, A.Segall, E.Alshina, R.Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-V2016.

- Version 11.0_nnvc-1.0 of the VTM NNVC software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).