| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0074 |
| --- | --- |

| Title: | HLS for ALF parameters for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Galpin, K.Naser, F.Leleannec | Tel: Email: | philippe.bordes@interdigital.com franck.galpin@interdigital.com karam.naser@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The contribution proposes an HLS compatible with VVC for signaling additional ALF post-filter parameters for reducing distortion of the up-sampled RPR pictures with original full-size pictures.

# Introduction

RPR allows coding some frames at lower resolution to dynamically adapt to the available transport bit rate. For low bit rates, RPR may compete with VVC at full resolution while reducing encoder and decoder complexity. However, its performance may depend on the video content and the chroma distortion may be significant.

In [1], it is shown one can reduce the chroma distortion of the up-sampled reconstructed pictures with an additional ALF post-filtering stage, applied on the up-sampled reconstructed pictures.

# Proposal

In VVC, the regular ALF parameters are coded at several places in the bitstream:

- SPS: sps_alf_enabled_flag, sps_ccalf_enabled_flag.

- PH,SH: ph_alf_enabled_flag, ph_alf_cb_enabled_flag, ph_alf_cr_enabled_flag, ph_alf_cc_cb_enabled_flag, ph_alf_cc_cr_enabled_flag, and aps_ids.

- APS: The APS of type ALF_APS contains filters coefficients alf_data()

- CTU: alf_ctb_flag[], alf_ctb_filter_alt_idx[], alf_ctb_cc_cb_idc, alf_ctb_cc_cr_idc and idx.

It is proposed HLS compatible with VVC for signaling additional ALF parameters describing another ALF process stage applied on the up-sampled reconstructed pictures.

Method 1: SEI and new APS, or SEI and re-use of APS.

A new SEI message contains SPS, PH,SH and CTU ALF parameters for the up-sampled pictures, and a new APS type ALF_RPR_APS contains the filters coefficients alf_data(). To avoid creating a new APS type, one can re-use existing ALF_APS but since the range value of aps_adaptation_parameter_set_id is [0,..7] the encoder should reserve apsId values for the regular ALF and others for the new ALF.

Method 2: use scalable VVC.

The HLS for scalable VVC allows for specifying another layer with regular ALF parameters. It can be used to perform up-scaling of the reconstructed base layer (BL) pictures as follows:

- If the value of picture size coded in the SPS and PPS of the enhancement layer (EL) is equal to the SPS picture size of the base layer (sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples), the CUs predicted with the ILRP uses implicit RPR up-sampling.

- In case all the CUs of the EL are coded without any motion nor residuals. The EL is equivalent to the base layer picture with an up-sampling using the RPR filters. In order to perform efficiently a skip mode, the following encoding choices are made:

  - IBC mode is disabled at picture/slice level.

  - Affine mode and temporal mode are disabled at picture/slice level

  - Ciip and geo mode are disabled at picture/slice level

  - mmvd is disabled at picture/slice level

  - the maximum number of merge candidate is fixed to 1

- in this case, for each CU, only the skip flag is read (and set to 1), the merge mode is deduced as regular without mmvd and the merge index is deduced to be 0. The only motion candidate being the null vector and the mode being skip, the whole slice/picture is coded as a direct copy of the BL after up-scaling.

- The DBF and SAO are disabled in the EL, and only ALF is applied. The final EL frame ends up to be the BL upscaled with RPR and the ALF filters applied on it.

Compared to method-1, it has advantage to not specify any additional syntax element. However, it supposes instancing two full-size VVC decoders since the end device may not presume what will be the coding modes in the EL. This could be solved with a new syntax (ex: SEI) signaling that the coded picture or the EL is skipped, except ILRP up-sampling and post-filtering (all CTUs coded in skip mode), allowing the decoder to alleviate the amount of resources dedicated to EL significantly.

# Proposed Syntax

The proposed methods 1 and 2 have been implemented int VTM-11-NNVC software [2] with cfg-1 (all frames down-sampled quarter size) and cfg-3 (combination of full size and quarter size coding adaptively per sequences and target rate, keeping monotonous BD-rate curves).

Method 1: SEI and new APS, or SEI and re-use of APS.

Table 1: syntax of SEI reconstructed picture rescaled ALF.

|  | sei_alf_rpr_parameters( payloadSize ) { | Descriptor |
| --- | --- | --- |
|  | alf_rpr_enabled_flag | u(1) |
|  | ccalf_rpr_enabled_flag | u(1) |
|  | rpr_chroma_format_idc |  |
|  | log2_rpr_ctu_size_minus5 | u(2) |
|  | nb_rpr_ctu_x_minus1 | ue(v) |
|  | nb_rpr_ctu_y_minus1 | ue(v) |
|  | poc_base | u(v) |
|  | num_alf_rpr_param_minus1 | u(v) |
|  | for( h = 0; h <= num_alf_rpr_param_minus1; h++ ) { | u(1) |
|  | delta_poc[h] | se(v) |
|  | if ( alf_rpr_enabled_flag ) { |  |
| SH,PH parameters | rpr_alf_enabled_flag[h] | u(1) |
|  | if( rpr_alf_enabled_flag[h] ) { |  |
|  | rpr_num_alf_aps_ids_luma[h] | u(3) |
|  | for( i = 0; i < rpr_num_alf_aps_ids_luma[h]; i++ ) |  |
|  | rpr_alf_aps_id_luma[ h ][ i ] | u(3) |
|  | if( rpr_chroma_format_idc != 0 ) { |  |
|  | rpr_alf_cb_enabled_flag[h] | u(1) |
|  | rpr_alf_cr_enabled_flag[h] | u(1) |
|  | } |  |
|  | if( rpr_alf_cb_enabled_flag[h] ¦ ¦ rpr_alf_cr_enabled_flag[h] ) |  |
|  | rpr_alf_aps_id_chroma[h] | u(3) |
|  | if( ccalf_rpr_enabled_flag ) { |  |
|  | rpr_alf_cc_cb_enabled_flag[h] | u(1) |
|  | if( rpr_alf_cc_cb_enabled_flag[h] ) |  |
|  | rpr_alf_cc_cb_aps_id[h] | u(3) |
|  | rpr_alf_cc_cr_enabled_flag[h] | u(1) |
|  | if( rpr_alf_cc_cr_enabled_flag[h] ) |  |
|  | rpr_alf_cc_cr_aps_id[h] | u(3) |
|  | } |  |
|  | } |  |
|  | } |  |
|  | for( CtbAddrY = 0; CtbAddrY <= nb_rpr_ctu_y_minus1; CtbAddrY ++ ) { |  |
|  | for( CtbAddrX = 0; CtbAddrX <= nb_rpr_ctu_x_minus1; CtbAddrX ++ ) { |  |
| Alf CTU parameters | if( rpr_alf_enabled_flag ){ |  |
|  | rpr_alf_ctb_flag[h][ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | if( alf_ctb_flag[h][ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
|  | if( rpr_num_alf_aps_ids_luma[h] > 0 ) |  |
|  | rpr_alf_use_aps_flag[h] | ae(v) |
|  | if( rpr_alf_use_aps_flag ) { |  |
|  | if( rpr_num_alf_aps_ids_luma[h] > 1 ) |  |
|  | rpr_alf_luma_prev_filter_idx[h] | ae(v) |
|  | } else |  |
|  | rpr_alf_luma_fixed_filter_idx[h] | ae(v) |
|  | } |  |
|  | if( rpr_alf_cb_enabled_flag[h] ) { |  |
|  | rpr_alf_ctb_flag[h][ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | if( alf_ctb_flag[h] [ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
|  | rpr_alf_ctb_filter_alt_idx[h][ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | } |  |
|  | if( rpr_alf_cr_enabled_flag[h] ) { |  |
|  | rpr_alf_ctb_flag[h][ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | if( alf_ctb_flag[h][ 2 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
|  | rpr_alf_ctb_filter_alt_idx[h][ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | } |  |
|  | } |  |
|  | if( rpr_alf_cc_cb_enabled_flag[h] ) |  |
|  | rpr_alf_ctb_cc_cb_idc[h][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | if( rpr_alf_cc_cr_enabled_flag ) |  |
|  | rpr_alf_ctb_cc_cr_idc[h][ CtbAddrX ][ CtbAddrY ] | ae(v) |
|  | } |  |
|  | } |  |

Table 2: APS modified syntax to support high-res ALF.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(5) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = ALF_RPR_APS ) |  |
| alf_data_rpr( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | ALF_RPR_APS | High-res ALF parameters |

Method 2: use scalable VVC.

Table 3: syntax of SEI coding hint information.

| coding_hint_info( payloadSize ) { | Descriptor |
| --- | --- |
| coding_hint_cancel_flag | u(1) |
| if( !coding_hint_cancel_flag ) { |  |
| all_cu_skip_ilrp | u(1) |
| } |  |
| } |  |

## Proposed semantics

Method 1: SEI and new APS, or SEI and re-use of APS.

A SEI reconstructed picture rescaled ALF provides ALF parameters for adaptive loop filter process applied on reconstructed pictures re-scaled (rpr).

alf_rpr_enabled_flag equal to 1 specifies that ALF is enabled for the rpr pictures with POC listed in the SEI. alf_rpr_enabled_flag equal to 0 specifies that ALF is disabled for the reconstructed pictures with POC listed in the SEI.

ccalf_rpr_enabled_flag equal to 1 specifies that CCALF is enabled for the rpr pictures with POC listed in the SEI. ccalf_rpr_enabled_flag equal to 0 specifies that CCALF is disabled for the rpr pictures with POC listed in the SEI.

rpr_chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

log2_rpr_ctu_size_minus5 plus 5 specifies the block size for applying rpr ALF. The value of log2_rpr_ctu_size_minus5 shall be in the range of 0 to 2, inclusive. The value 3 for log2_rpr_ctu_size_minus5 is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore the rpr SEIs with log2_rpr_ctu_size_minus5 equal to 3.

The variables CtbLog2SizeY and CtbSizeY are derived as follows:

CtbLog2SizeY = log2_rpr_ctu_size_minus5 + 5

CtbSizeY = 1  <<  CtbLog2SizeY

nb_rpr_ctu_x_minus1 plus 1 specifies the horizontal number of rpr ALF blocks parameters.

nb_rpr_ctu_y_minus1 plus 1 specifies the vertical number of rpr ALF blocks parameters.

poc_base specifies the POC of the first picture in the POC list.

num_alf_rpr_param_minus1 plus 1 specifies the POC list size.

delta_poc[ i ] plus poc_base specifies the POC of the i-th picture in the POC list.

rpr_alf_enabled_flag[ i ] equal to 1 specifies that rpr ALF is enabled for the Y, Cb, or Cr colour component of the i-th picture. rpr_alf_enabled_flag[ i ] equal to 0 specifies that ALF is disabled for all colour components in the i-th picture. When not present, the value of rpr_alf_enabled_flag[ i ] is inferred to be equal to 0.

rpr_num_alf_aps_ids_luma[ i ] specifies the number of ALF APSs that the i-th picture refers to.

rpr_alf_aps_id_luma[ i ][ j ] specifies the aps_adaptation_parameter_set_id of the j-th ALF APS that the luma component of the i-th picture refers to.

When present, rpr_alf_cb_enabled_flag[ i ] equal to 1 specifies that the rpr adaptive loop filter is enabled for the Cb colour component of the i-th picture, and rpr_alf_cb_enabled_flag[ i ] equal to 0 specifies that the rpr adaptive loop filter is disabled for the Cb colour component of the i-th picture. When rpr_alf_cb_enabled_flag[ i ] is not present, it is inferred to be equal to 0.

When present, rpr_alf_cr_enabled_flag[ i ] equal to 1 specifies that the rpr adaptive loop filter is enabled for the Cr colour component of the i-th picture, and rpr_alf_cr_enabled_flag[ i ] equal to 0 specifies that the rpr adaptive loop filter is disabled for the Cr colour component of the i-th picture. When rpr_alf_cr_enabled_flag[ i ] is not present, it is inferred to be equal to 0.

ph_alf_aps_id_chroma[ i ] specifies the aps_adaptation_parameter_set_id of the ALF APS that the chroma component of the i-th picture refers to.

When present, rpr_alf_cc_cb_enabled_flag[ i ]  equal to 1 specifies that the cross-component adaptive loop filter for the Cb colour component is enabled for the i-th picture, and rpr_alf_cc_cb_enabled_flag[ i ] equal to 0 specifies that the cross-component adaptive loop filter for the Cb colour component is disabled for the i-th picture. When not present, the value of rpr_alf_cc_cb_enabled_flag[ i ] is inferred to be equal to 0.

rpr_alf_cc_cb_aps_id[ i ] specifies the aps_adaptation_parameter_set_id of the ALF APS that the Cb colour component of the i-th picture refers to.

When present, rpr_alf_cc_cr_enabled_flag[ i ]  equal to 1 specifies that the cross-component adaptive loop filter for the Cr colour component is enabled for the i-th picture, and rpr_alf_cc_cr_enabled_flag[ i ] equal to 0 specifies that the cross-component adaptive loop filter for the Cr colour component is disabled for the i-th picture. When not present, the value of rpr_alf_cc_cr_enabled_flag[ i ] is inferred to be equal to 0.

rpr_alf_cc_cr_aps_id[ i ] specifies the aps_adaptation_parameter_set_id of the ALF APS that the Cr colour component of the i-th picture refers to.

rpr_alf_ctb_flag[ i ][ cIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that the rpr adaptive loop filter is applied to the block of the colour component indicated by cIdx of the block at luma location ( xCtb, yCtb ). rpr_alf_ctb_flag[ i ][ cIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the rpr adaptive loop filter is not applied to the block of the colour component indicated by cIdx of the block at luma location ( xCtb, yCtb ).

When rpr_alf_ctb_flag[ cIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

rpr_alf_use_aps_flag[ i ] equal to 0 specifies that one of the fixed filter sets is applied to the luma block. rpr_alf_use_aps_flag[ i ] equal to 1 specifies that a filter set from an APS is applied to the luma block. When rpr_alf_use_aps_flag[ i ] is not present, it is inferred to be equal to 0.

rpr_alf_luma_prev_filter_idx[ i ] specifies the previous filter that is applied to the luma block. The value of rpr_alf_luma_prev_filter_idx[ i ] shall be in a range of 0 to rpr_num_alf_aps_ids_luma − 1, inclusive. When rpr_alf_luma_prev_filter_idx[ i ] is not present, it is inferred to be equal to 0.

rpr_alf_luma_fixed_filter_idx[ i ] specifies the fixed filter that is applied to the luma block. The value of rpr_alf_luma_fixed_filter_idx[ i ] shall be in a range of 0 to 15, inclusive.

rpr_alf_ctb_filter_alt_idx[ i ][ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifies the index of the alternative chroma filter applied to the block of the chroma component, with chromaIdx equal to 0 for Cb and chromaIdx equal 1 for Cr, of the block at luma location ( xCtb, yCtb ). When rpr_alf_ctb_filter_alt_idx[ i ][ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to zero.

rpr_alf_ctb_cc_cb_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the block of the Cb colour component at luma location ( xCtb, yCtb ). When rpr_alf_cc_cb_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] not equal to 0, rpr_alf_cc_cb_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] − 1 specifies the filter set index of the cross-component filter applied to the block of the Cb colour component at luma location ( xCtb, yCtb ).

When alf_ctb_cc_cb_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

rpr_alf_ctb_cc_cr_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the block of the Cr colour component at luma location ( xCtb, yCtb ). When rpr_alf_cc_cr_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] not equal to 0, rpr_alf_cc_cr_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] − 1 specifies the filter set index of the cross-component filter applied to the block of the Cr colour component at luma location ( xCtb, yCtb ).

When rpr_alf_ctb_cc_cr_idc[ i ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

Method 2: use scalable VVC.

coding_hint_cancel_flag equal to 1 indicates that the coding hint information SEI message cancels the persistence of any previous coding hint information SEI message in output order that applies to the current layer. Coding_hint_cancel_flag equal to 0 indicates that coding hint information follows.

coding_hint_flag specifies the persistence of the coding hint information SEI message.

coding_hint_persistence_flag equal to 0 specifies that the resampling information applies to the current decoded picture only. Let picA be the current picture. coding_hint_persistence_flag equal to 1 specifies that the resampling information persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	A picture picB in the current layer in an access unit containing a coding hint information SEI message applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

all_cu_skip_ilrp equal to 1 specifies that all the CU of the CLVS are coded in skip mode. If the current layer is not independent, all the reference indexes are equal to ILRP. all_cu_skip equal to 0 specifies that some CU of the CLVS may be coded in another mode than skip mode. If the current layer is not independent, some reference idx may be different from ILRP.

# References

- P.Bordes, F.Galpin, “"ALF parameters for RPR," 23rd Meeting, by teleconference, 7–16 July 2021, document JVET-Wxxxx.

- B.Choi, Z.Li, W.Wang, W.Jiang, “AHG9/AHG11: SEI message for carriage of neural network information for post filtering,” 21st Meeting, by teleconference, 6–15 Jan. 2021, document JVET-U0091.

- H.Teo, H.Sun, C.Lim, “AHG9: ALF control SEI message,” 20th Meeting, by teleconference, 7 – 16 Oct. 2020, document JVET-T0076.

- M.Hannuksela, E.Aksu, F.Cricri, H.Tavakoli, ” AHG9: On post-filter SEI,” 22nd Meeting, by teleconference, 20–28 Apr. 2021, document: JVET-V0058.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).