| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0076 |
| --- | --- |

| Title: | AHG9: Independently coded region output SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Xiang Li Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

A new SEI message “Independently coded region output SEI message” that carries useful information for individual access and output of an independently coded region is proposed. The proposed SEI message can be used for cropping and outputting one or more specific rectangular regions which may consists of one or more subpictures. When a decoder system tries to early access and use one or more specific rectangular regions by discarding other regions, the information carried by the proposed SEI message can be used for cropping and outputting the rectangular regions which may consists of one or more subpictures. It is asserted that the use cases of the proposed SEI message include immersive media (VR360, point cloud coding) and game/e-sport contents.

# Introduction

Various immersive media (VR360, point cloud) and game/e-sport streaming applications need the support of partial access of one or more subpictures instead of full access of the entire picture. For example, usually for VR360 applications, only partial regions corresponding to the current viewport of the user are displayed on head-mount display or other portal devices. Video-based point cloud applications also need the support of regional access of contents, to display a specific projected view from the volumetric content.

In VVC version 1, the subpicture partitioning functionality is supported. When subpic_treated_as_pic_flag[i] in SPS is equal to 1, the boundaries of the i-th subpicture are treated as picture boundary, so that the boundaries may be padded for motion compensated prediction. An independently coded region associated with one or more subpictures can be early accessed by a system instead of full decoding of the entire picture. As long as the independently coded region has a rectangular shape, the coded data of the region can be accessed, decoded and outputted independently.

The proposed independent region output window SEI message carrys width, height, associated subpicture IDs, and output window offsets. The proposed SEI message can be used for cropping and outputting one or more specific rectangular regions which may consists of one or more subpictures. When a decoder system tries to early access and use one or more specific rectangular regions by discarding other regions, the information carried by the proposed SEI message can be used for cropping and outputting the rectangular regions which may consists of one or more subpictures.

Figure 1. An example of output windows for a 360 image projected by cubemap with six faces and padded regions.

For example, a 360 video can be projected with a cubemap projection. Six faces may be rearranged onto a picture with padded regions to alleviate the boundary discontinuity, as shows in Fig. 1. Then, the picture can be partitioned into six sub-pictures. Depending on the current viewport, a decoder can output only the target faces, which are composed of one or more of subpictures, by discarding the padded regions and other subpictures (faces) with the cropping information in the proposed SEI message.

For general purposes of the feature, each independent region may be overlapped with each other. Since an independent region can be treated as a group of subpictures, a subpicture may belong to multiple independent regions. Also, an independent region can nest another independent region, as long as each independent region is independently decodable. For example, Fig.2 shows an example with subpicture partitioning (dotted yellow line) and an indpendent region (red box). The independent region is composed four subpictures and has an output window (blue box).

Figure 2. An example of independent region output windows to output multiple region of interest for e-sport contents.

# Proposed spec text

independent regions output SEI message syntax

| Independent_region_output ( payloadSize ) { | Descriptor |
| --- | --- |
| independent_region_output_cancel_flag | u(1) |
| if( !independent_region_output_cancel_flag ) { |  |
| independent_region_output_persistence_flag | u(1) |
| ir_pic_width_in_luma_samples | ue(v) |
| ir_pic_height_in_luma_samples | ue(v) |
| ir_num_indepen_region_minus1 | ue(v) |
| ir_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= ir_num_indepen_region_minus1; i++ ) { |  |
| ir_num_subpics_minus1[ i ] | ue(v) |
| for( j = 0; j <= ir_num_subpics_minus1[ i ]; j++ ) |  |
| ir_subpic_id[ i ] [ j ] | u(v) |
| ir_indepen_coded_region_left_offset[ i ] | ue(v) |
| ir_indepen_coded_region_top_offset[ i ] | ue(v) |
| ir_indepen_coded_region_width_in_luma_samples[ i ] | ue(v) |
| ir_indepen_coded_region_height_in_luma_samples[ i ] | ue(v) |
| ir_indepen_coded_region_output_window_present_flag [ i ] | u(1) |
| if( ir_indepen_coded_region_output_window_present_flag [ i ] ) { |  |
| ir_output_win_left_offset[ i ] | ue(v) |
| ir_output_win_right_offset[ i ] | ue(v) |
| ir_output_win_top_offset[ i ] | ue(v) |
| ir_output_win_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The independent region output (IRO) SEI message provides information on region-wise output.

When a IRO SEI is present, the associated picture shall have PicOutputFlag equal to 1. The offset values of the independent regions output window SEI are applied to the decoded picture before cropping. When the i-th independent region is accessed and ir_indepen_coded_region_output_window_present_flag[ i ] is equal to 1, the cropping window specified by ir_output_win_left_offset[ i ], ir_output_win_right_offset[ i ], ir_output_win_top_offset[ i ] and ir_output_win_bottom_offset[ i ] is applied for cropping instead of the cropping window specified by PPS.

independent_region_output_cancel_flag equal to 1 indicates that the IRO SEI message cancels the persistence of any previous IRO SEI message in output order that applies to the current layer. independent_region_output_cancel_flag equal to 0 indicates that independent region output information follows, and when a decoder has a suitable output interface, it may output each independent region right after its decoding has been completed.

independent_region_output_persistence_flag specifies the persistence of the IRO SEI message for the current layer.

independent_region_output_persistence_flag equal to 0 specifies that the IRO SEI message applies to the current decoded picture only.

independent_region_output_persistence_flag equal to 1 specifies that the IRO SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a IRO SEI message is output that follows the current picture in output order.

ir_pic_width_in_luma_samples specifies the width of each decoded picture associated with this SEI message in units of luma samples. ir_pic_width_in_luma_samples shall not be equal to 0.

ir_pic_height_in_luma_samples specifies the height of each decoded picture associated with this SEI message in units of luma samples. ir_pic_height_in_luma_samples shall not be equal to 0.

ir_num_indepen_coded_region_minus1 plus 1 specifies the number of the independently coded regions of the PU containing this SEI message.

ir_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element ir_subpic_id[ i ]. The value of subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive

ir_num_subpics_minus1[ i ] plus 1 specifies the number of the subpictures associated with the i-th independently coded region.

ir_subpic_id[ i ][ j ] specifies the subpicture ID of the j-th subpicture associated with the i-th independently coded region. The length of the ir_subpic_id[ i ] syntax element is ir_subpic_id_len_minus1 + 1 bits.

The subpicture with ir_subpic_id[ i ][ j ] shall have subpic_treated_as_pic_flag[i] equal to 1.

ir_indepen_coded_region_left_offset[ i ] and ir_indepen_coded_region_top_offset[ i ] specifies the left-top position of the i-th indepently coded region. ir_indepen_coded_region_left_offset[ i ] and /ir_indepen_coded_region_top_offset[ i ] shall not be equal to 0 and shall be less than ir_pic_width_in_luma_samples and ir_pic_height_in_luma_amples, respectively.

ir_indepen_coded_region_width_in_luma_samples[ i ] specifies the width of the i-th indepently coded region. ir_indepen_coded_region_width_in_luma_samples[ i ] shall not be equal to 0.

ir_indepen_coded_region_height_in_luma_samples[ i ] specifies the height of the i-th indepently coded region. ir_indepen_coded_region_height_in_luma_samples[ i ] shall not be equal to 0.

ir_indepen_coded_region_output_window_present_flag [ i ] equal to 1 indicates that the i-th independently coded region output cropping window offset parameters follow next in the SEI message. ir_indepen_coded_region_output_window_present_flag [ i ] equal to 0 indicates that the the i-th independently coded region output cropping window offset parameters are not present.

ir_output_win_left_offset[ i ], ir_output_win_right_offset[ i ], ir_output_win_top_offset[ i ], and ir_output_win_bottom_offset[ i ] specify a rectangular region specified in picture coordinates for output of the i-th independently coded region. When ir_indepen_coded_region_output_window_present_flag [ i ] is equal to 0, the values of ir_output_win_left_offset[ i ], ir_output_win_right_offset[ i ], ir_output_win_top_offset[ i ], and ir_output_win_bottom_offset[ i ] are inferred to be equal to 0.

Note: Each independent coded region may be overalpped with each other.

– Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| independent region output window | Specified by the syntax of the SEI message |

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).