| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0077 |
| --- | --- |

| Title: | AHG9: Comments on Multiview-related SEI messages in VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The following modifications related to scalability dimension information (SDI) SEI message and multiview acquisition information (MAI) SEI message in VSEI are proposed:

Proposal-1: Defining the variable ViewId in the semantics of SDI SEI message.

Proposal-2: Clarifying the target layer that the MAI SEI message is applied to.

# Proposal-1

In the latest draft of the VSEI amendment in JVET-V2006-v1, the variable ViewId is not defined, but referred by several semantics. To resolve the problem, the following modification on the semantics of sdi_view_id_val[ i ] is proposed.

Proposed text changes:

sdi_view_id_val[ i ] specifies the view identifier value of the variable ViewId of the i-th layer in the current CVS. The length of the sdi_view_id_val[ i ] syntax element is sdi_view_id_len_minus1 + 1 bits.

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

NOTE 1 – disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table Y2 are derived from the respective variables in the s, e, n and v columns of Table Y2 as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 2 – The above specification is similar to that found in IEC 60559:1989.

Table Y2 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

# Proposal-2

In HEVC/H.265, the target layer that the Multiview acquisition information (MAI) SEI message is applied to is clearly specified by the following semantics:

G.14.3.6 Multiview acquisition information SEI message semantics … In the semantics below, index i refers to the syntax elements and variables that apply to the layer with nuh_layer_id equal to nestingLayerIdList[ 0 ][ i ]. …

However, in the current draft of VSEI, the target layer that is associated with the intrinsic and the extrinsic parameters of MAI SEI message is not clearly specified. To address the issue, one of the following options needs to be reflected.

Proposed text changes:

8.20.2 Multiview acquisition information SEI message semantics

The multiview acquisition information (MAI) SEI message specifies various parameters of the acquisition environment for the layers that may be present in the current CVS, i.e., the CVS containing the MAI SEI message. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

When an MAI SEI message is present in any AU of a CVS, an MAI SEI message shall be present for the first AU of the CVS. All MAI SEI messages in a CVS shall have the same content.

An MAI SEI message shall not be contained in a scalable nesting SEI message.

When a CVS does not contain an SDI SEI message, the CVS shall not contain an MAI SEI message.

When an AU contains both an SDI SEI message and an MAI SEI message, the SDI SEI message shall precede the MAI SEI message in decoding order.

Some of the views for which the MAI is included in an MAI SEI message may not be present in the current CVS.

Option-1: In the semantics below, index i refers to the syntax elements and variables that apply to the j-th layer in the current CVS with sdi_view_id_val[ j ] equal to sdi_view_id_val[ i ].

Option-2 (if the proposal-1 is adopted) In the semantics below, index i refers to the syntax elements and variables that apply to the layer in the current CVS with ViewId equal to sdi_view_id_val[ i ].

The extrinsic camera parameters are specified according to a right-handed coordinate system, where the upper left corner of the image is the origin, i.e., the ( 0, 0 ) coordinate, with the other corners of the image having non-negative coordinates. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(X)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ] and T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

…

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).