| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0079-v3 |
| --- | --- |

| Title: | AHG12: CTB level filter shape selection of CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Ru-Ling Liao Jie Chen Yan Ye Xinwei Li 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes CTB level filter shape selection of CCALF process. Two filter shapes are proposed. In the proposed method, in each APS, up-to 16 filters and their associated shapes and coefficients are signalled. For each CTB, an index is signalled to the decoder to specify which filter shape and coefficients are used for that CTB. In addition to multiple filter shapes, this contribution also proposes to remove the power of 2 constraint of filter coefficient values.

Following results are reported, as compared to ECM-1.0:

- All Intra: 0.05% (Y), -2.17%(U), -2.70%(V), 101% (EncT), 101%(DecT)

- Random access: x.xx% (Y), - x.xx %(U), - x.xx %(V),  xxx% (EncT), xxx%(DecT)

- Low delay B: -0.01% (Y), -4.14%(U), -3.68%(V), 100% (EncT), 100%(DecT)

# Introduction

The cross component adaptive loop filter (CCALF) of ECM-1.0 is the same as in VVC except that in ECM-1.0 the constraint related to virtual boundary processing is removed. In ECM-1.0 (also in VVC), the CCALF process uses a linear filter to filter luma sample values and generate a residual correction for the chroma samples.  An 8-tap hexagon shaped filter is used in CCALF process, which is illustrated in Figure 1.

Figure 1: CCALF filter shape in ECM-1.0

The CCALF filter of ECM-1.0 may not achieve optimal compression performance. The compression performance of current CCALF design may improve using long tap filters and multiple filter shapes.

# Proposed method

In ECM-1.0, a single filter shape (8-tap hexagon shape filters shown in Figure 1) is used for filtering. However, the correlation between neighboring pixels may depend on the characteristics of the video content. A single filter shape may not be optimal for all types of content. In this contribution, two filter shapes are proposed; one is 17-tap and another is 16-tap shown in Figure 2.

Figure 2: Proposed CCALF filter shapes

In the proposed method, for a given slice, the encoder can collect the statistics of the slice, analyze those and can signal up to 16 filters through APS. For each filter, the shape index (shape_idx in Figure 2) and associated filter coefficients are signalled to the decoder. Similar to existing method, the signaling is performed through APS. For each CTB, an index is signalled to the decoder to specify which filter shape and coefficients are used for that CTB.

In ECM-1.0, CCALF filter coefficients are restricted to having only values in the form of power of 2, from the following set: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. In this contribution, on top of multiple filter shapes, it is also proposed to remove that constraint. In the proposed method, the value of the filter coefficient can be any integer value between -64 to +64, inclusive.

The detail of the signaling of filter shapes, and filter coefficients of the proposed method through APS is shown in Table 1. The differences between existing and proposed signaling are highlighted.

Table 1: APS syntax of the proposed method

| alf_data( ) { | Descriptor |
| --- | --- |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| alf_cc_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cb_filters_signalled_minus1 + 1; k++ ) { |  |
| alf_cc_cb_filters_shape_idx[k] | u(1) |
| noCoeff = alf_cc_cb_filters_shape_idx[k] ? 15: 16 |  |
| for( j = 0; j < 7 noCoeff; j++ ) { |  |
| alf_cc_cb_mapped_coeff_abs[ k ][ j ] | u(3) ue(v) |
| if( alf_cc_cb_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cb_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| alf_cc_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cr_filters_signalled_minus1 + 1; k++ ) { |  |
| alf_cc_cr_filters_shape_idx[k] | u(1) |
| noCoeff = alf_cc_cr_filters_shape_idx[k] ? 15: 16 |  |
| for( j = 0; j < 7 noCoeff; j++ ) { |  |
| alf_cc_cr_mapped_coeff_abs[ k ][ j ] | u(3) ue(v) |
| if( alf_cc_cr_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cr_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

# Simulation results

The proposed method is implemented on top of ECM-1.0. The performance is evaluated based on the test conditions suggested in JVET-V2017[1]. For combined YUV gain, as per methodology described in JVET-S2005, at first weighted PSNR (with weight of 6:1:1) is computed, then, BD-rate is computed from weighted PSNR [2].

Following two tests are conducted. In both tests, the power of 2 constraint of filter coefficient values are removed and up-to 16 filters are signalled through APS.

- Test 1 (Single filter shape): In this test, only cross-shape filter (shown in Figure 1) is used. Since only one shape is used, the shape indices alf_cc_cb_filters_shape_idx[], alf_cc_cr_filters_shape_idx[] are not signalled in the bit-stream.

- Test 2 (Two filter shapes): In this test, both filter shapes (shown in Figure 1) are used.

Table 2: results of test 1 over ECM-1.0

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.07% | -1.13% | -3.33% | 101% | 100% | -0.43% |
| Class A2 | 0.09% | -2.55% | -3.88% | 101% | 101% | -0.51% |
| Class B | 0.09% | -3.31% | -2.52% | 101% | 101% | -0.43% |
| Class C | 0.00% | -0.72% | -0.51% | 101% | 101% | -0.12% |
| Class E | -0.05% | 0.20% | -2.22% | 101% | 101% | -0.26% |
| Overall | 0.04% | -1.66% | -2.38% | 101% | 101% | -0.35% |
| Class D | 0.00% | -0.56% | -0.91% | 101% | 101% | -0.13% |
| Class F | 0.05% | -0.60% | 0.01% | 101% | 101% | -0.02% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.07% | -2.93% | -2.20% | 100% | 100% | -0.50% |
| Class C | 0.03% | -1.21% | -1.18% | 100% | 100% | -0.22% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.01% | -1.01% | -1.43% | 100% | 99% | -0.26% |
| Class F | 0.00% | -0.66% | -0.37% | 100% | 100% | -0.10% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.03% | -5.06% | -3.83% | 100% | 101% | -0.82% |
| Class C | -0.03% | -3.97% | -2.55% | 100% | 99% | -0.61% |
| Class E | -0.11% | -1.79% | -4.56% | 100% | 99% | -0.78% |
| Overall | -0.03% | -3.88% | -3.59% | 100% | 100% | -0.74% |
| Class D | 0.09% | -3.65% | -3.03% | 100% | 100% | -0.47% |
| Class F | 0.20% | -1.46% | -2.08% | 100% | 100% | -0.22% |

Table 3: results of test 2 over ECM-1.0

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.07% | -1.26% | -3.31% | 101% | 100% | -0.45% |
| Class A2 | 0.10% | -3.01% | -4.49% | 101% | 101% | -0.61% |
| Class B | 0.09% | -3.70% | -2.91% | 102% | 101% | -0.49% |
| Class C | 0.00% | -0.93% | -0.75% | 101% | 102% | -0.16% |
| Class E | -0.03% | -1.36% | -2.53% | 101% | 101% | -0.45% |
| Overall | 0.05% | -2.17% | -2.70% | 101% | 101% | -0.42% |
| Class D | 0.01% | -1.06% | -1.14% | 101% | 102% | -0.18% |
| Class F | 0.06% | -1.82% | -1.23% | 101% | 101% | -0.25% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.01% | -0.95% | -1.92% | 100% | 99% | -0.38% |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.06% | -3.20% | -2.43% | 100% | 100% | -0.56% |
| Class C | 0.03% | -1.30% | -1.31% | 100% | 100% | -0.24% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | 0.03% | -1.32% | -1.57% | 101% | 100% | -0.27% |
| Class F | 0.04% | -1.35% | -1.04% | 101% | 100% | -0.22% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over ECM-1.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.05% | -5.20% | -4.13% | 100% | 101% | -0.85% |
| Class C | 0.00% | -4.14% | -2.68% | 100% | 99% | -0.61% |
| Class E | -0.12% | -2.36% | -4.28% | 100% | 100% | -0.83% |
| Overall | -0.01% | -4.14% | -3.68% | 100% | 100% | -0.76% |
| Class D | 0.14% | -3.93% | -2.19% | 100% | 101% | -0.38% |
| Class F | -0.14% | -2.98% | -3.07% | 100% | 100% | -0.70% |

The proposed method is also tested on top of EE2-5.1 described in JVET-W0066. Table 4 shows the simulation results. The anchor is EE2-5.1 software.

Table 4: Simulation results of the proposed method over EE2-5.1

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over EE2-5.1(CCSAO) |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.06% | -1.25% | -3.21% |  |  | -0.43% |
| Class A2 | 0.09% | -2.67% | -4.20% |  |  | -0.54% |
| Class B | 0.08% | -3.44% | -2.55% |  |  | -0.43% |
| Class C | -0.01% | -0.76% | -0.72% |  |  | -0.15% |
| Class E | -0.02% | -1.28% | -2.55% |  |  | -0.43% |
| Overall | 0.04% | -1.99% | -2.53% |  |  | -0.39% |
| Class D | 0.01% | -1.07% | -1.25% |  |  | -0.19% |
| Class F | 0.05% | -1.94% | -1.23% |  |  | -0.24% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over EE2-5.1(CCSAO) |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -0.02% | -0.63% | -1.75% |  |  | -0.33% |
| Class A2 | 0.05% | -2.13% | -3.68% |  |  | -0.61% |
| Class B | 0.06% | -3.06% | -2.18% |  |  | -0.50% |
| Class C | 0.02% | -1.32% | -1.00% |  |  | -0.21% |
| Class E |  |  |  |  |  |  |
| Overall | 0.03% | -1.92% | -2.08% |  |  | -0.41% |
| Class D | 0.05% | -1.37% | -1.54% |  |  | -0.25% |
| Class F | 0.01% | -1.40% | -0.85% |  |  | -0.22% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over EE2-5.1(CCSAO) |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.08% | -4.39% | -3.15% |  |  | -0.62% |
| Class C | 0.08% | -3.09% | -2.07% |  |  | -0.39% |
| Class E | -0.02% | -2.09% | -3.53% |  |  | -0.61% |
| Overall | 0.05% | -3.39% | -2.89% |  |  | -0.54% |
| Class D | 0.06% | -2.97% | -2.35% |  |  | -0.38% |
| Class F | -0.13% | -2.37% | -1.45% |  |  | -0.44% |

# Conclusions

In order to improve the compression performance of the existing CCALF process, a CTB level filter shape selection is proposed. Two filter shapes are proposed in this contribution. In addition to multiple filter shapes, it is also proposed to remove the power of 2 constraint of the filter coefficients values. The proposed methods can achieve following coding gain as compared to ECM-1.0.

- All Intra: 0.05% (Y), -2.17%(U), -2.70%(V), -0.42% (YUV), 101% (EncT), 101%(DecT)

- Random access: x.xx% (Y), - x.xx %(U), - x.xx %(V), - x.xx %(YUV),   xxx% (EncT), xxx%(DecT)

- Low delay B: -0.01% (Y), -4.14%(U), -3.68%(V), -0.76% (YUV), 100% (EncT), 100%(DecT)

# References

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, April 2021.

W.-J. Chien, J. Boyce, “Methodology and reporting template for coding tool testing”, JVET-S2005, June 2020.

C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “EE2-5.1: Cross-component Sample Adaptive Offset” JVET-W0066, July 2021.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).