| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0082 |
| --- | --- |

| Title: | AHG10: GOP-based RPR encoder control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

VTM currently lacks encoder control for selecting when to use Reference Picture Resampling (RPR). This contribution proposes a GOP-based selection method that decides when to encode pictures in reduced resolution with RPR or if its better to encode them in the source resolution. The selection is based on QP and picture self-similarity after re-scaling. The self-similarity test is only conducted on the first source picture in display order within each GOP. If the re-scaled picture is determined to have sufficient similarity with the source picture, all pictures in the GOP are encoded at reduced resolution, otherwise they are encoded in the source resolution.

The BD-rate performance (Y/Cb/Cr) compared to CTC with VTM-13.0 as anchor is as follows:

RA (luma/Cb/Cr):   -0.03%/0.61%/0.58%

LDB (luma/Cb/Cr): 0.27%/0.35%/0.35%

AI (luma/Cb/Cr): 0.11%	/0.63%/0.67%

It is asserted that the method, when it is selected, in some cases can improve subjective quality compared to encoding in source resolution for low bitrates.

# Introduction

When encoding at restricted bitrates, it can sometimes be useful to encode at a lower resolution rather than at the source resolution. Since VVC provides a technique for enabling flexible switching of resolution without requiring encoding of an intra picture at the switching point, it is possible to do this more efficiently than in previous standards. However, the VVC reference encoder lacks a method to decide when to enable encoding at reduced resolution. This contribution proposes to add such functionality.

# Proposal

The first picture in display order in each GOP is downscaled according to the selected scaling ratio. The downscaled picture is then upscaled to the source resolution and compared with the source picture by means of PSNR, and this PSNR score is then compared with a configurable threshold parameter.  If the re-scaled picture is sufficiently similar to the source picture, all pictures in the GOP are encoded at reduced resolution. Otherwise all pictures are encoded at the source resolution.

For a GOP hierarchy of 32 pictures, the first picture in display order is checked, and if it is sufficiently similar to the corresponding source picture, that picture and the next 31 pictures in display order are encoded at reduced resolution. Otherwise the first 32 pictures in display order are encoded at the source resolution. Then based on an evaluation of picture 32, the encoding resolution for that picture and the following 31 pictures is decided.

Configurable parameters are added to enable control of how careful the selection of encoding in reduced resolution should be and above which QP the method becomes activated for:

GopBasedRPREnable : 1 #A value of 1 enables the method (default is 0=off)

GopBasedRPRThresholdQP : 37, #GOP-based RPR check is made for QP >= GopBasedRPRThresholdQP

GopBasedRPRQPoffset: -6, #This offset is added to the QP when encoding at reduced resolution (-6 gives typically a similar bitrate when encoding at quarter resolution) .

GopBasedRPRSimilarityThresholdLuma : 38.0, #Encoding in reduced resolution if PSNR for luma after re-scaling is higher than GopBasedRPRSimilarityThresholdLuma, see also GopBasedRPRSimilarityThresholdChroma.

GopBasedRPRSimilarityThresholdChroma : 0.0, Encoding in reduced resolution if PSNR for chroma components after re-scaling are both higher than GopBasedRPRSimilarityThresholdChroma, see also GopBasedRPRSimilarityThresholdLuma.

# Results

The method is implemented on top of VTM-13.0 [1] and objective results are provided for VTM CTC [2] and NNVC CTC [3] for two configurations evaluated in the source resolution based on the VTM upscaling approach (--UpscaledOutput=2). The results are provided for the case of ScalingRatioHor=2.0 and ScalingRatioVer=2.0, meaning that when a GOP is selected to be encoded in reduced resolution the resolution is half of the source resolution vertically and horizontally. Encoding and decoding time are not reliable.

Config 1: Selection to encode in reduced resolution only based on QP:

GopBasedRPREnable: 1

GopBasedRPRThresholdQP: 37

GopBasedRPRQPoffset: -6

GopBasedRPRSimilarityThresholdLuma: 0.0 (RA), 0.0 (LDB)

GopBasedRPRSimilarityThresholdChroma: 0.0 (RA), 0.0 (LDB)

Config 2: Selection to encode in reduced resolution only based on QP and similarity with source after re-scaling:

GopBasedRPREnable: 1

GopBasedRPRThresholdQP: 37

GopBasedRPRQPoffset: -6

GopBasedRPRSimilarityThresholdLuma: 38.0 (RA), 36.0 (LDB)

GopBasedRPRSimilarityThresholdChroma: 0.0 (RA), 0.0 (LDB)

## CTC (QP 22, 27, 32, 37)

### Config 1: Selection to encode in reduced resolution only based on QP

Random access

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,37% | 0,73% | 0,70% | 89% | 89% |
| Class A2 | 0,49% | 1,36% | 1,35% | 86% | 89% |
| Class B | 2,66% | 0,77% | 0,65% | 86% | 89% |
| Class C | 2,33% | 1,24% | 1,26% | 79% | 84% |
| Class E |  |  |  |  |  |
| Overall | 1,33% | 1,01% | 0,96% | 85% | 88% |
| Class D | 1,89% | -0,25% | 0,39% | 78% | 85% |
| Class F | 2,41% | 1,22% | 1,42% | 86% | 90% |

Low delay B

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,60% | 0,78% | 0,46% | 86% | 86% |
| Class A2 | -0,30% | 1,93% | 1,71% | 83% | 88% |
| Class B | 2,00% | 1,68% | 1,64% | 85% | 91% |
| Class C | 2,00% | 1,74% | 1,78% | 84% | 89% |
| Class E | 3,07% | 0,64% | 0,98% | 85% | 97% |
| Overall | 2,27% | 1,44% | 1,52% | 85% | 92% |
| Class D | 1,83% | 1,19% | 1,62% | 81% | 90% |
| Class F | 2,11% | 1,02% | 1,40% | 86% | 93% |

All intra

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,27% | 1,36% | 1,11% | 106% | 91% |
| Class A2 | -0,20% | 1,51% | 1,54% | 96% | 93% |
| Class B | 1,28% | 0,77% | 0,99% | 92% | 89% |
| Class C | 1,59% | 1,04% | 1,08% | 85% | 88% |
| Class E | 2,00% | 0,11% | 0,43% | 92% | 90% |
| Overall | 0,96% | 0,94% | 1,03% | 93% | 90% |
| Class D | 1,39% | 0,46% | 0,82% | 84% | 90% |
| Class F | 1,72% | 0,36% | 0,60% | 88% | 89% |

### Config 2: Selection to encode in reduced resolution based on QP and similarity with source after re-scaling

Random access

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,37% | 0,73% | 0,70% | 98% | 94% |
| Class A2 | 0,49% | 1,36% | 1,35% | 95% | 87% |
| Class B | 0,45% | 0,58% | 0,51% | 108% | 101% |
| Class C | 0,00% | 0,00% | 0,00% | 87% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,61% | 0,58% | 97% | 95% |
| Class D | 0,00% | 0,00% | 0,00% | 95% | 95% |
| Class F | 0,00% | 0,00% | 0,00% | 103% | 90% |

Low delay B

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,60% | 0,78% | 0,46% | 89% | 81% |
| Class A2 | -0,30% | 1,93% | 1,71% | 86% | 87% |
| Class B | 0,12% | 0,73% | 0,71% | 102% | 90% |
| Class C | 0,09% | 0,16% | 0,14% | 112% | 111% |
| Class E | 0,78% | -0,03% | 0,04% | 106% | 98% |
| Overall | 0,27% | 0,35% | 0,35% | 107% | 98% |
| Class D | 0,02% | 0,02% | 0,03% | 113% | 115% |
| Class F | 0,00% | 0,00% | 0,00% | 112% | 97% |

All intra

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,27% | 1,36% | 1,11% | 112% | 95% |
| Class A2 | -0,20% | 1,51% | 1,54% | 103% | 96% |
| Class B | 0,18% | 0,45% | 0,63% | 112% | 110% |
| Class C | 0,15% | 0,15% | 0,16% | 112% | 113% |
| Class E | 0,62% | -0,05% | 0,12% | 110% | 105% |
| Overall | 0,11% | 0,63% | 0,67% | 110% | 105% |
| Class D | 0,04% | 0,04% | 0,04% | 117% | 137% |
| Class F | 0,00% | 0,00% | 0,00% | 113% | 119% |

## CTC (QP 27, 32, 37, 42)

### Config 1: Selection to encode in reduced resolution only based on QP

Random access

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -5,91% | 6,62% | 4,60% | 82% | 80% |
| Class A2 | 2,38% | 12,39% | 12,35% | 76% | 80% |
| Class B | 18,68% | 8,48% | 7,74% | 77% | 79% |
| Class C | 21,59% | 11,19% | 11,32% | 64% | 72% |
| Class E |  |  |  |  |  |
| Overall | 11,28% | 9,61% | 8,99% | 74% | 78% |
| Class D | 17,56% | 6,79% | 8,49% | 61% | 70% |
| Class F | 20,46% | 14,37% | 16,07% | 76% | 79% |

Low delay B

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -7,34% | 7,93% | 4,51% | 75% | 75% |
| Class A2 | -1,84% | 16,26% | 14,70% | 70% | 79% |
| Class B | 16,23% | 13,02% | 12,33% | 72% | 81% |
| Class C | 18,66% | 14,00% | 14,74% | 71% | 81% |
| Class E | 23,67% | 5,69% | 6,10% | 71% | 91% |
| Overall | 18,90% | 13,46% | 13,40% | 72% | 83% |
| Class D | 16,87% | 11,00% | 13,17% | 68% | 84% |
| Class F | 19,44% | 14,46% | 16,42% | 75% | 86% |

All intra

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3,12% | 10,92% | 7,63% | 116% | 81% |
| Class A2 | -1,18% | 10,50% | 10,30% | 100% | 85% |
| Class B | 12,55% | 7,16% | 7,89% | 100% | 100% |
| Class C | 16,44% | 8,35% | 8,20% | 78% | 80% |
| Class E | 17,18% | 2,32% | 1,85% | 90% | 81% |
| Overall | 9,29% | 7,80% | 7,31% | 93% | 80% |
| Class D | 13,54% | 5,45% | 6,10% | 74% | 82% |
| Class F | 16,19% | 10,83% | 12,02% | 81% | 77% |

### Config 2: Selection to encode in reduced resolution based on QP and similarity with source after re-scaling

Random access

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -5,91% | 6,62% | 4,61% | 87% | 81% |
| Class A2 | 2,38% | 12,39% | 12,35% | 81% | 81% |
| Class B | 1,73% | 3,71% | 3,14% | 103% | 93% |
| Class C | 0,00% | 0,00% | 0,00% | 85% | 88% |
| Class E |  |  |  |  |  |
| Overall | -0,13% | 5,04% | 4,44% | 90% | 87% |
| Class D | 0,00% | 0,00% | 0,00% | 93% | 88% |
| Class F | 0,00% | 0,00% | 0,00% | 102% | 87% |

Low delay B

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -7,34% | 7,93% | 4,51% | 78% | 73% |
| Class A2 | -1,84% | 16,26% | 14,70% | 73% | 79% |
| Class B | 0,30% | 5,19% | 4,67% | 99% | 85% |
| Class C | 0,39% | 0,88% | 0,74% | 112% | 107% |
| Class E | 6,39% | 0,88% | 0,47% | 98% | 94% |
| Overall | 1,85% | 3,27% | 2,92% | 102% | 94% |
| Class D | 0,08% | 0,24% | 0,10% | 115% | 112% |
| Class F | 0,00% | 0,00% | 0,00% | 113% | 96% |

All intra

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3,12% | 10,92% | 7,63% | 118% | 83% |
| Class A2 | -1,18% | 10,50% | 10,30% | 104% | 83% |
| Class B | 0,69% | 3,20% | 3,79% | 108% | 113% |
| Class C | 0,68% | 0,94% | 0,92% | 114% | 112% |
| Class E | 5,27% | 0,43% | 0,28% | 106% | 98% |
| Overall | 0,50% | 4,74% | 4,29% | 111% | 97% |
| Class D | 0,15% | 0,16% | 0,18% | 116% | 130% |
| Class F | 0,00% | 0,00% | 0,00% | 114% | 115% |

# Conclusions

It has been shown that RPR can provide some additional benefit for VTM for 4K resolution compared to CTC by the proposed GOP-based RPR selection method. It is therefore suggested to include the GOP-based RPR selection method in the VTM software.

# References

[1] VTM-13.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-13.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[3] S. Liu, A. Segall, E. Alshina, R-L. Liao,	“Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-V2016, Meeting by Teleconference, April 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).