| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 07–16 July 2021 | Document: JVET-W0083-v1 |
| --- | --- |

| Title: | AHG9: Bug fixes for some SEI messages in the VSEI amendment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yang Wang2, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com wangyang.cs@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes some changes to the latest draft of the VSEI amendment in JVET-V2006, for fixing some asserted bugs. Involved SEI messages are the annotated regions SEI message, the depth representation information (DRI) SEI message, and the extended DRAP indication SEI message.

It is asserted that some of issues for the annotated regions SEI message and the DRI SEI message also apply for HEVC and/or AVC.

The proposed changes are summarized in the contribution and the exact text changes for VSEI are provided in an attachment to this contribution. Text changes for HEVC and AVC are not provided, but once decided those are asserted to be the same as the corresponding VSEI changes.

# Issues

The following issues for the annotated regions SEI message, the depth representation information SEI message, and the EDRAP indication SEI message have been observed:

- The syntax table for the annotated regions SEI message has the column "C", which is for AVC syntax table but not for VSEI syntax table, thus should be removed.

- For the annotated regions SEI message, the value range of the ue(v)-coded syntax element ar_object_label_idx[ ar_object_idx[ i ] ] is missing.

- For the DRI SEI message, the descriptor (i.e., coding method) of the depth_nonlinear_representation_model[ i ] syntax element is unspecified.

- For the DRI SEI message, the value ranges for the ue(v)-coded syntax elements depth_representation_type, disparity_ref_view_id, depth_nonlinear_representation_num_minus1, and depth_nonlinear_representation_model[ i ] are missing.

- In the semantics of the u(5)-coded syntax element da_mantissa_len_minus1, the specification of the value range of 0 to 31, inclusive, is redundant.

- For the EDRAP indication SEI message, the semantics of the edrap_leading_pictures_decodable_flag syntax element is missing, and in the syntax table there is a typo of "minsu1" that should be "minus1".

# Proposed changes

The proposed changes, one-to-one corresponding to the issues listed above, are summarized below and the exact text changes for VSEI are provided in an attachment to this contribution:

- Remove the column "C" from the syntax table for the annotated regions SEI message.

- Specify that the value of ar_object_label_idx[ ar_object_idx[ i ] ] shall be in the range of 0 to 255, inclusive.

Justification: This value depends on the maximum number of labels for one object, and the value range of ar_label_idx[ i ] is 0 to 255, inclusive.

Do the same for AVC and HEVC.

- Specify the depth_nonlinear_representation_model[ i ] syntax element to be ue(v)-coded. Justification: In the HM software, the coding method is ue(v), and in AVC, ue(v) is present in the syntax table for this syntax element (although there are differences in the syntax between HEVC and AVC for this SEI message).

Do the same for HEVC.

  - Furthermore, specify the value range of this syntax element to be 0 to 2^16 − 1, inclusive. This is based on: 1) the positions of additional nodes are transmitted in the form of the deviations of depth_nonlinear_representation_model[ i ] from the straight-line curve; 2) these deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with the spacing depending on the value of nonlinear_depth_representation_num_minus1; and 3) the maximum bit depth value allowed by the syntax is 16.

Do the same for AVC and HEVC.

- Specify the value ranges for the ue(v)-coded syntax elements depth_representation_type, disparity_ref_view_id, depth_nonlinear_representation_num_minus1, and depth_nonlinear_representation_model[ i ] as follows:

  - The value of depth_representation_type shall be in the range of 0 to 15, inclusive.

Justification: Currently only values 0 to 3, inclusive, are used.

Do the same for AVC and HEVC.

  - The value of disparity_ref_view_id shall be in the range of 0 to 1023, inclusive.

Justification: The maximum view ID in the AVC extensions is 1023, and in VSEI and HEVC extensions view ID is u(v)-coded with the maximum length being 16 bits.

Do the same for HEVC (this syntax element does not exist in AVC).

  - The value of depth_nonlinear_representation_num_minus1 shall be in the range of 0 to 62, inclusive.

Justification: This is the number of piece-wise linear segments for a depth mapping table.

Do the same for AVC and HEVC.

- Remove the specification of the value range of 0 to 31, inclusive, for the u(5)-coded syntax element da_mantissa_len_minus1.

Do the same for AVC and HEVC.

- Add the missing semantics for the syntax element edrap_leading_pictures_decodable_flag as follows and remove the same constraint that was specified at the beginning of the subclause before specific syntax element semantics (this is basically moving the same constraint that was specified at the beginning of the subclause before specific syntax element semantics):

edrap_leading_pictures_decodable_flag equal to 1 specifies that both of the following constraints apply:

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order shall follow, in output order, any picture that is in the same layer and precedes the EDRAP picture in decoding order.

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order and precedes the EDRAP picture in output order shall not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

edrap_leading_pictures_decodable_flag equal to 0 does not impose such constraints.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).