| x`Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0085-v2 |
| --- | --- |

| Title: | AHG9: Picture quality metrics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, D. Rusanovskyy, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JVET-V0062 and proposes the inclusion of a new SEI message, picture quality metrics SEI message, in the VSEI specification. Three quality metrics, PSNR, wPSNR and WS-PSNR, are specified in the new SEI message for SDR, HDR and 360 video content, respectively.

# Introduction

Picture quality metrics measure a vareity of picture distortions during the acquisition, processing, compression, storage, transmission and reproduction. The metrics are broadly used in codec evaluation, specification and acceptance testing, and quality of service measurement.

In general, the full reference quality metrics can provide the best fidelity measurement accuracy for the quality oriented applications, while most of these applications such as are based on the parameters such as resolutions, bitrate, QP or quality ranking since the system or the clients cannot access the full reference. In addition, some post-processing technologies utilize the quality metrics to enhance the reconstructed picture quality, such as cross-segment decoding for quality recovery implemented in the Green MPEG. Furthermore, the user generated content (UGC) from mobile devices or drones is usually pre-compressed and the capture quality is unkown, it would be beneficial to convey the the UGC quality metrics to the clients and services.

Various quality metrics have been explored and documented for usage in the MPEG projects and describied in standardization specifications:

Technical report ISO/IEC TR 23002-8, “Working practices using objective metrics for evaluation of video coding efficiency experiments”, describes usage of the metric MSE and PSNR for individual frames, PSNR_YUV for a sequence, PSNRL100, wPSNR, DE100 for HDR content, and WS-PSNR for 360 video [1].

ISO/IEC 23001-10, “Carriage of timed metadata metrics of media in ISOBMFF”, specifies usage of the quality metrics of media such as PSNR, structure similarity index (SSIM), multi-scale structural similarity index (MS-SSIM), video quality metric (VQM), perceptual evaluation of video quality (PEVQ) and mean opinion score (MOS) in the file format [2].

ISO/IEC 23001-11, Green MPEG, specifies usage of PSNR, as defined in ISO/IEC 23001-10, for the quality recovery after low-power encoding [3]. The quality metrics metadata of AVC/HEVC green metadata SEI message indicates the type of the objective quality metric, PSNR, as shown in Table 1.

Table 1 Green metadata SEI message quality metadata

|  | Size (bits) | Descriptor |
| --- | --- | --- |
| xsd_metric_type | 8 | unsigned integer |
| xsd_metric_value | 16 | unsigned integer |

SC29 AG5 is also exploring quality metrics and methodologies for immersive visual media, including 360 video, MIV, V-PCC and G-PCC.

Since the full reference quality metrics is only available at the encoder side, a SEI message carrying such information is able to provide the relevant information to the system and the applications.

This contribution proposes inclusion of the Picture Quality Metrics SEI message in the VSEI specification, and three quality metrics, PSNR, wPSNR and WS-PSNR, are specified for the SDR, HDR and 360 video content, respectively. Both sequence level and frame-based quality metric types are specified in the SEI message, the sequence level quality metrics may apply to the segment based streaming services and the frame based quality metrics may apply to the post-processing technologies.

# Proposal

The proposed picture metrics SEI message is provided below:

Table 2 Picture quality metric SEI message

| Picture_quality_metrics( payloadSize ) { | Descriptor |
| --- | --- |
| pqm_cnt_minus1 | u(8) |
| for( i = 0; i <= pqm_cnt_minus1; i++ ) { |  |
| pqm_type[ i ] | u(8) |
| pqm_value[ i ] | u(16) |
| } |  |
| } |  |

The Picture Quality Metric SEI message carries information on the quality of the associated pictures or video sequence, evaluated for the specified objective quality metric.

pqm_cnt_minus1 plus 1 specifies the number of quality metrics that are indicated by the SEI message.

pqm_type[ i ] indicates the i-th quality metric type associated with the decoded picture or coded layer video sequence (CLVS) as specified in Table 3. The value of pqm_type from 6 to 255 are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification.

Table 3 Interpretation of pqm_type

| pqm_type | metric |
| --- | --- |
| 0 | PNSR_Y |
| 1 | PSNR_Ysequence |
| 2 | wPSNR_Y |
| 3 | wPSNR_Ysequence |
| 4 | WS-PSNR_Y |
| 5 | WS-PSNR_Ysequence |

pqm_value[ i ] specifies the value of the i-th quality metric, and only luma component is used for the computation.

When pqm_type is 0, the quality metric indicates the luma PSNR of the associated picture. The 16-bit unsigned integer pqm_value is interpreted as a result of PSNR_Y quality metric (in dB), as defined in ISO/IEC 23002-8, and derived as follows (expressed in floating point) with the exception of PSNR_Y equal to infinity for pqm_value value is equal to 0.

PSNR_Y=pqm_value[ 0i ]100

When pqm_type is 1, the quality metric indicates the average luma PSNR of the CLVS that the associated picture belongs to. The 16-bit unsigned integer pqm_value is interpreted as a result of CLVS level luma PSNR_Y quality metric (in dB), as defined in ISO/IEC 23002-8, and derived as follows (expressed in floating point) with the exception of PSNR_Y equal to infinity for pqm_value value is equal to 0.

PSNR_Ysequence=pqm_value[ 1i ]100

When pqm_type is 2, the quality metric indicates the luma weighted PSNR of the associated picture. The 16-bit unsigned integer pqm_value is interpreted as wPSNR_Y value (in dB), as defined in ISO/IEC 23002-8, and derived as follows (expressed in floating point) with the exception of wPSNR_Y equal to infinity for pqm_value value is equal to 0.

wPSNR_Y=pqm_value[ 2i ]100

When pqm_type is 3, the quality metric indicates the average luma weighted luma PSNR of the CLVS that the associated picture belongs to. The 16-bit unsigned integer pqm_value is interpreted as CLVS level wPSNR_Y value (in dB) as defined in ISO/IEC 23002-8, , and derived as follows (expressed in floating point) with the exception of wPSNR_Y equal to infinity for pqm_value value is equal to 0.

wPSNR_Ysequence=pqm_value[ 3i ]100

When pqm_type is 4, the quality metric indicates the luma WS-PSNR of the associated picture. The 16-bit unsigned integer pqm_value is interpreted as WS-PSNR_Y value  (in dB), as defined in ISO/IEC 23002-8,  and derived as follows (expressed in floating point) with the exception of WS-PSNR_Y equal to infinity for pqm_value value is equal to 0.

WS_PSNR_Y=pqm_value[4i ]100

When pqm_type is 5, the quality metric indicates the average luma WS-PSNR of the CLVS that the associated picture belongs to. The 16-bit unsigned integer pqm_value is interpreted as CLVS level WS-PSNR_Y value  (in dB) as follows (expressed in floating point) with the exception of WS-PSNR_Y equal to infinity for pqm_value value is equal to 0.

WS_PSNR_Ysequence=pqm_value[5i ]100

The CLVS level picture metrics of all picture quality SEI messages that apply to the same CLVS shall have the same content.

The picture metric with the same picture metric type in a picture metric SEI message shall have the same content.

# Quality metrics definition

## Peak Signal to Noise Ratio (PSNR)

An informative description of PSNR can be found in [1]. For an individual frame, the mean square error is calculated between the luma channel decY of the decoded output image and the luma channel origY of the original image according to

MSE_Y=1W*Hy=0H-1x=0W-1decYx,y-origYx,y2

where decY(x,y) and origY(x,y) are the luma sample values at position x,y of the decoded and original images at the same time instance, respectively. W and H are the width and height of the luma component, respectively. A luma PSNR value for the frame may then be calculated as

PSNR_Y= 10*log10255≪bitDepth-82MSE_Y

where bitDepth = 10 for 10-bit inputs and where ≪ denotes a bitwise left-shift operation.

The aggregate PSNR for a test sequence is calculated as the average of the PSNR values for the individual frames:

PSNR_Ysequence=1NumFramesk=0NumFrames-1PSNR_Yk

where PSNR_Yk is the PSNR_Y value for frame k and NumFrames is the number of frames in the sequence.

## Weighted PSNR (wPSNR)

The weighted PSNR metric is calculated from the weighted mean squared error of the pixel values. An informative description of the metric can be found in [4].

The wPSNR metric is calculated as:

wPSNR=10*logX2wMSE,

where X is the maximum pixel value for the specific bit depth and wMSE is given as

wMSE= all pixels i in blockwilumaxorig,i*(xorig, i-xdec,i)2,

where wilumaxorig,iis a weight that is a function of the luma value corresponding to the pixel xorig,i, xorig,i is the original value at location i, and xdec,i is the reconstructed value at location i.

The calculation of the weight is computed as:

XL = bitDepth >= 10 ? luma(xorig,i) >> (bitDepth - 10) : luma(xorig,i) << (10 - bitDepth);

yi = 0.015*xL − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

where bitDepth is the bit depth of xorig.

wPSNR for a given video sequence is computed as an average of all picture-level wPSNR values obtained for all pictures in the sequence, i.e., for a sequence with N pictures we have

wPSNRsequence=1Nn=0N-1wPSNRpicture(n)

Only luma component of the video signal is used for wPSNR_Y computation.

## Weighted to Spherically uniform PSNR (WS-PSNR)

WS-PSNR calculates PSNR using all image samples on the 2D projection plane. The distortion at each position i,j is weighted by the spherical area covered by that sample position. An informative description of the metric can be found in [5].

The weighted mean squared error (WMSE) is first calculated as:

| WMSE=1i=0M-1j=0N-1w(i,j)i=0M-1j=0N-1(yi,j-y'(i,j))2*w(i,j) |  |
| --- | --- |

The WS-PSNR is then calculated as:

| WS_PSNR=10logMAXI2WMSE |  |
| --- | --- |

where MAXI is the maximum intensity level of the images.

Since WS-PSNR is calculated based on the projection plane, different weights are derived for different projection formats and the further detail can be found in [5].

WS-PSNR for a given video sequence is computed as an average of all picture-level WS-PSNR values obtained for all pictures in the sequence, i.e., for a sequence with N pictures we have

WS_PSNRsequence=1Nn=0N-1WS_PSNRpicture(n)

Only luma component of the video signal is used for WS-PSNR_Y computation.

# Reference

- ISO/IEC JTC 1/SC 29 TR 23002-8, " Working practices using objective metrics for evaluation of video coding efficiency experiments," 2020

- ISO/IEC JTC 1/SC 29 FDIS 23001-10, "Carriage of Timed Metadata Metrics of Media in ISO Base Media File Format," 2015

- ISO/IEC JTC 1/SC 29 23001-11, " Energy-Efficient Media Consumption (Green Metadata)," 2018

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-V2011, 22nd JVET meeting Apr. 2021.

- Y. Ye and J. Boyce, " Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 12." Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-T2004, 20nd JVET meeting Oct. 2020

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).