| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0086-v1 |
| --- | --- |

| Title: | EE2-1.3/EE2-1.4: Unsymmetric partitioning methods in video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents experimental results for EE2-1.3/EE2-1.4, exploring unsymmetric partitioning methods denoted as unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) partitioning, which can be applied for blocks with dimensions no greater than 64×64. On top of ECM-1.0, four sub-tests with different configurations are conducted as listed below:

Test 1.3a: UQT with high efficiency.

Test 1.3b: UQT with low complexity.

Test 1.4a: UQT+UBT with high efficiency.

Test 1.4b: UQT+UBT with low complexity.

Simulation results (BD-rate changes on the luma component, encoding time and decoding time) of all sub-tests are reported as below:

Tool-on tests (ECM-1.0 turning off all new features as the anchor):

Test 1.3a:  AI: -0.23%, 171%, 102%; RA: -0.69%, 173%, 101%; LB: -0.69%, 168%, 101%.

Test 1.3b:  AI: -0.09%, 120%, 101%; RA: -0.40%, 119%, 101%; LB: -0.29%, 110%, 101%.

Test 1.4a:  AI: -0.69%, 193%, 104%; RA: -1.45%, 182%, 102%; LB: -1.30%, 164%, 103%.

Test 1.4b:  AI: -0.63%, 179%, 104%; RA: -1.19%, 161%, 102%; LB: -1.05%, 145%, 102%.

Tool-off tests (ECM-1.0 as the anchor):

Test 1.3a:  AI: -0.20%, 171%, 100%; RA: -0.52%, 170%, 100%; LB: -0.50%, 156%, 98%.

Test 1.3b:  AI: -0.05%, 119%, 100%; RA: -0.21%, 116%, 99%; LB: -0.12%, 106%, 99%.

Test 1.4a:  AI: -0.41%, 173%, 99%; RA: -1.02%, 173%, 101%; LB: -0.89%, 155%, 101%.

Test 1.4b:  AI: -0.39%, 161%, 99%; RA: -0.76%, 154%, 101%; LB: -0.61%, 135%, 100%.

# Introduction

In JVET-V0097[1], unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) partitioning were proposed. EE tests on UBT and UQT are reported in this proposal.

# Proposed methods

## UBT

Four types of UBT are proposed as shown in Fig. 1. UBT-H1 and UBT-H2 split a CU with dimension W×H into two child-CUs with dimensions {W×H/4, W×3H/4} and {W×3H/4, W×H/4}, respectively. UBT-V1 and UBT-V2 split a CU with dimension W×H into two child-CUs with dimensions {W/4×H, 3W/4×H} and {3W/4×H, W/4×H}, respectively.

With UBT, the width or height of a CU or TU may be a non-dyadic number (not in a form of 2N). Some coding tools like ISP, MIP and SBT are disabled in those CUs. Besides, 6-point, 12-point, 24-point, and 48-point transforms/inverse-transforms are introduced to code residual blocks with non-dyadic dimensions.

## UQT

Four types of UQT are proposed as shown in Fig. 2. UQT-H1 and UQT-H2 split a CU with dimension W×H into four child-CUs with dimensions {W×H/8, W×H/2, W×H/4, W×H/8} and {W×H/8, W×H/4, W×H/2, W×H/8}, respectively. UQT-V1 and UQT-V2 split a CU with dimension W×H into four child-CUs with dimensions {W/8×H, W/2×H, W/4×H, W/8×H} and {W/8×H, W/4×H, W/2×H, W/8×H}, respectively.

| UBT-H1 | UBT-H2 |
| --- | --- |
| UBT-V1 | UBT-V2 |

Fig. 1 UBTs.

| UQT-H1 | UQT-H2 |
| --- | --- |
| UQT-V1 | UQT-V2 |

Fig. 2 UQTs.

## Signaling of partitioning methods

Whether to apply vertical or horizontal UBT/UQT partitioning is indicated by mtt_split_cu_vertical_flag in VVC.

An mtt_split_cu_ubt_flag is signaled before mtt_split_cu_binary_flag to indicate whether UBT is applied. If UBT is signaled to be applied, a second flag is further signaled to indicate whether type-1 or type-2 UBT is applied.

To avoid redundant represenation, UQT is signaled in a two-step manner as demonstrated in Fig. 3. First, an mtt_split_cu_uqt_flag is signaled when mtt_split_cu_binary_flag is equal to 0. If mtt_split_cu_uqt_flag is equal to 0, regular TT is applied. Otherwise, if mtt_split_cu_uqt_flag is equal to 1, the CU is firstly split into three parts as {1/8, 3/4, 1/8}, and then the central part is further split into to two parts as {2/3, 1/3} or {1/3, 2/3}, indicated by a third flag. Cominbing the two steps, a CU is split into four parts as {1/8, 1/2, 1/4, 1/8} or {1/8, 1/4, 1/2, 1/8}.

Fig. 3 A two-step method to signal UQTs.

## Normative constraints on partitioning

Some normative constrains are imposed on partitioning to reduce redundancy and complexity.

- QT is not allowed for a CU if any of its ancestor CUs is split from UBT or UQT.

- UBT and UQT can only be applied to CUs no greater than 64×64.

- The width and height on luma component must be in a form of 4N after any partitioning.

- UBT and UQT are not used on chroma components for dual-tree coding structure.

- Local dual-tree is disabled when UBT and UQT are used.

- For a CU with dimension W×H:

  - If W is a non-dyadic number, no vertical partitioning is allowed on the CU.

  - If H is a non-dyadic number, no horizontal partitioning is allowed on the CU.

# Experimental results

The proposed unsymmetric partitioning methods are tested on top of ECM-1.0 [2], according to the common test conditions defined in [3]. Per the requests of the description, four sub-tests with different configurations are conducted as listed below:

Test 1.3a: UQT with high efficiency.

Test 1.3b: UQT with low complexity.

Test 1.4a: UQT+UBT with high efficiency.

Test 1.4b: UQT+UBT with low complexity.

The results of tool-on test (based on ECM-1.0 turning off all new features as the anchor) and tool-off test (based on ECM-1.0 with all new features on as the anchor) for Test 1.3a ~ Test 1.4b are tabulated in Table 1 and Table 8, respectively.

Table 1: Performance of the tool-on test in Test 1.3a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.35% | -0.02% | 153% | 101% | -0.50% | -0.41% | -0.56% | 167% | 101% |  |  |  |  |  |
| Class A2 | -0.20% | -0.45% | -0.53% | 162% | 102% | -0.64% | -0.95% | -0.84% | 172% | 101% |  |  |  |  |  |
| Class B | -0.22% | -0.32% | -0.44% | 172% | 101% | -0.64% | -0.73% | -0.64% | 173% | 101% | -0.57% | -0.57% | -0.43% | 167% | 100% |
| Class C | -0.33% | -0.66% | -0.78% | 186% | 105% | -0.94% | -1.03% | -1.08% | 180% | 100% | -0.92% | -1.16% | -0.89% | 192% | 101% |
| Class E | -0.28% | -0.31% | -0.09% | 175% | 101% |  |  |  |  |  | -0.57% | -0.13% | -1.22% | 142% | 101% |
| Overall | -0.23% | -0.42% | -0.40% | 171% | 102% | -0.69% | -0.79% | -0.78% | 173% | 101% | -0.69% | -0.66% | -0.78% | 168% | 101% |
| Class D | -0.19% | -0.93% | -0.81% | 179% | 107% | -0.88% | -1.31% | -1.19% | 167% | 102% | -0.93% | -1.87% | -1.64% | 184% | 102% |
| Class F | -0.19% | -0.34% | -0.33% | 145% | 98% | -0.75% | -1.08% | -1.09% | 161% | 100% | -1.18% | -0.55% | -0.16% | 162% | 98% |

Table 2: Performance of the tool-on test in Test 1.3b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.32% | 0.04% | 117% | 101% | -0.25% | -0.26% | -0.21% | 119% | 100% |  |  |  |  |  |
| Class A2 | -0.07% | -0.53% | -0.44% | 120% | 103% | -0.32% | -0.53% | -0.55% | 120% | 100% |  |  |  |  |  |
| Class B | -0.07% | -0.23% | -0.40% | 120% | 100% | -0.37% | -0.35% | -0.42% | 119% | 101% | -0.32% | -0.39% | -0.17% | 119% | 100% |
| Class C | -0.16% | -0.63% | -0.62% | 121% | 102% | -0.62% | -0.64% | -0.68% | 119% | 101% | -0.51% | -0.52% | -0.50% | 105% | 101% |
| Class E | -0.12% | -0.09% | -0.15% | 120% | 101% |  |  |  |  |  | 0.03% | -0.36% | -0.48% | 104% | 101% |
| Overall | -0.09% | -0.36% | -0.34% | 120% | 101% | -0.40% | -0.44% | -0.48% | 119% | 101% | -0.29% | -0.42% | -0.36% | 110% | 101% |
| Class D | -0.08% | -0.91% | -0.92% | 119% | 109% | -0.56% | -1.23% | -1.08% | 115% | 103% | -0.32% | -1.42% | -1.54% | 107% | 104% |
| Class F | 0.02% | -0.37% | -0.38% | 109% | 100% | -0.33% | -0.72% | -0.81% | 115% | 100% | -0.06% | 0.51% | 0.76% | 104% | 98% |

Table 3: Performance of the tool-on test in Test 1.4a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.60% | -0.33% | 185% | 103% | -1.29% | -1.52% | -1.74% | 174% | 102% |  |  |  |  |  |
| Class A2 | -0.69% | -0.67% | -0.77% | 194% | 105% | -1.39% | -1.87% | -1.75% | 180% | 102% |  |  |  |  |  |
| Class B | -0.70% | -0.62% | -0.76% | 199% | 103% | -1.46% | -1.75% | -1.91% | 180% | 102% | -1.42% | -2.18% | -2.54% | 178% | 102% |
| Class C | -0.70% | -0.88% | -0.89% | 190% | 106% | -1.60% | -1.78% | -2.03% | 192% | 102% | -1.48% | -2.06% | -2.05% | 169% | 104% |
| Class E | -0.84% | -0.48% | -0.47% | 195% | 103% |  |  |  |  |  | -0.85% | -0.54% | -1.06% | 139% | 102% |
| Overall | -0.69% | -0.66% | -0.67% | 193% | 104% | -1.45% | -1.73% | -1.88% | 182% | 102% | -1.30% | -1.73% | -2.01% | 164% | 103% |
| Class D | -0.55% | -1.10% | -1.23% | 187% | 108% | -1.47% | -2.53% | -2.15% | 177% | 101% | -1.54% | -2.63% | -2.59% | 175% | 106% |
| Class F | -0.39% | -0.54% | -0.58% | 170% | 102% | -1.19% | -1.73% | -1.72% | 170% | 101% | -1.17% | -1.33% | -1.18% | 155% | 100% |

Table 4: Performance of the tool-on test in Test 1.4b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.59% | -0.33% | 169% | 103% | -1.04% | -1.41% | -1.53% | 153% | 102% |  |  |  |  |  |
| Class A2 | -0.64% | -0.73% | -0.77% | 178% | 105% | -1.11% | -1.60% | -1.40% | 160% | 101% |  |  |  |  |  |
| Class B | -0.65% | -0.60% | -0.76% | 184% | 103% | -1.20% | -1.68% | -1.57% | 160% | 102% | -1.19% | -1.88% | -1.78% | 156% | 102% |
| Class C | -0.63% | -0.87% | -0.92% | 179% | 105% | -1.35% | -1.67% | -1.79% | 171% | 103% | -1.27% | -1.61% | -1.30% | 148% | 103% |
| Class E | -0.76% | -0.47% | -0.45% | 179% | 103% |  |  |  |  |  | -0.55% | -0.51% | -1.62% | 126% | 101% |
| Overall | -0.63% | -0.66% | -0.67% | 179% | 104% | -1.19% | -1.60% | -1.59% | 161% | 102% | -1.05% | -1.45% | -1.58% | 145% | 102% |
| Class D | -0.50% | -1.09% | -1.17% | 173% | 110% | -1.23% | -2.29% | -1.85% | 160% | 102% | -1.17% | -2.25% | -2.67% | 152% | 105% |
| Class F | -0.35% | -0.68% | -0.58% | 158% | 103% | -0.95% | -1.46% | -1.60% | 154% | 101% | -1.23% | -0.79% | -0.63% | 140% | 102% |

Table 5: Performance of the tool-off test in Test 1.3a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.06% | 0.03% | 154% | 100% | -0.40% | -0.47% | -0.49% | 158% | 99% |  |  |  |  |  |
| Class A2 | -0.19% | -0.08% | -0.11% | 161% | 99% | -0.49% | -0.62% | -0.66% | 161% | 99% |  |  |  |  |  |
| Class B | -0.19% | -0.10% | -0.14% | 174% | 100% | -0.50% | -0.46% | -0.41% | 171% | 99% | -0.40% | -0.38% | -0.07% | 153% | 98% |
| Class C | -0.25% | -0.16% | -0.27% | 184% | 102% | -0.65% | -0.58% | -0.82% | 184% | 102% | -0.58% | -0.95% | -1.01% | 172% | 98% |
| Class E | -0.28% | -0.21% | -0.07% | 175% | 101% |  |  |  |  |  | -0.56% | 0.54% | -0.46% | 143% | 99% |
| Overall | -0.20% | -0.10% | -0.12% | 171% | 100% | -0.52% | -0.53% | -0.59% | 170% | 100% | -0.50% | -0.34% | -0.48% | 156% | 98% |
| Class D | -0.15% | -0.07% | -0.03% | 174% | 104% | -0.50% | -0.42% | -0.50% | 174% | 102% | -0.48% | -1.14% | -0.70% | 166% | 105% |
| Class F | -0.15% | -0.24% | -0.27% | 155% | 101% | -0.42% | -0.48% | -0.47% | 154% | 100% | -0.51% | -0.50% | -0.65% | 146% | 101% |

Table 6: Performance of the tool-off test in Test 1.3b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.12% | -0.02% | 118% | 98% | -0.13% | 0.03% | -0.10% | 114% | 98% |  |  |  |  |  |
| Class A2 | -0.06% | 0.00% | -0.05% | 120% | 98% | -0.17% | -0.23% | -0.26% | 114% | 98% |  |  |  |  |  |
| Class B | -0.04% | -0.02% | -0.10% | 119% | 99% | -0.21% | -0.17% | -0.12% | 117% | 99% | -0.20% | -0.09% | 0.00% | 112% | 99% |
| Class C | -0.08% | -0.11% | -0.13% | 120% | 103% | -0.32% | -0.29% | -0.41% | 118% | 102% | -0.04% | -0.18% | -0.19% | 102% | 98% |
| Class E | -0.09% | -0.17% | -0.04% | 120% | 101% |  |  |  |  |  | -0.09% | -0.27% | -0.71% | 103% | 100% |
| Overall | -0.05% | -0.04% | -0.07% | 119% | 100% | -0.21% | -0.18% | -0.22% | 116% | 99% | -0.12% | -0.16% | -0.24% | 106% | 99% |
| Class D | -0.05% | -0.07% | -0.04% | 120% | 104% | -0.18% | -0.12% | -0.32% | 108% | 102% | -0.02% | 0.28% | -1.06% | 107% | 103% |
| Class F | 0.13% | -0.15% | 0.07% | 108% | 100% | -0.01% | -0.23% | -0.04% | 109% | 99% | 0.11% | 0.80% | 0.61% | 97% | 100% |

Table 7: Performance of the tool-off test in Test 1.4a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.06% | -0.32% | 169% | 99% | -1.02% | -1.21% | -1.40% | 166% | 103% |  |  |  |  |  |
| Class A2 | -0.41% | -0.17% | -0.26% | 174% | 98% | -1.00% | -1.10% | -1.28% | 167% | 100% |  |  |  |  |  |
| Class B | -0.45% | -0.23% | -0.28% | 177% | 97% | -1.07% | -1.40% | -1.26% | 173% | 100% | -1.05% | -2.03% | -1.83% | 163% | 101% |
| Class C | -0.31% | -0.21% | -0.21% | 169% | 99% | -0.98% | -1.25% | -1.47% | 183% | 102% | -0.87% | -1.90% | -1.75% | 157% | 98% |
| Class E | -0.49% | -0.38% | -0.19% | 175% | 104% |  |  |  |  |  | -0.64% | -1.22% | -1.25% | 140% | 103% |
| Overall | -0.41% | -0.21% | -0.25% | 173% | 99% | -1.02% | -1.26% | -1.35% | 173% | 101% | -0.89% | -1.78% | -1.66% | 155% | 101% |
| Class D | -0.20% | -0.21% | -0.12% | 164% | 100% | -0.78% | -1.43% | -0.99% | 165% | 101% | -0.83% | -0.96% | -1.74% | 156% | 101% |
| Class F | -0.11% | -0.22% | -0.24% | 162% | 101% | -0.59% | -0.93% | -0.65% | 155% | 100% | -0.67% | -1.06% | -1.32% | 140% | 102% |

Table 8: Performance of the tool-off test in Test 1.4b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.02% | -0.21% | 155% | 99% | -0.76% | -1.01% | -1.12% | 147% | 103% |  |  |  |  |  |
| Class A2 | -0.40% | -0.28% | -0.31% | 161% | 97% | -0.74% | -0.95% | -0.99% | 152% | 100% |  |  |  |  |  |
| Class B | -0.43% | -0.26% | -0.30% | 164% | 97% | -0.80% | -1.17% | -1.21% | 154% | 100% | -0.69% | -1.38% | -1.02% | 141% | 101% |
| Class C | -0.31% | -0.16% | -0.17% | 161% | 99% | -0.72% | -0.98% | -1.20% | 160% | 102% | -0.59% | -1.51% | -1.18% | 133% | 98% |
| Class E | -0.47% | -0.38% | -0.15% | 163% | 102% |  |  |  |  |  | -0.49% | -1.00% | -1.84% | 126% | 102% |
| Overall | -0.39% | -0.22% | -0.23% | 161% | 99% | -0.76% | -1.04% | -1.15% | 154% | 101% | -0.61% | -1.33% | -1.28% | 135% | 100% |
| Class D | -0.19% | -0.19% | -0.24% | 156% | 101% | -0.52% | -1.09% | -0.50% | 142% | 101% | -0.41% | -1.44% | -0.53% | 138% | 102% |
| Class F | -0.12% | -0.09% | -0.15% | 152% | 99% | -0.33% | -0.66% | -0.39% | 142% | 101% | -0.38% | -0.16% | -0.17% | 125% | 102% |

# Conclusions

This contribution presents two unsymmetric partitioning methods UBT and UQT. The proposed methods contribute promising coding gains. It is proposed to adopt the new partitioning methods in ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “AHG12: Unsymmetric partitioning methods in video coding,” JVET-V0097, Apr. 2021.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).