| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0087-v2 |
| --- | --- |

| Title: | EE2-1.5: A combining test of EE2-1.2 and EE2-1.4b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang F. Le Léannec, K. Naser, T. Dumas, A. Robert, F. Galpin, E. François 8910 University Center Lane San Diego, CA 92122, USA 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com fabrice.leleannec@interdigital.com |
| Source: | Bytedance Inc. InterDigital |  |  |

_____________________________

# Abstract

This contribution presents experimental results for EE2-1.5, which is a combining test of EE2-1.2 and EE2-1.4b. Some algorithms in EE2-1.2 are ported to the code of EE2-1.4b. On top of ECM-1.0, simulation results (BD-rate changes on the luma component, encoding time and decoding time) are reported as below:

Tool-on test (ECM-1.0 turning off all new features as the anchor):

AI: -0.xx%, 1xx%, 1xx%; RA: -0.xx%, 1xx%, 1xx%; LB: -0.xx%, 1xx%, 1xx%.

Tool-off test (ECM-1.0 as the anchor):

AI: -0.xx%, 1xx%, 1xx%; RA: x%, x%, x%; LB: x%, x%, x%.

# Introduction

In JVET-V0097[1], unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) partitioning were proposed. In JVET-V0083[2], asymmetric binary tree splitting (ABT) was proposed, which is similar to UBT conceptually. EE tests on a combination of the method in JVET-V0097 (EE2-1.4b) and the method in JVET-V0083 (EE2-1.2) are reported in this contribution.

# Proposed methods

Some algorithms in EE2-1.2 are ported to the code of EE2-1.4b, comprising the following:

- The maximum MTT hierarchy depth can be configured by temporal layers in the RA and LDB configurations.

- A maximum ABT size parameter is used to enable the use of ABT split mode according to the considered CU size and is configured/used independently from the maximum BT and TT size parameters.

- As opposed to EE2 tests 1.4.b, 1.4.b of UBT, the recursive splitting of coding units with non-dyadic size in a direction is allowed. Typically, a CU with non-dyadic size in a direction can be further split in binary fashion in the considered direction.

- Non symmetric binary split is allowed only if the resulting CU size is 12 or 24.

- The split redundancy limitation normative rules used in test EE2-1.1 and described in JVET-V0083 are used.

In addition, several encoder settings and fast encoding methods, taken from for ABT proposal of contribution JVET-K0197 [5], are used in this EE2 test. They are listed below.

- In tool-off test, the encoding configurations used for classes A, B, C and D are respectively as follows:

| Class A | Class B | Classes C and D |
| --- | --- | --- |
| CTUSize: 256 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxMTTHierarchyDepthByTid: 333333 MaxBTLumaISlice: 64 MaxBTChromaISlice: 64 MaxBTNonISlice: 128 MaxTTLumaISlice: 32 MaxTTChromaISlice: 32 MaxTTNonISlice: 64 MaxABTLumaISlice: 64 MaxABTChromaISlice: 64 MaxABTNonISlice: 64 |

- RD-evaluate ABT split mode according to symmetric binary split mode RD costs. For a given CU, the first evaluated split mode is NO_SPLIT. Then symmetric BT splitting is evaluated, then ABT splitting is evaluated. The ABT splitting is not evaluated if the BT splitting already evaluated in the same orientation is not competitive enough compared to the NO_SPLIT split mode. This takes the following form.

| - If( RDCost(HOR_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric horizontal |
| --- | --- |
| - If( RDCost(VER_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric vertical |
| - If( RDCost(HOR_SPLIT) > β×RDCost(VER_SPLIT) ) | then disable asymmetric horizontal |
| - If( RDCost(VER_SPLIT) > β×RDCost(HOR_SPLIT) ) | then disable asymmetric vertical |

In practice the values α and β are equal to 1.2. This encoder speed-up is used in all tests conducted in this contribution, in all slices.

- Adaptive use of ABT in large blocks: ABT splitting is dynamically activated/deactivated in large blocks (typically 64 in width or height), according to the average use of ABT in large blocks, in past coded picture in the same intra period and temporal depth. If the average usage of blocks larger than 32 in width or height is lower than a threshold in the previous coded picture in same intra period and with same temporal depth, then the maximum block size in which ABT partitioning is allowed is decreased to 32.

- Extension of some heuristics to ABT: some VTM heuristics are extended to ABT, to limit the number of blocks for which ABT partitioning is being tested in the RD optimization process. This concerns the heuristics that deactivates the testing of Ternary Tree partitioning based on the result issued from the testing of the horizontal and vertical binary split that took place before.

# Experimental results

## Test 1.5a: UQT+ABT

The proposed combination is tested on top of ECM-1.0 [3], according to the common test conditions defined in [4]. The results of tool-off test (based on ECM-1.0 with all new features on as the anchor) and tool-on test (based on ECM-1.0 turning off all new features as the anchor) are tabulated in Table 1 and Table 2, respectively.

Table 1: Performance of the UQT+ABT tool-off test in Test 1.5

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | 0.06% | -0.19% | 229% | 95% |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  | -0.68% | -1.50% | -1.50% | 158% | 97% |
| Class B | -0.50% | -0.25% | -0.28% | 232% | 99% | -0.65% | -0.83% | -0.79% | 141% | 107% | -0.46% | -0.61% | -1.89% | 132% | 97% |
| Class C | -0.28% | -0.13% | -0.22% | 214% | 101% | -1.17% | -0.20% | -0.39% | 189% | 97% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | -0.43% | -0.22% | -0.15% | 218% | 103% |  |  |  |  |  | -0.68% | -1.43% | -1.34% | 164% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -1.22% | -0.99% | -0.91% | 153% | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | 0.05% | 0.12% | 208% | 108% | -0.80% | -0.46% | -0.61% | 183% | 100% | -0.68% | -1.50% | -1.50% | 158% | 97% |

Table 2: Performance of the UQT+ABT tool-on test in Test 1.5

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.12% | 0.98% | 1.26% | 109% | 108% |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.67% | -0.55% | -0.59% | 129% | 114% |  |  |  |  |  |
| Class B | -0.82% | -0.46% | -0.62% | 234% | 102% | -0.76% | -0.32% | -0.64% | 128% | 106% | -1.18% | -1.70% | -1.96% | 173% | 104% |
| Class C | -0.69% | -0.77% | -0.76% | 205% | 104% | -0.57% | -0.68% | -0.82% | 131% | 105% | -1.41% | -2.19% | -2.37% | 170% | 100% |
| Class E | -0.87% | -0.40% | -0.36% | 215% | 107% |  |  |  |  |  | -0.49% | -0.19% | -1.64% | 128% | 93% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.56% | -0.20% | -0.30% | 125% | 108% | -1.08% | -1.48% | -2.02% | 160% | 100% |
| Class D |  |  |  |  |  | -0.43% | -1.18% | -1.08% | 141% | 110% | -1.29% | -3.30% | -2.57% | 165% | 97% |

In addition, several trade-offs between encoding time and BD-rate gain are proposed for random access configuration. The tool-off results of these trade-offs are shown in the following Table 3.

Table 3: various trade-offs obtained with UQT+ABT in tool-off case, random access.

- UQT+ABT Trade-off 1, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.57% | -0.73% | -0.68% | 136% | 109% |
| Class C | -1.17% | -0.20% | -0.39% | 189% | 97% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | -0.93% | -0.83% | 148% | #NUM! |
| Class D | -0.80% | -0.46% | -0.61% | 183% | 100% |
| Class F | -1.17% | -0.98% | -1.44% | 153% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- UQT+ABT Trade-off 2, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.83% | -0.79% | 141% | 107% |
| Class C | -1.17% | -0.20% | -0.39% | 189% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.80% | -0.46% | -0.61% | 183% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

(c) UQT+ABT Trade-off 3, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.69% | -0.90% | -0.86% | 148% | 115% |
| Class C | -1.17% | -0.20% | -0.39% | 189% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.80% | -0.46% | -0.61% | 183% | 100% |
| Class F | -1.17% | -0.98% | -1.44% | 153% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

(d) UQT+ABT Trade-off 4, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.61% | -1.93% | -1.89% | 214% | 106% |
| Class C | -0.99% | -1.35% | -1.73% | 184% | 91% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.70% | -1.41% | -1.64% | 177% | 93% |
| Class F | -1.17% | -0.98% | -1.44% | 153% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

The following table xx show the various trade-offs obtained in tool-on configuration with the UQT+ABT partitioning scheme.

Table 4: various enc.time/coding efficiency trade-offs of the UQT-ABT partitioning scheme, in tool-on configuration

- UQT+ABT Trade-off 1, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 1.08% | 1.31% | 106% | 109% |
| Class A2 | -0.61% | -0.42% | -0.49% | 126% | 113% |
| Class B | -0.69% | -0.23% | -0.52% | 125% | 111% |
| Class C | -0.48% | -0.60% | -0.68% | 125% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.50% | -0.11% | -0.19% | 121% | 113% |
| Class D | -0.35% | -1.04% | -1.00% | 134% | 111% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- UQT+ABT Trade-off 2, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.98% | 1.26% | 109% | 108% |
| Class A2 | -0.67% | -0.55% | -0.59% | 129% | 114% |
| Class B | -0.76% | -0.32% | -0.64% | 128% | 106% |
| Class C | -0.57% | -0.68% | -0.82% | 131% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -0.20% | -0.30% | 125% | 108% |
| Class D | -0.43% | -1.18% | -1.08% | 141% | 110% |
| Class F | -0.09% | -0.39% | -0.40% | 125% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- UQT+ABT Trade-off 3, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.97% | 1.19% | 113% | 109% |
| Class A2 | -0.70% | -0.55% | -0.62% | 132% | 113% |
| Class B | -0.80% | -0.34% | -0.66% | 130% | 104% |
| Class C | -0.62% | -0.76% | -0.91% | 133% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.60% | -0.23% | -0.35% | 128% | 106% |
| Class D | -0.48% | -1.29% | -1.18% | 142% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- UQT+ABT Trade-off 4, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | 0.17% | 0.08% | 167% | 108% |
| Class A2 | -1.44% | -1.42% | -1.52% | 186% | 114% |
| Class B | -1.55% | -1.23% | -1.55% | 189% | 106% |
| Class C | -1.71% | -2.07% | -2.24% | 205% | 105% |
| Class E |  |  |  |  |  |
| Overall | -1.44% | -1.21% | -1.40% | 188% | 108% |
| Class D | -1.47% | -2.45% | -2.44% | 205% | 110% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Test 1.5b: ABT (based on test 1.4 of JVET-W0086)

This section presents the results obtained when using ABT over the code base of EE2 test 1.4, where the ABT aspects listed in section 2

Table 5: Performance of the ABT tool-off test in Test 1.5

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | 0.08% | -0.06% | 201% | 98% | -1.70% | -1.03% | -1.17% | 120% | 112% |  |  |  |  |  |
| Class A2 | -0.39% | -0.12% | -0.19% | 191% | 96% |  |  |  |  |  | -0.68% | -1.50% | -1.50% | 158% | 97% |
| Class B | -0.40% | -0.06% | -0.17% | 192% | 99% | -0.43% | -0.60% | -0.59% | 115% | 109% | -0.46% | -0.61% | -1.89% | 132% | 97% |
| Class C | -0.15% | 0.03% | -0.08% | 172% | 100% | -0.81% | 0.11% | 0.01% | 147% | 96% |  |  |  |  |  |
| Class E | -0.26% | -0.14% | -0.15% | 177% | 102% |  |  |  |  |  | -0.68% | -1.43% | -1.34% | 164% | 97% |
| Overall | -0.32% | -0.04% | -0.13% | 186% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.06% | 0.17% | 166% | 105% | -0.55% | -0.05% | -0.07% | 148% | 99% | -0.68% | -1.50% | -1.50% | 158% | 97% |

Table 6: Performance of the ABT tool-on test in Test 1.5

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 0.02% | 1.17% | 1.38% | 90% | 115% |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.40% | -0.17% | -0.36% | 103% | 113% |  |  |  |  |  |
| Class B | -0.73% | -0.33% | -0.60% | 187% | 98% | -0.52% | -0.14% | -0.23% | 103% | 114% | -0.96% | -1.73% | -1.96% | 138% | 101% |
| Class C | -0.58% | -0.67% | -0.74% | 162% | 97% | -0.28% | -0.39% | -0.40% | 105% | 105% | -1.31% | -1.72% | -1.95% | 155% | 102% |
| Class E | -0.71% | -0.31% | -0.35% | 168% | 94% |  |  |  |  |  | -0.54% | -0.40% | -0.86% | 126% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.32% | 0.05% | 0.02% | 101% | 112% | -0.97% | -1.40% | -1.68% | 140% | 102% |
| Class D | -0.37% | -0.74% | -0.92% | 158% | 100% | -0.27% | -1.15% | -1.06% | 115% | 113% | -1.26% | -2.97% | -2.15% | 149% | 98% |

In addition, several trade-offs between encoding time and BD-rate gain are proposed for random access configuration. The tool-off results of these trade-offs are shown in the following tables.

Table 7: various trade-offs obtained with ABT in tool-off case, random access.

- ABT Trade-off 1, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.64% | -1.02% | -1.12% | 115% | 112% |
| Class A2 | -1.25% | -1.75% | -0.77% | 123% | 114% |
| Class B | -0.34% | -0.52% | -0.47% | 109% | 105% |
| Class C | -0.81% | 0.11% | 0.01% | 147% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.91% | -0.70% | -0.53% | 122% | 106% |
| Class D | -0.55% | -0.05% | -0.07% | 148% | 99% |
| Class F | -1.17% | -0.98% | -1.44% | 153% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 2, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -1.03% | -1.17% | 120% | 112% |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | -0.60% | -0.59% | 115% | 109% |
| Class C | -0.81% | 0.11% | 0.01% | 147% | 96% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.55% | -0.05% | -0.07% | 148% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 3, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.74% | -1.05% | -1.23% | 123% | 115% |
| Class A2 |  |  |  |  |  |
| Class B | -0.48% | -0.65% | -0.61% | 118% | 112% |
| Class C | -0.81% | 0.11% | 0.01% | 147% | 96% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.55% | -0.05% | -0.07% | 148% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 4, tool-off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.35% | -1.64% | -1.66% | 169% | 110% |
| Class C | -0.81% | 0.11% | 0.01% | 147% | 96% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.55% | -0.05% | -0.07% | 148% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 8: various trade-offs obtained with UQT+ABT in tool-on case, random access.

- ABT Trade-off 1, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 1.23% | 1.47% | 88% | 114% |
| Class A2 | -0.31% | -0.11% | -0.27% | 103% | 114% |
| Class B | -0.46% | -0.05% | -0.14% | 101% | 109% |
| Class C | -0.18% | -0.25% | -0.26% | 102% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | 0.14% | 0.13% | 99% | 111% |
| Class D | -0.20% | -1.03% | -0.95% | 114% | 117% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 2, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 1.17% | 1.38% | 90% | 115% |
| Class A2 | -0.40% | -0.17% | -0.36% | 103% | 113% |
| Class B | -0.52% | -0.14% | -0.23% | 103% | 114% |
| Class C | -0.28% | -0.39% | -0.40% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | 0.05% | 0.02% | 101% | 112% |
| Class D | -0.27% | -1.15% | -1.06% | 115% | 113% |
| Class F | 0.14% | -0.12% | -0.21% | 109% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 3, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 1.13% | 1.33% | 93% | 110% |
| Class A2 | -0.42% | -0.22% | -0.37% | 108% | 119% |
| Class B | -0.56% | -0.16% | -0.22% | 106% | 107% |
| Class C | -0.32% | -0.43% | -0.45% | 108% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | 0.01% | 0.00% | 104% | 111% |
| Class D | -0.32% | -1.19% | -1.08% | 119% | 113% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- ABT Trade-off 4, tool-on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | 0.32% | 0.22% | 134% | 115% |
| Class A2 | -1.15% | -1.03% | -1.24% | 143% | 113% |
| Class B | -1.28% | -1.04% | -1.17% | 147% | 114% |
| Class C | -1.40% | -1.75% | -1.86% | 156% | 105% |
| Class E |  |  |  |  |  |
| Overall | -1.18% | -0.95% | -1.09% | 146% | 112% |
| Class D | -1.30% | -2.34% | -2.34% | 165% | 113% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution presents a combination test of EE2-1.1 and EE2-1.4b. The proposed methods contribute promising coding gains. It is proposed to adopt them in ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “AHG12: Unsymmetric partitioning methods in video coding,” JVET-V0097, Apr. 2021.

- F. Le Léannec, K. Naser, T. Dumas, A. Robert, F. Galpin, E. François, “EE2 related: asymmetric binary tree splitting on top of VVC,” JVET-V0083, Apr. 2021.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

- “CE1: Asymmetric Binary Tree (tests 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1 and 8.0.2)”, F. Le Léannec, T. Poirier, F. Galpin (Technicolor), JVET-K0197, July 2018

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).