| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0088-v2 |
| --- | --- |

| Title: | EE2-3.3: GPM with MMVD (JVET-V0103 and JVET-V0125) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Kai Zhang, Li Zhang Na Zhang, Yang Wang (Bytedance) Xiaoyu Xiu, C.-W. Kuo, Xianglin Wang (Kwai) | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com {xiaoyuxiu, cheweikuo, xianglinwang}@kwai.com |
| Source: | Bytedance Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-3.3 on GPM with MMVD. In the EE test, the GPM coding efficiency is further enhanced by extending the MMVD approach to the GPM mode. Simulation results are summarized as below, where the tool-on test is based on ECM-1.0 with all new normative and non-normative changes being disabled; and the tool-off test is based on ECM-1.0:

|  | Tool-off |  |  |  |  |  | Tool-on |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y |  | U | V | Enc | Dec |
| RA | -0.22% | -0.43% | -0.48% | 106% | 99% | -0.43% |  | -0.80% | -0.86% | 109% | 98% |
| LD | -0.35% | -0.65% | -0.60% | 107% | 98% | -0.70% |  | -1.24% | -1.22% | 110% | 98% |

V2 of this proposal updates the tool-on results of EE2-3.3.

# Introduction

In JVET-R0357 [1], JVET-V0103 [2] and JVET-V0125 [3], geometry partition mode (GPM) with MMVD was proposed to further improve the coding efficiency of the GPM mode in the VVC. Specifically, in those schemes, additional MV differences (MVDs) are further applied on top of the existing GPM merge candidates to improve the precision of the MVs used by the two GPM partitions. Moreover, to reduce the signaling overhead, the MVDs are signaled in the same manner as the merge mode with MVD (MMVD) in the VVC. At JVET-V meeting, GPM with MMVD was included in exploration experiment (EE) 2-3.3 [4] for further evaluation.

# Proposal

This contribution reports the results of EE2-3.3. Specifically, in EE2-3.3, two flags are signaled to separately indicate whether additional MVD is applied to each GPM partition. When the flag of one GPM partition is true, its corresponding MVD is signaled in the same way as the MMVD, i.e., one distance index plus one direction index. To enable more MV combinations, the merge indices of two GPM partitions are allowed to be the same when the MVDs that are applied to the two partitions are not identical. Additionally, an MV pruning procedure is introduced to construct the GPM merge candidate list when GPM with MMVD is applied.

Additionally, two different sets of MVDs are supported for the GPM in EE2-3.3 which are selected according to one indication flag at picture header. When the flag is equal to 0, the existing MVD set used by the MMVD, which includes 8 distances {1/4-pel, 1/2-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel} and 4 horizontal/vertical directions, are supported for the GPM CUs in the current picture; otherwise, another MVD set, which include 9 distance {1/4-pel, 1/2-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel} and 8 directions (4 horizontal/vertical directions plus 4 diagonal directions), are applied.

# Experimental Results

The proposed method was implemented on top of ECM-1.0 [5] and tested following the common test conditions defined in [6]. Both the results of tool-on test (based on ECM-1.0 with all new normative and non-normative changes being disabled) and tool-off test (based on ECM-1.0) are tabulated in Table 1 and Table 2, respectively.

Table 1: Performance of EE2-3.3 test (tool-on)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.48% | -0.71% | 110% | 98% |  |  |  |  |  |
| Class A2 | -0.47% | -0.75% | -0.73% | 110% | 99% |  |  |  |  |  |
| Class B | -0.34% | -0.74% | -0.71% | 108% | 98% | -0.52% | -1.14% | -1.06% | 111% | 96% |
| Class C | -0.64% | -1.15% | -1.26% | 107% | 98% | -0.99% | -1.58% | -1.96% | 112% | 100% |
| Class E |  |  |  |  |  | -0.62% | -0.95% | -0.48% | 107% | 96% |
| Overall | -0.43% | -0.80% | -0.86% | 109% | 98% | -0.70% | -1.24% | -1.22% | 110% | 98% |
| Class D | -0.61% | -1.23% | -1.22% | 110% | 101% | -1.04% | -1.98% | -1.97% | 113% | 99% |
| Class F | -0.28% | -0.52% | -0.61% | 106% | 99% | -0.57% | -1.43% | -1.56% | 107% | 97% |

Table 2: Performance of EE2-3.3 test (tool-off)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.34% | -0.41% | 104% | 99% |  |  |  |  |  |
| Class A2 | -0.25% | -0.34% | -0.38% | 106% | 100% |  |  |  |  |  |
| Class B | -0.17% | -0.42% | -0.36% | 107% | 98% | -0.23% | -0.53% | -0.15% | 107% | 99% |
| Class C | -0.33% | -0.58% | -0.76% | 107% | 100% | -0.50% | -1.23% | -1.18% | 108% | 97% |
| Class E |  |  |  |  |  | -0.36% | -0.08% | -0.58% | 105% | 98% |
| Overall | -0.22% | -0.43% | -0.48% | 106% | 99% | -0.35% | -0.65% | -0.60% | 107% | 98% |
| Class D | -0.28% | -0.58% | -0.63% | 107% | 100% | -0.45% | -0.67% | -0.64% | 108% | 98% |
| Class F | -0.12% | -0.23% | -0.15% | 107% | 100% | -0.29% | -0.21% | -0.69% | 106% | 99% |

# Conclusions

The contribution reports the results of EE2-3.3 on GPM with MMVD. It is asserted that the proposed method does not obviously increase hardware implementation cost compared to the existing GPM design in VVC while bringing promising coding gains. It is proposed to adopt the proposed method to the ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang, “Geometric prediction mode with motion vector differences,” JVET-R0357, Apr. 2020.

- Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang, “AHG12: Geometric prediction mode with motion vector differences,” JVET-V0103, Apr. 2021.

- X. Xiu, H.-J. Jhu, C.-W. Kuo, W. Chen, Y.-W. Chen, X. Wang, “AHG12: Evaluation of GPM with MMVD for coding efficiency improvement over VVC”, JVET-V0125, Apr. 2021.

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-V2024, Apr. 2021.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).