| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0090-v1 |
| --- | --- |

| Title: | EE2-3.1/EE2-3.2: Adaptive Reordering of Merge Candidates with Template/Bilateral Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2, Hongbin Liu1, Zhipin Deng1,Yang Wang1 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.1 on adaptive reordering of merge candidates with template matching (ARMC-TM) and EE2-3.2 on adaptive reordering of merge candidates with bilateral matching (ARMC-BM). With the proposed methods, merge candidates are divided into several subgroups, and merge candidates in a subgroup are reordered according to cost values based on template matching in EE2-3.1 and bilateral matching in EE2-3.2. ARMC-TM is applied to the regular merge mode, the template matching merge mode, and the affine merge mode. ARMC-BM is only applied to the regular merge mode.

On top of the ECM-1.0, simulation results of the proposed methods are reported as follows:

Tool-on test of EE2-3.1: RA: -0.41%, 100%, 109%; LB:	-0.48%, 100%, 111%

Tool-off test of EE2-3.1: RA: -0.33%, 99%, 103%; LB: -0.45%, 99%, 104%

Tool-on test of EE2-3.2: RA: -0.01%, 100%, 103%; LB:	0.00%, 100%, 101%

Tool-off test of EE2-3.2: RA:

# Introduction

In current ECM design, merge candidate list is generated in a fixed order, which may not be optimal. Adaptive merge candidate list generation process can be used to improve the coding efficiency of the merge candidate index. In JVET-V0099, an adaptive reordering of merge candidates with template matching (ARMC) method was proposed to improve the coding efficiency.

# Proposed methods

In this contribution, adaptive reordering of merge candidates with template matching (ARMC-TM) and adaptive reordering of merge candidates with bilateral matching (ARMC-BM) are tested per the description of EE2.

## ARMC-TM

The reordering method is applied to regular merge mode, template matching (TM) merge mode, and affine merge mode (excluding the SbTMVP candidate). For the TM merge mode, merge candidates are reordered before the refinement process.

After a merge candidate list is constructed, merge candidates are divided into several subgroups. The subgroup size is set to 5 for regular merge mode and TM merge mode. The subgroup size is set to 3 for affine merge mode. Merge candidates in each subgroup are reordered ascendingly according to cost values based on template matching. For simplification, merge candidates in the last but not the first subgroup are not reordered.

The template matching cost of a merge candidate is measured by the sum of absolute differences (SAD) between samples of a template of the current block and their corresponding reference samples. The template comprises a set of reconstructed samples neighboring to the current block. Reference samples of the template are located by the motion information of the merge candidate.

When a merge candidate utilizes bi-directional prediction, the reference samples of the template of the merge candidate are also generated by bi-prediction as shown in Figure 1.

Figure 1. Template and reference samples of the template in reference pictures.

For subblock-based merge candidates with subblock size equal to Wsub × Hsub, the above template comprises several sub-templates with the size of Wsub × 1, and the left template comprises several sub-templates with the size of 1 × Hsub. As shown in Figure 2, the motion information of the subblocks in the first row and the first column of current block is used to derive the reference samples of each sub-template.

Figure 2. Template and reference samples of the template for block with sub-block motion using the motion information of the subblocks of the current block.

## ARMC-BM

ARMC-BM is only applied to regular merge mode. In order to verify the effectiveness of the scheme, the successive first N merge candidates (starting from the first merge candidate) satisfying all the following conditions are reordered.

- Merge candidates utilize the bi-directional prediction.

- One reference picture is before while the other reference picture is after the current picture in the displaying order.

- The distances (i.e. POC differences) from two reference pictures to the current picture are same.

- BCW weighting index is equal to BCW_DEFAULT.

- Merge candidates come from adjacent spatial or temporal neighbouring blocks.

The bilateral matching cost of a merge candidate is measured by the sum of absolute differences (SAD) between the two reference blocks of the current block from reference list 0 and reference list 1 which are located by the motion information of the merge candidate as shown in Figure 3.

Figure 3. Reference blocks of the current block from reference list 0 and reference list 1

# Experimental Results

The proposed methods have been implemented on top of the ECM-1.0 [1]. Tool-on/Tool-off testing results under RA/LDB configurations are summarized in Table 1~Table 4, respectively. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the ARMC-TM (tool-on test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.47% | -0.41% | 101% | 104% |  |  |  |  |  |
| Class A2 | -0.49% | -0.58% | -0.64% | 100% | 107% |  |  |  |  |  |
| Class B | -0.40% | -0.47% | -0.42% | 100% | 110% | -0.40% | -0.47% | -0.22% | 100% | 111% |
| Class C | -0.38% | -0.45% | -0.51% | 100% | 113% | -0.38% | -0.32% | -0.41% | 100% | 115% |
| Class E |  |  |  |  |  | -0.75% | -1.22% | -1.04% | 102% | 107% |
| Overall | -0.41% | -0.49% | -0.49% | 100% | 109% | -0.48% | -0.61% | -0.49% | 100% | 111% |
| Class D | -0.31% | -0.47% | -0.50% | 100% | 117% | -0.34% | -0.39% | -0.34% | 100% | 117% |
| Class F | -0.38% | -0.43% | -0.46% | 99% | 108% | -0.64% | -0.47% | -0.74% | 99% | 110% |

Table 2: Coding performance of the ARMC-TM (tool-off test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.51% | -0.34% | 100% | 103% |  |  |  |  |  |
| Class A2 | -0.29% | -0.26% | -0.28% | 100% | 103% |  |  |  |  |  |
| Class B | -0.31% | -0.24% | -0.32% | 99% | 103% | -0.26% | -0.26% | 0.53% | 99% | 104% |
| Class C | -0.37% | -0.35% | -0.38% | 98% | 104% | -0.36% | -0.60% | -0.22% | 99% | 105% |
| Class E |  |  |  |  |  | -0.90% | -0.48% | -0.78% | 99% | 103% |
| Overall | -0.33% | -0.33% | -0.33% | 99% | 103% | -0.45% | -0.43% | -0.05% | 99% | 104% |
| Class D | -0.27% | -0.20% | -0.14% | 98% | 104% | -0.12% | -0.46% | -0.21% | 99% | 107% |
| Class F | -0.40% | -0.45% | -0.28% | 97% | 102% | -0.36% | -0.37% | -0.79% | 99% | 106% |

Table 3: Coding performance of the ARMC-BM (tool-on test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.02% | -0.08% | -0.09% | 100% | 102% |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.02% | 100% | 103% | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | -0.02% | -0.04% | -0.07% | 100% | 103% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | -0.01% | -0.02% | -0.03% | 100% | 103% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.06% | -0.06% | -0.04% | 100% | 105% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | -0.02% | -0.01% | 0.00% | 99% | 101% | 0.00% | 0.00% | 0.00% | 99% | 101% |

Table 4: Coding performance of the ARMC-BM (tool-off test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Conclusions

In this contribution, ARMC-TM and ARMC-BM methods are proposed. ARMC-TM method can bring some coding gain with a good trade-off between coding performance and complexity. It is recommended to adopt ARMC-TM into ECM.

# References

[1]	https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-1.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-V2017, Apr. 2021.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).