In VVC (ITU-T H.266 | ISO/IEC 23090-3)

Modify clause D.9.2 as follows

    - Use of the film grain characteristics SEI message

For purposes of interpretation of the film grain characteristics SEI message, the following variables are specified:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

To

    - Use of the film grain characteristics SEI message

For purposes of interpretation of the film grain characteristics SEI message, the following variables are specified:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	When the bit-accurate grain blending process is invoked, PicOrderCnt is set equal to PicOrderCntVal.

In section 5.8, add the following mathematic functions:

Log2( x )	base-2 logarithm of x.		(xx)

Min( x, y ) =   x;x <= yy;x > y		(xx)

## Film grain characteristics SEI message

### Film grain characteristics SEI message syntax

| film_grain_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| fg_characteristics_cancel_flag | u(1) |
| if( !fg_characteristics_cancel_flag ) { |  |
| fg_model_id | u(2) |
| fg_separate_colour_description_present_flag | u(1) |
| if( fg_separate_colour_description_present_flag ) { |  |
| fg_bit_depth_luma_minus8 | u(3) |
| fg_bit_depth_chroma_minus8 | u(3) |
| fg_full_range_flag | u(1) |
| fg_colour_primaries | u(8) |
| fg_transfer_characteristics | u(8) |
| fg_matrix_coeffs | u(8) |
| } |  |
| fg_blending_mode_id | u(2) |
| fg_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fg_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if( fg_comp_model_present_flag[ c ] ) { |  |
| fg_num_intensity_intervals_minus1[ c ] | u(8) |
| fg_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fg_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fg_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fg_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fg_num_model_values_minus1[ c ]; j++ ) |  |
| fg_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fg_characteristics_persistence_flag | u(1) |
| } |  |
| } |  |

### Film grain characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for film grain synthesis.

NOTE 1 – For example, an encoder could use the film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques. Alternatively, an encoder could use the film grain characteristics SEI message to affect the visibility of compression artefacts. Synthesis of simulated film grain on the decoded images for the display process is optional and does not need to exactly follow the specified semantics of the film grain characteristics SEI message. When synthesis of simulated film grain on the decoded images for the display process is performed, there is no requirement that the process by which the synthesis is performed be the same as the process described in the current version of this Specification.

NOTE 2 – The display process is not specified in this Specification.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	When the syntax element fg_separate_colour_description_present_flag of the film grain characteristics SEI message is equal to 0, the following additional variables:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	When the process specified in clause 1.1.3 is invoked, a picture order count of the current picture, denoted herein by PicOrderCount, which is used to seed a pseudo-random number generator.

NOTE 3 – SMPTE RDD 5 specifies a film grain simulator based on the information provided in the film grain characteristics SEI message for high definition 8-bit 4:2:0 chroma format video material (with PicWidthInLumaSamples less than or equal to 1920,  PicHeightInLumaSamples less than or equal to 1080, BitDepthY equal to 8, BitDepthC equal to 8, and ChromaFormatIdc equal to 1) when film grain block size is 8x8.

NOTE 4 – Clause x.x specifies a bit-accurate film grain blending process based on the information provided in the film grain characteristics SEI message. The process is identical to the process specified in SMPTE RDD 5, Film Grain Technology, except for two modifications: 1) use of a transform already specified in VVC instead of the transform specified in SMPTE RDD 5; and 2) increase in grain block size for picture width greater than 1920 or picture height greater than 1080.

The process specified in clause 1.1.3 is for application to decoded pictures that have 4:2:0 colour format and uses the same luma and chroma bit depth and the same colour representation domain as the decoded pictures. When the process specified in clause 1.1.3 is not invoked, the film grain models specified in the film grain characteristics SEI message are expressed for application to decoded pictures that have 4:4:4 colour format with luma and chroma bit depths corresponding to the luma and chroma bit depths of the film grain model and use the same colour representation domain as the identified film grain model. When the process specified in clause 1.1.3 is not invoked and when the colour format of the decoded video is not 4:4:4 or the decoded video uses a different luma or chroma bit depth from that of the film grain model or uses a different colour representation domain from that of the identified film grain model, an unspecified conversion process is expected to be applied to convert the decoded pictures to the form that is expressed for application of the film grain model.

NOTE 5 – When the process specified in clause 1.1.3 is not invoked, a decoder could, if desired, down-convert the model information for chroma in order to simulate film grain for other chroma formats (4:2:0 or 4:2:2) rather than up-converting the decoded video (using a method not specified in this Specification) before performing film grain generation.

fg_characteristics_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous film grain characteristics SEI message in output order that applies to the current layer. fg_characteristics_cancel_flag equal to 0 indicates that film grain modelling information follows.

fg_model_id identifies the film grain simulation model as specified in Table 5. The value of fg_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fg_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fg_model_id equal to 2 or 3.

Table 5 – fg_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

fg_separate_colour_description_present_flag equal to 1 indicates that a distinct combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message is present in the film grain characteristics SEI message syntax. fg_separate_colour_description_present_flag equal to 0 indicates that the combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message are the same as indicated in VUI parameters for the CLVS.

NOTE 6 – When fg_separate_colour_description_present_flag is equal to 1, any of the luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients specified for the film grain characteristics specified in the SEI message could differ from that for the pictures in the CLVS.

When VUI parameters are not present for the CLVS or the value of vui_colour_description_present_flag is equal to 0, and equivalent information to that conveyed when vui_colour_description_present_flag is equal to 1 is not conveyed by external means, fg_separate_colour_description_present_flag shall be equal to 1.

The decoded image Idecoded used in the equations in this clause is in the same colour representation domain as the simulated film grain signal. Therefore, when any of these parameters does differ from that for the pictures in CLVS, the decoded image Idecoded used in the equations in this clause would be in a different colour representation domain than that for the pictures in the CLVS. For example, when the value of fg_bit_depth_luma_minus8 + 8 is greater than BitDepthY (i.e., the bit depth of the luma component of the pictures in the CLVS), the bit depth of Idecoded used in the equations in this clause is also greater than BitDepthY. In such a case, the decoded image Idecoded corresponding to an actual decoded picture would be generated by converting the actual decoded picture to be in the same colour representation domain as the simulated film grain signal. The process for converting an actual decoded picture to the 4:4:4 colour format with same colour representation domain as the simulated film grain signal is not specified in this Specification.

fg_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the film grain characteristics specified in the SEI message. When fg_bit_depth_luma_minus8 is not present in the film grain characteristics SEI message, the value of fg_bit_depth_luma_minus8 is inferred to be equal to BitDepthY − 8.

The value of fgBitDepth[ 0 ] is derived as follows:

fgBitDepth[ 0 ] = fg_bit_depth_luma_minus8 + 8		(16)

fg_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the Cb and Cr components of the film grain characteristics specified in the SEI message. When fg_bit_depth_chroma_minus8 is not present in the film grain characteristics SEI message, the value of fg_bit_depth_chroma_minus8 is inferred to be equal to BitDepthC − 8.

The value of fgBitDepth[ c ] for c = 1 and 2 is derived as follows:

fgBitDepth[ c ] = fg_bit_depth_chroma_minus8 + 8, with c = 1, 2	(17)

fg_full_range_flag has the same semantics as specified in clause 7.3 for the vui_full_range_flag syntax element, except as follows:

–	fg_full_range_flag specifies the video full range flag of the film grain characteristics specified in the SEI message, rather than the video full range flag used for the CLVS.

–	When fg_full_range_flag is not present in the film grain characteristics SEI message, the value of fg_full_range_flag is inferred to be equal to vui_full_range_flag.

fg_colour_primaries has the same semantics as specified in clause 7.3 for the vui_colour_primaries syntax element, except as follows:

–	fg_colour_primaries specifies the colour primaries of the film grain characteristics specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When fg_colour_primaries is not present in the film grain characteristics SEI message, the value of fg_colour_primaries is inferred to be equal to vui_colour_primaries.

fg_transfer_characteristics has the same semantics as specified in clause 7.3 for the vui_transfer_characteristics syntax element, except as follows:

–	fg_transfer_characteristics specifies the transfer characteristics of the film grain characteristics specified in the SEI message, rather than the transfer characteristics used for the CLVS.

–	When fg_transfer_characteristics is not present in the film grain characteristics SEI message, the value of fg_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

fg_matrix_coeffs has the same semantics as specified in clause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	fg_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When fg_matrix_coeffs is not present in the film grain characteristics SEI message, the value of fg_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for fg_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

fg_blending_mode_id identifies the blending mode used to blend the simulated film grain with the decoded images as specified in Table 6. fg_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fg_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fg_blending_mode_id equal to 2 or 3.

Table 6 – fg_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of fg_blending_mode_id, the blending mode is specified as follows:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(18)

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] +	(19)
			Round( ( Idecoded[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Idecoded[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Idecoded[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1.

fg_log2_scale_factor specifies a scale factor used in the film grain characterization equations.

fg_comp_model_present_flag[ c ] equal to 0 indicates that film grain is not modelled on the c-th colour component, where c equal to 0 refers to the luma component, c equal to 1 refers to the Cb component, and c equal to 2 refers to the Cr component. fg_comp_model_present_flag[ c ] equal to 1 indicates that syntax elements specifying modelling of film grain on colour component c are present in the SEI message.

When fg_separate_colour_description_present_flag is equal to 0 and ChromaFormatIdc is equal to 0, the value of fg_comp_model_present_flag[ 1 ] and fg_comp_model_present_flag[ 2 ] shall be equal to 0.

fg_num_intensity_intervals_minus1[ c ] plus 1 specifies the number of intensity intervals for which a specific set of model values has been estimated.

NOTE 7 – The intensity intervals could overlap in order to simulate multi-generational film grain.

fg_num_model_values_minus1[ c ] plus 1 specifies the number of model values present for each intensity interval in which the film grain has been modelled. The value of fg_num_model_values_minus1[ c ] shall be in the range of 0 to 5, inclusive.

fg_intensity_interval_lower_bound[ c ][ i ] specifies the lower bound of the i-th intensity interval for which the set of model values applies.

fg_intensity_interval_upper_bound[ c ][ i ] specifies the upper bound of the i-th intensity interval for which the set of model values applies.

The variable intensityIntervalIdx[ c ][ x ][ y ][ j ] represents the j-th index to the list of intensity intervals selected for the sample value Idecoded[ c ][ x ][ y ] for c = 0..2, x = 0..PicWidthInLumaSamples − 1, y = 0..PicHeightInLumaSamples − 1, and j = 0..numApplicableIntensityIntervals[ c ][ x ][ y ] − 1, where numApplicableIntensityIntervals[ c ][ x ][ y ] is derived below.

Depending on the value of fg_model_id, the selection of the one or more intensity intervals for the sample value Idecoded[ c ][ x ][ y ] is specified as follows:

–	The variable numApplicableIntensityIntervals[ c ][ x ][ y ] is initially set equal to 0. 	If fg_model_id is equal to 0, the following applies:

- The top-left sample location ( xB, yB ) of the current 8x8 block b that contains the sample value Idecoded[ c ][ x ][ y ] is derived as ( xB, yB ) = ( x / 8blockAvgSize, y / 8 ).

- The average value bavg of the current 8x8 block b is derived as follows:

sum8x8 = 0
for( i = 0; i < 8; i++ )
	for( j = 0; j < 8; j++ )
		sum8x8  +=  Idecoded[ c ][ xB * 8 + i ][yB * 8 + j ]	(20)
bavg = Clip3( 0, 255, ( sum8x8 + ( 1  <<  ( fgBitDepth[ c ] − 3 ) ) )  >>  ( fgBitDepth[ c ] − 2 ) )

- The value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

for( i = 0, j = 0; i  <=  fg_num_intensity_intervals_minus1[ c ]; i++ )
	if( bavg  >=  fg_intensity_interval_lower_bound[ c ][ i ]  &&
			bavg  <=  fg_intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i		(21)
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

–	Otherwise (fg_model_id is equal to 1), the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

I8[ c ][ x ][ y ] = ( fgBitDepth[ c ]  = =  8 ) ? ( Idecoded[ c ][ x ][ y ] :
		Clip3( 0, 255, ( Idecoded[ c ][ x ][ y ] + ( 1  <<  ( fgBitDepth[ c ] − 9 ) ) )  >>  ( fgBitDepth[ c ] − 8 ) )
for( i = 0, j = 0; i  <=  fg_num_intensity_intervals_minus1[ c ]; i++ )
	if( I8[ c ][ x ][ y ]  >=  fg_intensity_interval_lower_bound[ c ][ i ]  &&	(22)
			I8[ c ][ x ][ y ]  <=  fg_intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

Samples that do not fall into any of the defined intervals (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is equal to 0) are not modified by the grain generation function. Samples that fall into more than one interval (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is greater than 1) will originate multi-generation grain. Multi-generation grain results from adding the grain computed independently for each of the applicable intensity intervals.

In the equations in the remainder of this clause, the variable sj in each instance of the list fg_comp_model_value[ c ][ sj ] is the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] derived for the sample value Idecoded[ c ][ x ][ y ].

fg_comp_model_value[ c ][ i ][ j ] specifies the j-th model value for the colour component c and the i-th intensity interval. The set of model values has different meaning depending on the value of fg_model_id.

The value of fg_comp_model_value[ c ][ i ][ j ] is constrained as follows, and could be additionally constrained as specified elsewhere in this clause:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2fgBitDepth[ c ] − 1, inclusive.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ j ] shall be in the range of −2( fgBitDepth[ c ] − 1 ) to 2( fgBitDepth[ c ] − 1 ) − 1, inclusive.

Depending on the value of fg_model_id, the synthesis of the film grain is modelled as follows:

–	If fg_model_id is equal to 0, a frequency filtering model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1 as specified by:

G[ c ][ x ][ y ] = ( fg_comp_model_value[ c ][ sj ][ 0 ] * Q[ c ][ x ][ y ] + fg_comp_model_value[ c ][ sj ][ 5 ] *
		G[ c − 1 ][ x ][ y ] )  >>  fg_log2_scale_factor	(23)

where Q[ c ] is a two-dimensional random process generated by filtering 16x16 blocks gaussRv with random-value elements gaussRvij generated with a normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance) and where the value of an element G[ c − 1 ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when c − 1 is less than 0.

NOTE 8 – A normalized Gaussian random variable can be generated from two independent, uniformly distributed random values over the interval from 0 to 1 (and not equal to 0), denoted as uRv0 and uRv1, using the Box-Muller transformation specified by:

gaussRvi,j = Sqrt( −2 * Ln( uRv0 ) ) * Cos( 2 * π * uRv1 )	(24)

where  is Archimedes' constant 3.141 592 653 589 793....

The band-pass filtering of blocks gaussRv can be performed in the discrete cosine transform (DCT) domain as follows:

for( y = 0; y < 16; y++ )
	for( x = 0; x < 16; x++ )
		if( ( x < fg_comp_model_value[ c ][ sj ][ 3 ]  &&  y < fg_comp_model_value[ c ][ sj ][ 4 ] )  | |	(25)
				x > fg_comp_model_value[ c ][ sj ][ 1 ]  | |  y > fg_comp_model_value[ c ][ sj ][ 2 ] )
			gaussRv[ x ][ y ] = 0
filteredRv = IDCT16x16( gaussRv )

where IDCT16x16( z ) refers to a unitary inverse discrete cosine transformation (IDCT) operating on a 16x16 matrix argument z as specified by:

IDCT16x16( z ) = r * z * rT		(26)

where the superscript T indicates a matrix transposition and r is the 16x16 matrix with elements rij specified by:

ri,j = (  i  = =  0  ? 1  : Sqrt( 2 )4* Cos i *  2 * j + 1   * π32 		(27)

where  is Archimedes' constant 3.141 592 653 589 793....

Q[ c ] is formed by the frequency-filtered blocks filteredRv.

NOTE 9 – Coded model values are based on blocks of size , but a decoder implementation could use other block sizes. For example, decoders implementing the  on 8x8 blocks could down-convert by a factor of two the set of coded model values fg_comp_model_value[ c ][ sj ][ i ] for i equal to 1..4.

NOTE 10 – To reduce the degree of visible blocks that result from mosaicking the frequency-filtered blocks filteredRv, decoders could apply a low-pass filter to the boundaries between frequency-filtered blocks.

–	Otherwise (fg_model_id is equal to 1), an auto-regression model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1 as specified by:

G[ c ][ x ][ y ] = ( fg_comp_model_value[ c ][ sj ][ 0 ] * n[ c ][ x][ y ] +
		fg_comp_model_value[ c ][ sj ][ 1 ] * ( G[ c ][ x − 1 ][ y ] +
		( ( fg_comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 1 ] )  >>
		fg_log2_scale_factor ) ) + fg_comp_model_value[ c ][ sj ][ 3 ] *
		( ( ( fg_comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x − 1 ][ y − 1 ] )  >>	(28)
		fg_log2_scale_factor ) + G[ c ][ x + 1 ][ y − 1 ] ) +
		fg_comp_model_value[ c ][ sj ][ 5 ] * ( G[ c ][ x − 2 ][ y ] +
		( ( fg_comp_model_value[ c ][ sj ][ 4 ] * fg_comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 2 ] )  >>
		( 2 * fg_log2_scale_factor ) ) ) +
		fg_	comp_model_value[ c ][ sj ][ 2 ] * G[ c − 1 ][ x ][ y ] )  >>  fg_log2_scale_factor

where n[ c ][ x ][ y ] is a random value with normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance for each value of c, x, and y) and the value of an element G[ c ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when any of the following conditions are true:

–	c is less than 0,

–	x is less than 0,

–	y is less than 0.

fg_comp_model_value[ c ][ i ][ 0 ] provides the first model value for the model as specified by fg_model_id. fg_comp_model_value[ c ][ i ][ 0 ] corresponds to the standard deviation of the Gaussian noise term in the generation functions specified in Equations 23 through 28.

fg_comp_model_value[ c ][ i ][ 1 ] provides the second model value for the model as specified by fg_model_id. When fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 1 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, fg_comp_model_value[ c ][ i ][ 1 ] is inferred as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 8.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 0.

fg_comp_model_value[ c ][ i ][ 1 ] is interpreted as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 1 ] indicates the horizontal high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 1 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y ) and ( x, y − 1 ).

fg_comp_model_value[ c ][ i ][ 2 ] provides the third model value for the model as specified by fg_model_id. When fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 2 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, fg_comp_model_value[ c ][ i ][ 2 ] is inferred as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to fg_comp_model_value[ c ][ i ][ 1 ]

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to 0.

fg_comp_model_value[ c ][ i ][ 2 ] is interpreted as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 2 ] indicates the vertical high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 2 ] indicates the colour correlation between consecutive colour components.

fg_comp_model_value[ c ][ i ][ 3 ] provides the fourth model value for the model as specified by fg_model_id. When fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 3 ] shall be greater than or equal to 0 and less than or equal to fg_comp_model_value[ c ][ i ][ 1 ].

When not present in the film grain characteristics SEI message, fg_comp_model_value[ c ][ i ][ 3 ] is inferred to be equal to 0.

fg_comp_model_value[ c ][ i ][ 3 ] is interpreted as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 3 ] indicates the horizontal low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 3 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y − 1 ) and ( x + 1, y − 1 ).

fg_comp_model_value[ c ][ i ][ 4 ] provides the fifth model value for the model as specified by fg_model_id. When fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 4 ] shall be greater than or equal to 0 and less than or equal to fg_comp_model_value[ c ][ i ][ 2 ].

When not present in the film grain characteristics SEI message, fg_comp_model_value[ c ][ i ][ 4 ] is inferred to be equal to fg_model_id.

fg_comp_model_value[ c ][ i ][ 4 ] is interpreted as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 4 ] indicates the vertical low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 4 ] indicates the aspect ratio of the modelled grain.

fg_comp_model_value[ c ][ i ][ 5 ] provides the sixth model value for the model as specified by fg_model_id.

When not present in the film grain characteristics SEI message, fg_comp_model_value[ c ][ i ][ 5 ] is inferred to be equal to 0.

fg_comp_model_value[ c ][ i ][ 5 ] is interpreted as follows:

–	If fg_model_id is equal to 0, fg_comp_model_value[ c ][ i ][ 5 ] indicates the colour correlation between consecutive colour components.

–	Otherwise (fg_model_id is equal to 1), fg_comp_model_value[ c ][ i ][ 5 ] indicates the second order spatial correlation for neighbouring samples ( x, y − 2 ) and ( x − 2, y ).

fg_characteristics_persistence_flag specifies the persistence of the film grain characteristics SEI message for the current layer.

fg_characteristics_persistence_flag equal to 0 specifies that the film grain characteristics SEI message applies to the current decoded picture only.

fg_characteristics_persistence_flag equal to 1 specifies that the film grain characteristics SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a film grain characteristics SEI message is output that follows the current picture in output order.

### Bit-accurate Grain blending process

The bit-accurate grain blending process and constraints are specified such that all decoders that conform to this version of this Specification will produce numerically identical cropped decoded output pictures.

Bitstreams conforming to the bit-accurate grain generation process shall obey the following constraints:

–	ChromaFormatIdc shall equal 1

–	FGC SEI message shall have fg_model_id equal to 0.

–	FGC SEI message shall have fg_separate_colour_description_present_flag equal to 0.

–	FGC SEI message shall have fg_blending_mode_id equal to 0.

–	FGC SEI message shall have fg_log2_scale_factor in the range 2 to 7, inclusive.

–	FGC SEI message shall have fg_intensity_interval_upper_bound[ c ][ i ] less than fg_intensity_interval_lower_bound[ c ][ i + 1 ].

–	FGC SEI message shall have fg_num_model_values_minus1[ c ] in the range 0..2, inclusive.

–	FGC SEI message shall have fg_comp_model_value[ c ][ i ][ 0 ] in the range 0..( 255  <<  ( fgBitDepth[ c ] – 8) ), inclusive.

–	FGC SEI message shall have fg_comp_model_value[ c ][ i ][ 1 ] in the range 2..14, inclusive.

–	FGC SEI message shall have fg_comp_model_value[ c ][ i ][ 2 ] in the range 2..14, inclusive.

Inputs to the process are the decoded picture sample arrays before grain blending decPictureL, decPictureCb, and decPictureCr.

Outputs of this process are modified decoded picture sample array after grain blending blendPictureL, blendPictureCb, and blendPictureCr.

The variable BlockSize is derived as follows:

BlockSize = Min( 1  <<  ( 3 + Ceil( Log2 (Max ( Ceil( PicHeightInLumaSamples ÷ 1080 ),
	 Ceil( PicWidthInLumaSamples ÷ 1920 ) ) ) ) ), 32 )

The grain blending process is derived as the following ordered steps:

–	The grain pattern database generation is invoked in clause 1.1.3.1 with a 13x13x64x64 array GrainDb as output.

–	The grain blending process as specified in clause 1.1.3.2 is invoked with decPictureL, decPictureCb, and decPictureCr. as input, and blendPictureL, blendPictureCb, and blendPictureCr as output.

#### Grain pattern database generation

Output of this process is a 13x13x64x64 grain pattern database array GrainDb.

The function Prng( x ), with x = 0.. 232 − 1, is defined as follows:

Prng( x ) = ( x  <<  1 ) + ( ( 1 + ( ( x & ( 1  <<  2 ) ) > 0 ) + ( ( x & ( 1  <<  30 ) ) > 0 ) ) % 2 )

The pseudo-random number generator array SeedLUT[ i ] with i = 0..255 is specified as follows:

SeedLUT[ i ] =

{

747538460 1088979410 1744950180 1767011913 1403382928  521866116 1060417601 2110622736

1557184770  105289385  585624216 1827676546 1191843873 1018104344 1123590530  663361569

2023850500   76561770 1226763489   80325252 1992581442  502705249  740409860  516219202

557974537 1883843076  720112066 1640137737 1820967556   40667586  155354121 1820967557

1115949072 1631803309   98284748  287433856 2119719977  988742797 1827432592  579378475

1017745956 1309377032 1316535465 2074315269 1923385360  209722667 1546228260  168102420

135274561  355958469  248291472 2127839491  146920100  585982612 1611702337  696506029

1386498192 1258072451 1212240548 1043171860 1217404993 1090770605 1386498193  169093201

541098240 1468005469  456510673 1578687785 1838217424 2010752065 2089828354 1362717428

970073673  854129835  714793201 1266069081 1047060864 1991471829 1098097741  913883585

1669598224 1337918685 1219264706 1799741108 1834116681  683417731 1120274457 1073098457

1648396544  176642749   31171789  718317889 1266977808 1400892508  549749008 1808010512

67112961 1005669825  903663673 1771104465 1277749632 1229754427  950632997 1979371465

2074373264  305357524 1049387408 1171033360 1686114305 2147468765 1941195985  117709841

809550080  991480851 1816248997 1561503561  329575568  780651196 1659144592 1910793616

604016641 1665084765 1530186961 1870928913  809550081 2079346113   71307521  876663040

1073807360  832356664 1573927377  204073344 2026918147 1702476788 2043881033   57949587

2001393952 1197426649 1186508931  332056865  950043140  890043474  349099312  148914948

236204097 2022643605 1441981517  498130129 1443421481  924216797 1817491777 1913146664

1411989632  929068432  495735097 1684636033 1284520017  432816184 1344884865  210843729

676364544  234449232   12112337 1350619139 1753272996 2037118872 1408560528  533334916

1043640385  357326099  201376421  110375493  541106497  416159637  242512193  777294080

1614872576 1535546636  870600145  910810409 1821440209 1605432464 1145147393  951695441

1758494976 1506656568 1557150160  608221521 1073840384  217672017  684818688 1750138880

16777217  677990609  953274371 1770050213 1359128393 1797602707 1984616737 1865815816

2120835200 2051677060 1772234061 1579794881 1652821009 1742099468 1887260865   46468113

1011925248 1134107920  881643832 1354774993  472508800 1892499769 1752793472 1962502272

687898625  883538000 1354355153 1761673473  944820481 2020102353   22020353  961597696

1342242816  964808962 1355809701   17016649 1386540177  647682692 1849012289  751668241

1557184768  127374604 1927564752 1045744913 1614921984   43588881 1016185088 1544617984

1090519041  136122424  215038417 1563027841 2026918145 1688778833  701530369 1372639488

1342242817 2036945104  953274369 1750192384   16842753  964808960 1359020032 1358954497

},

The Gaussian pseudo-random array gaussianLUT[m], with m=0..2047 is specified as follows:

gaussianLUT[ m ] =

{

-11   12  103  -11   42  -35   12   59   77   98  -87    3   65  -78   45   56

-51   21   13  -11  -20  -19   33 -127   17   -6 -105   18   19   71   48  -10

-38   42   -2   75  -67   52  -90   33  -47   21   -3  -56   49    1  -57  -42

-1  120 -127 -108  -49    9   14  127  122  109   52  127    2    7  114   19

30   12   77  112   82  -61 -127  111  -52  -29    2  -49  -24   58  -29  -73

12  112   67   79   -3 -114  -87   -6   -5   40   58  -81   49  -27  -31  -34

-105   50   16  -24  -35  -14  -15 -127  -55  -22  -55 -127 -112    5  -26  -72

127  127   -2   41   87  -65  -16   55   19   91  -81  -65  -64   35   -7  -54

99   -7   88  125  -26   91    0   63   60  -14  -23  113  -33  116   14   26

51  -16  107   -8   53   38  -34   17   -7    4  -91    6   63   63  -15   39

-36   19   55   17  -51   40   33  -37  126  -39 -118   17  -30    0   19   98

60  101  -12  -73  -17  -52   98    3    3   60   33   -3   -2   10  -42 -106

-38   14  127   16 -127  -31  -86  -39  -56   46  -41   75   23  -19  -22  -70

74  -54   -2   32  -45   17  -92   59  -64  -67   56 -102  -29  -87  -34  -92

68    5  -74  -61   93  -43   14  -26  -38 -126  -17   16 -127   64   34   31

93   17  -51  -59   71   77   81  127  127   61   33 -106  -93    0    0   75

-69   71  127  -19 -111   30   23   15    2   39   92    5   42    2   -6   38

15  114  -30  -37   50   44  106   27  119    7  -80   25  -68  -21   92  -11

-1   18   41  -50   79 -127  -43  127   18   11  -21   32  -52   27  -88  -90

-39  -19  -10   24 -118   72  -24  -44    2   12   86 -107   39  -33 -127   47

51  -24  -22   46    0   15  -35  -69   -2  -74   24   -6    0   29   -3   45

32  -32  117  -45   79  -24  -17 -109  -10  -70   88  -48   24  -91  120  -37

50 -127   58   32  -82  -10  -17   -7   46 -127  -15   89  127   17   98  -39

-33   37   42  -40  -32  -21  105  -19   19   19  -59   -9   30    0 -127   34

127  -84   75   24  -40  -49 -127 -107  -14   45  -75    1   30  -20   41  -68

-40   12  127   -3    5   20  -73  -59 -127   -3   -3  -53   -6 -119   93  120

-80  -50    0   20  -46   67   78  -12  -22 -127   36  -41   56  119   -5 -116

-22   68  -14  -90   24  -82  -44 -127  107  -25  -37   40   -7   -7  -82    5

-87   44  -34    9 -127   39   70   49  -63   74  -49  109  -27  -89  -47  -39

44   49   -4   60  -42   80    9 -127   -9  -56  -49  125  -66   47   36  117

15  -11  -96  109   94  -17  -56   70    8  -14   -5   50   37  -45  120  -30

-76   40  -46    6    3   69   17  -78    1  -79    6  127   43   26  127 -127

28  -55  -26   55  112   48  107   -1  -77   -1   53   -9  -22  -43  123  108

127  102   68   46    5    1  123  -13  -55  -34  -49   89   65 -105   -5   94

-53   62   45   30   46   18  -35   15   41   47  -98  -24   94  -75  127 -114

127  -68    1  -17   51  -95   47   12   34  -45  -75   89 -107   -9  -58  -29

-109  -24  127  -61  -13   77  -45   17   19   83  -24    9  127  -66   54    4

26   13  111   43 -113  -22   10  -24   83   67  -14   75 -123   59  127  -12

99  -19   64  -38   54    9    7   61  -56    3  -57  113 -104  -59    3   -9

-47   74   85  -55  -34   12  118   28   93  -72   13  -99  -72  -20   30   72

-94   19  -54   64  -12  -63  -25   65   72  -10  127    0 -127  103  -20  -73

-112 -103   -6   28  -42  -21  -59  -29  -26   19   -4  -51   94  -58  -95  -37

35   20  -69  127  -19 -127  -22 -120  -53   37   74 -127   -1  -12 -119  -53

-28   38   69   17   16 -114   89   62   24   37  -23   49 -101  -32   -9  -95

-53    5   93  -23  -49   -8   51    3  -75  -90  -10  -39  127  -86  -22   20

20  113   75   52  -31   92  -63    7  -12   46   36  101  -43  -17  -53   -7

-38  -76  -31  -21   62   31   62   20 -127   31   64   36  102  -85  -10   77

80   58  -79   -8   35    8   80  -24   -9    3  -17   72  127   83  -87   55

18 -119 -123   36   10  127   56  -55  113   13   26   32  -13  -48   22  -13

5   58   27   24   26  -11  -36   37  -92   78   81    9   51   14   67  -13

0   32   45  -76   32  -39  -22  -49 -127  -27   31   -9   36   14   71   13

57   12  -53  -86   53  -44  -35    2  127   12  -66  -44   46 -115    3   10

56  -35  119  -19  -61   52  -59 -127  -49  -23    4   -5   17  -82   -6  127

25   79   67   64  -25   14  -64  -37 -127  -28   21  -63   66  -53  -41  109

-62   15  -22   13   29  -63   20   27   95  -44  -59 -116  -10   79  -49   22

-43  -16   46  -47 -120  -36  -29  -52  -44   29  127  -13   49   -9 -127   75

-28  -23   88   59   11  -95   81  -59   58   60  -26   40  -92   -3  -22  -58

-45  -59  -22  -53   71  -29   66  -32  -23   14  -17  -66  -24  -28  -62   47

38   17   16  -37  -24  -11    8  -27  -19   59   45  -49  -47   -4  -22  -81

30  -67 -127   74  102    5  -18   98   34  -66   42  -52    7  -59   24  -58

-19  -24 -118  -73   91   15  -16   79  -32  -79 -127  -36   41   77  -83    2

56   22  -75  127  -16  -21   12   31   56 -113 -127   90   55   61   12   55

-14 -113  -14   32   49  -67  -17   91  -10    1   21   69  -70   99  -19 -112

66  -90  -10   -9  -71  127   50  -81  -49   24   61  -61 -111    7  -41  127

88  -66  108 -127   -6   36  -14   41  -50   14   14   73 -101  -28   77  127

-8 -100   88   38  121   88 -125  -60   13  -94 -115   20  -67  -87  -94 -119

44  -28  -30   18    5  -53  -61   20  -43   11  -77  -60   13   29    3    6

-72   38  -60  -11  108  -53   41   66  -12 -127 -127  -49   24   29   46   36

91   34  -33  116  -51  -34  -52   91    7  -83   73  -26 -103   24  -10   76

84    5   68  -80  -13  -17  -32  -48   20   50   26   10   63 -104  -14   37

127  114   97   35    1  -33  -55  127 -124  -33   61   -7  119  -32 -127  -53

-42   63    3   -5  -26   70  -58  -33  -44  -43   34  -56 -127  127   25  -35

-11   16  -81   29  -58   40 -127 -127   20  -47  -11  -36  -63  -52  -32  -82

78  -76  -73    8   27  -72   -9  -74  -85  -86  -57   25   78  -10  -97   35

-65    8  -59   14    1  -42   32  -88  -44   17   -3   -9   59   40   12 -108

-40   24   34   18  -28    2   51 -110   -4  100    1   65   22    0  127   61

45   25  -31    6    9   -7  -48   99   16   44   -2  -40   32  -39  -52   10

-110  -19   56 -127   69   26   51   92   40   61  -52   45  -38   13   85  122

27   66   45 -111  -83   -3   31   37   19  -36   58   71   39  -78  -47   58

-78    8  -62  -36  -14   61   42 -127   71   -4   24  -54   52 -127   67   -4

-42   30  -63   59   -3   -1  -18  -46  -92  -81  -96  -14  -53  -10  -11  -77

13    1    8  -67 -127  127  -28   26  -14   18  -13  -26    2   10  -46  -32

-15   27  -31  -59   59   77 -121   28   40  -54  -62  -31  -21  -37  -32   -6

-127  -25  -60   70 -127  112 -127  127   88   -7  116  110   53   87 -127    3

16   23   74 -106  -51    3   74  -82 -112  -74   65   81   25   53  127  -45

-50 -103  -41  -65  -29   79  -67   64  -33  -30   -8  127    0  -13  -51   67

-14    5  -92   29  -35   -8  -90  -57   -3   36   43   44  -31  -69   -7   36

39  -51   43  -81   58    6  127   12   57   66   46   59  -43  -42   41  -15

-120   24    3  -11   19  -13   51   28    3   55  -48  -12   -1    2   97  -19

29   42   13   43   78  -44   56 -108  -43  -19  127   15  -11  -18  -81   83

-37   77 -109   15   65  -50   43   12   13   27   28   61   57   30   26  106

-18   56   13   97    4   -8  -62 -103   94  108  -44   52   27  -47   -9  105

-53   46   89  103  -33   38  -34   55   51   70  -94  -35  -87 -107  -19  -31

9  -19   79  -14   77    5  -19 -107   85   21  -45  -39  -42    9  -29   74

47  -75   60 -127  120 -112  -57  -32   41    7   79   76   66   57   41  -25

31   37  -47  -36   43  -73  -37   63  127  -69  -52   90  -33  -61   60  -55

44   15    4  -67   13  -92   64   29  -39   -3   83   -2  -38  -85  -86   58

35  -69  -61   29  -37  -95  -78    4   30   -4  -32  -80  -22   -9  -77   46

7  -93  -71   65    9  -50  127  -70   26  -12  -39 -114   63 -127 -100    4

-32  111   22  -60   65 -101   26  -42   21  -59  -27  -74    2  -94    6  126

5   76  -88   -9  -43 -101  127    1  125   92  -63   52   56    4   81 -127

127   80  127  -29   30  116  -74  -17  -57  105   48   45   25  -72   48  -38

-108   31  -34    4  -11   41 -127   52 -104  -43  -37   52    2   47   87   -9

77   27  -41  -25   90   86  -56   75   10   33   78   58  127  127   -7  -73

49  -33 -106  -35   38   57   53  -17   -4   83   52 -108   54 -125   28   23

56  -43  -88  -17   -6   47   23   -9    0  -13  111   75   27  -52  -38  -34

39   30   66   39   38  -64   38    3   21  -32  -51  -28   54  -38  -87   20

52  115   18  -81  -70    0  -14  -46  -46   -3  125   16  -14   23  -82  -84

-69  -20  -65 -127    9   81  -49   61    7  -36  -45  -42   57  -26   47   20

-85   46  -13   41  -37  -75  -60   86  -78 -127   12   50    2   -3   13   47

5   19  -78  -55  -27   65  -71   12 -108   20  -16   11  -31   63  -55   37

75  -17  127  -73  -33  -28 -120  105   68  106 -103 -106   71   61    2   23

-3   33   -5  -15  -67  -15  -23  -54   15  -63   76   58 -110    1   83  -27

22   75  -39  -17  -11   64  -17 -127  -54  -66   31   96  116    3 -114   -7

-108  -63   97    9   50    8   75  -28   72  112  -36 -112   95  -50   23  -13

-19   55   21   23   92   91   22  -49   16  -75   23    9  -49  -97  -37   49

-36   36 -127  -86   43  127  -24  -24   84   83  -35  -34  -12  109  102  -38

51  -68   34   19  -22   49  -32  127   40   24  -93   -4   -3  105    3  -58

-18    8  127  -18  125   68   69  -62   30  -36   54  -57  -24   17   43  -36

-27  -57  -67  -21  -10  -49   68   12   65    4   48   55  127  -75   44   89

-66  -13  -78  -82  -91   22   30   33  -40  -87  -34   96  -91   39   10  -64

-3  -12  127  -50  -37  -56   23  -35  -36  -54   90  -91    2   50   77   -6

-127   16   46   -5  -73    0  -56  -18  -72   28   93   60   49   20   18  111

-111   32  -83   47   47  -10   35  -88   43   57  -98  127  -17    0    1  -39

-127   -2    0   63   93    0   36  -66  -61  -19   39 -127   58   50  -17  127

88  -43 -108  -51  -16    7  -36   68   46  -14  107   40   57    7   19    8

3   88  -90  -92  -18  -21  -24   13    7   -4  -78  -91   -4    8  -35   -5

19    2 -111    4  -66  -81  122  -20  -34  -37  -84  127   68   46   17   47

},

The variable fgCutFreqH[ h ] is set equal to ( ( h + 3 ) << 2) – 1, with h = 0..12.

The variable fgCutFreqV[ v ] is set equal to ( ( v + 3 ) << 2) – 1, with v = 0..12.

The variable fgCoeffs[ h ][ v ][ i ][ j ] is initially set equal to 0, with h = 0..12, v = 0..12, i = 0..63, j = 0..63, and is derived as follows:

for(h = 0; h < 13 , h++ ) {
	for(v = 0; v < 13 , v++ ) {
		prngVal = SeedLUT( h + 13 * v ) 
		for( j = 0; j <= fgCutFreqV[ v ] ; j++ ) {
			for( i = 0; i <= fgCutFreqH[ h ] ; i + = 4 ) {
				for( k = 0; k < 4; k++ )
					fgCoeffs[ h ][ v ][ i + k ][ j ] = gaussianLUT[ ( prngVal + k ) % 2048 ]
				prngVal = Prng( prngVal )
			}
		}
		fgCoeffs[ h ][ v ][ 0 ][ 0 ] = 0
	}
}

For given h, and v, with h = 0..12, v = 0..12, the 64x64 array GrainDb[ h ][ v ][ x][ y ] is derived as follows:

- The variables nonZeroW, nonZeroH, nTbW and nTbH are all set to equal to 64.

- Each (vertical) column of scaled transform coefficients fgCoeffs[ h ][ v ][ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list fgCoeffs[ h ][ v ][ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to 0 as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = ( e[ x ][ y ] + 256 )  >>  9	 	(xxx)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in in Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to 0 as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

- The nTbW x nTbH array GrainDb[ h ][ v ][ x ][ y ] of grain database samples with x = 0..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

GrainDb[ h ][ v ][ x ][ y ] = Clip3( -127, 127 , ( r[ x ][ y ] + 256 )  >>  9 )	(xxx)

The grain pattern database GrainDb is further refined as follows:

deblockFactor[ v ] = { 64, 71, 77, 84, 90, 96, 103, 109, 116, 122, 128, 128, 128 }	(xx)
for( j = 0; j < 64 ; j += BlockSize ) {
	for( i = 0; i < 64 ; i++ ) {
		GrainDb[ h ][ v ][ i ][ j ] = ( GrainDb[ h ][ v ][ i ][ j ] * deblockFactor[ v ] )  >>  7	(xx)
		GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1  ] = 
			( GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1 ] * deblockFactor[ v ] )  >>  7
		}
	}
}

#### Grain blending process

Inputs to this process are the decoded picture prior to grain blending, i.e., the arrays decPictureL, decPictureCb and decPictureCr.

Outputs of this process are the modified decoded picture after grain blending, i.e., the array blendPictureL, blendPictureCb and blendPictureCr.

For cIdx = 0..2, the following applies:

- If cIdx is equal to 0, decSamples, PicWidth, PicHeight, PicWidthInBlock and PicHeightInBlock are derived as follows.

  - decSamples corresponds to the decoded picture sample array decPictureL and blendSamples corresponds to the blended sample array blendPictureL

  - PicWidth is set equal to PicWidthInLumaSamples

  - PicHeight is set equal to PicHeightInLumaSamples

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ BlockSize )

- Otherwise (cIdx is not equal to 0), decSamples, blendSamples, PicWidth, PicHeight, PicWidthInBlock and PicHeightInBlock are derived as follows.

  - If cIdx is equal to 1, decSamples corresponds to the decoded picture sample array decPictureCb and blendSamples corresponds to the blended sample array blendPictureCb

  - Otherwise (cIdx is not equal to 1), decSamples corresponds to the decoded picture sample array decPictureCr and blendSamples corresponds to the blended sample array blendPictureCr.

  - PicWidth is set equal to PicWidthInLumaSamples / 2

  - PicHeight is set equal to PicHeightInLumaSamples / 2

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ 2 ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ 2 ÷ BlockSize )

- If fg_comp_model_present_flag[ cIdx ] is equal to 0, blendSamples[ i ][ j ] is set equal to decSamples[ i ][ j ], with i=0.. PicWidth, j=0..PicHeight

- Otherwise (fg_comp_model_present_flag[ cIdx ] is not equal to 0), the following applies:

  - The seed initialization for current picture is invoked in clause 1.1.3.2.1 with cIdx as inputs, and array prngArray as output.

  - The grain blending process is performed on a block basis as follows. For every block with block location ( rx, ry ), where rx = 0..PicWidthInBlock − 1 and ry = 0..PicHeightIn Block − 1, the grain block blending process is invoked as specified in clause 1.1.3.2.2, with the location ( xCurr, yCurr ) set equal to ( rx * BlockSize,  ry * BlockSize ), cIdx assigned to cIdx, ( xIdx, yIdx ) set equal to ( rx, ry ), the array prngArray and decSamples as inputs, and blendSamples as outputs.

##### Seed initialization for current picture

Input to this process is a variable cIdx specifying the colour component.

The output of this process are picture seed array prngArray.

The function MSB16( x ) with x = 0..232 – 1, is defined as follows:

MSB16( x ) = ( ( x & 0xFFFF0000 )  >>  16 )

The function LSB16(x) with x = 0..232 – 1, is defined as follows:

LSB16( x ) = ( x & 0x0000FFFF )

The function BIT0( x ) with x = 0..232 – 1, is defined as follows:

BIT0(x) = ( x & 0x1 )

The variable picOffset is set equal to PicOrderCnt

The variable cOffset is set equal to ( cIdx  ==  0) ? 0 : ( cIdx  == 1 ) ? 85 : 170 )

The variable prngInit is set equal to SeedLUT[ ( picOffset + cOffset) % 256 ]

The array prngArray[ i ][ j ] with i = 0.. PicWidthInBlock – 1, j = 0.. PicHeightInBlock – 1 is derived as follows:

- The variable prngVal is set equal to prngInit

- The variable bIdc is set equal to ( ( BlockSize  ==  8 ) ? 2 : 1 )

- The following applies,

for( j = 0; j < PicHeightInBlock / bIdc ; j ++ ) {
	for( i = 0; i < PicWidthInBlock / bIdc; i ++ ) {
		if (bIdc  = =  2) {
			prngArray[ 2 * i ][ 2 * j ] = prngVal
			prngArray[ 2 * i + 1 ][ 2 * j ] = prngVal
			prngArray[ 2 * i ][ 2 * j + 1 ] = prngVal
			prngArray[ 2 * i + 1 ][ 2 * j + 1] = prngVal
		}else
			prngArray[i][j] = prngVal
		prngVal = Prng(prngVal)
}

##### Grain block blending process

Inputs to this process are:

- a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

- a variable cIdx specifying the colour component of the current block,

- a block index ( xIdx, yIdx ) specifying the block index of the current block,

- a picture seed array prngArray,

- a decoded sample array decSamples.

Outputs to this process are blended sample array blendSamples.

The variable picWidth is set equalt to ( cIdx  = =  0 ) ? PicWidthInLumaSamples : PicWidthInLumaSamples / 2

The variable picHeight is set equal to ( cIdx  = =  0 ) ? PicHeightInLumaSamples : PicHeightInLumaSamples / 2

The variable intensityIntevalIdx for the current block is initialized to be – 1 and is derived as follows:

sumBlock = 0
avgRatio = Log2( BlockSize / 8)
for( i = 0; i < BlockSize ; i++ )
	for( j = 0; j < BlockSize ; j++ )
		sumBlock += decSamples[ Min( picWidth − 1, xCurr + i ) ][ Min( picHeight − 1, yCurr + j ) ]	(xx)
bavg = Clip3( 0, 255, ( sumBlock + ( 1  <<  ( fgBitDepth[ cIdx ] + 2 * avgRatio − 3 ) ) )  >>  ( fgBitDepth[ cIdx ] + 2 * avgRatio − 2 )

for( i = 0; i  <=  fg_num_intensity_intervals_minus1[ cIdx ]; i++ )
	if( bavg  >=  fg_intensity_interval_lower_bound[ cIdx ][ i ]  &&
			bavg  <=  fg_intensity_interval_upper_bound[ cIdx ][ i ] ) {
		intensityIntervalIdx = i		(xx)
		break
}

The derivation of decSamples for the current block is as follows:

- If intensityIntervalIdx is equal to – 1, blendSamples[ xCurr + i ][ yCurr + j ] is set equal to decSamples[ xCurr + i ][ yCurr + j ], for i=0..Min(picWidth – xCurr – 1, BlockSize – 1), j=0.. Min(picHeight – yCurr – 1, BlockSize – 1;

- Otherwise (intensityIntervalIdx is not equal to – 1), the following applies:

  - The variable scaleFactor is set equal to fg_comp_model_value[ cIdx ][ intensityIntervalIdx ][ 0 ] * ( 1 – 2 * BIT0( prngArray[ xIdx ][ yIdx ] )

  - The variable hFreq is derived as follows:

hFreq = fg_comp_model_value[ cIdx ][ intensityIntervalIdx ][ 1 ]

hFeq = (cIdx  = =  0) ? hFreq : Clip3( 2, 14, ( hFreq  <<  1 ) )

hFeq = hFreq – 2

  - The variable vFreq is derived as follows:

vFreq = fg_comp_model_value[ cIdx ][ intensityIntervalIdx][ 2 ]

vFeq = (cIdx  = =  0) ? vFreq : Clip3( 2, 14, ( vFreq  <<  1 ) )

vFeq = vFreq – 2

  - The variable horzDenom = (BlockSize  = =  32)? 36: 52

  - The variable vertDenom = (BlockSize  = =  32)? 40: 56

  - The variable horzOffset is derived as follows:

horzOffset = ( MSB16( prngArray[ xIdx ][ yIdx ] )  %  horzDenom )

horzOffset  & =  0xFFFC

horzOffset  + =  ( BlockSize  = =  8 ) ? ( xIdx & 0x0008 ) : 0

  - The variable vertOffset is derived as follows:

vertOffset = ( LSB16( prngArray[ xIdx ][ yIdx ] )  %  vertDenom )

vertOffset  & =  0xFFF8

vertOffset  + =  ( BlockSize  = =  8 ) ? ( yIdx & 0x0008) : 0

  - grainSamples is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 ):

grainSamples[ xCurr + i ][ yCurr + j ] = 
		( scaleFactor * GrainDb[ hFreq ][ vFreq ][ i + horzOffset ][ j + vertOffset] )  >>  ( fg_log2_scale_factor + 6 )

- if xCurr > 0, the following applies:

for( k = 0; k < BlockSize; k++ ) {
  	l1 = grainSamples[ xCurr – 2 ][ k ]
  	l0 = grainSamples[ xCurr – 1 ][ k ]
  	r0 = grainSample[ xCurr ][ k ]
  	r1 = grainSample[ xCurr + 1 ][ k ]
  grainSamples[ xCurr ][ k ] = ( ( l0 + ( r0  <<  1 ) + r1 )  >>  2 )
  grainSamples[ xCurr – 1 ][ k ] = ( ( l1 + ( l0  <<  1 ) + r0 )  >>  2 )
}

  - blendSamples[ xCurr + i ][ yCurr + j ] is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 )

blendSample[ xCurr + i ][ yCurr + j ] = Clip3( 0, ( 1  <<  fgBitDepth[ cIdx ] ) − 1,		(xx)
 decodedSamples[ xCurr + i ][ yCurr + j ] + grainSamples[ xCurr + i ][ yCurr + j ] )

