| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0095-v4 |
| --- | --- |

| Title: | AHG9: Fixed-point grain blending process for film grain characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen Dolby Laboratories, Inc. Edouard François, Miloš Radosavljević InterDigital Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam Ittiam Systems | Email: | {sean.mccarthy, pyin}@dolby.com {edouard.francois,milos.radosavljevic}@interdigital.com {vijayakumar.gr, kaustubh.patankar}@ittiam.com |
| Source: | Dolby Laboratories, Inc. InterDigital Ittiam Systems |  |  |

_____________________________

# Abstract

This contribution provides an example of SMPTE RDD 5, Film Grain Technology, as a fixed-point process with support for 4K and 8K, and VVC DCT2. SMPTE RDD 5 is referenced in the current the film grain characteristics (FGC) SEI message specified in VSEI (ITU-T H.274 | ISO/IEC 23002-7).

The fixed-point grain blending process is identical to the process specified in SMPTE RDD 5 except for two modifications: 1) use of a transform already specified in VVC instead of the transform specified in SMPTE RDD 5; and 2) increase in grain block size for resolutions greater than 1920x1080.

This contribution proposes the following:

Proposal 1: Adopt either or both of the following:

- Add, as an example, a description of the fixed-point grain blending process to the next version of the FGC SEI message specified in VSEI. (Proposed text is provided as JVET-W0095-spec-text-v3.)

- Begin a new technical report on use of the FGC SEI message using the description of the fixed-point grain blending process provided in this contribution as input to a first draft. (Proposed text is provided as JVET-W0095-process-text-v2)

Proposal 2: Adopt the provided reference software implementation of the fixed-point grain blending process into the next version of VTM to facilitate experimentation.

Version 2 of this contribution includes specification text and some editorial corrections and clarifications. Version 3 of this contribution removes language in previous versions related to conformance and clarifies 3 specific proposals. Version 4 replaces ‘bit-accurate’ with ‘fixed-point’, and, in the specification text, specifies the VVC DCT2 explicitly instead of normatively referencing VVC.

# Problem Statement

As currently specified, the FGC SEI message does not provide an example of a fixed-point process by which an implementation of a grain blending process could be known to produce equivalent output pictures compared to another implementation of a grain blending process.

SMPTE RDD 5 [1] specifies a fixed-point grain blending process and is already referenced in VSEI. However, SMPTE RDD 5 restricts grain block size to 8x8, which is not implementation friendly for resolutions above 1920x1080.

# Proposal

The proposed fixed-point grain blending process addresses the implementation and quality problems associated with fixed grain block size for resolutions greater than 1920x1080.

The proposed fixed-point grain blending process is identical to the process already referenced in the FGC SEI message and specified in SMPTE RDD 5, Film Grain Technology, except for two modifications:

- The transformation process as specified in VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4, with trType inferred to equal 0) is used [2][3] instead of the transform specified in SMPTE RDD 5. The VVC transform is more mature than the RDD 5 transform. The VVC transform may also be already available on decoders and may thus be considered implementation friendly.

- For picture resolutions greater than 1920x1080, the grain block size is increased proportionally from the fixed 8x8-size in RDD 5 to 16x16 for resolutions up to 3840x2160 (4K) and 32x32 for resolutions greater than 4K including 8K. Use of larger block sizes for larger resolutions reduces implementation complexity and makes the grain pattern block size proportionally consistent across 2k, 4k, and 8k resolutions.

A real-time implementation of fixed-point grain blending process on smartphones is described in reference [4].

As is the case for SMPTE RDD5, the fixed-point grain blending process is particularly intended for use when the following conditions apply:

–	ChromaFormatIdc equal 1

–	fg_model_id equal to 0.

–	fg_separate_colour_description_present_flag equal to 0.

–	fg_blending_mode_id equal to 0.

–	fg_log2_scale_factor in the range 2 to 7, inclusive.

–	fg_intensity_interval_upper_bound[ c ][ i ] less than fg_intensity_interval_lower_bound[ c ][ i + 1 ].

–	fg_num_model_values_minus1[ c ] in the range 0..2, inclusive.

–	fg_comp_model_value[ c ][ i ][ 0 ] in the range 0..( 255  <<  ( fgBitDepth[ c ] – 8) ), inclusive.

–	fg_comp_model_value[ c ][ i ][ 1 ] in the range 2..14, inclusive.

–	fg_comp_model_value[ c ][ i ][ 2 ] in the range 2..14, inclusive.

Also as is the case for SMPTE RDD5, the fixed-point grain blending process requires definition of a picture order count, which we propose to denote as PicOrderCnt and set equal to the value of PicOrderCntVal as specified in VVC. The variable PicOrderCnt is used as a seed for the pseudo-random number generator.

Additionally, we propose text to clarify some of the initial paragraphs and notes of the FGC SEI message specification.

Specification text and reference software for the fixed-point grain blending process are available.

# Conclusion

It is proposed to adopt the provided either or both of the following: A) add text to provide an example of a fixed-point grain blending process to the FGC SEI message; B) begin a technical report on use of the FGC SEI message in VSEI (ITU-T H.274 | ISO/IEC 23002-7). It is also proposed to adopt the provided reference software into the next version of VTM.

# References

- “RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Enhancement of film grain parameter estimation for different intensity intervals”, JVET-W0072, Teleconference, 7-16 July, 2021

- Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, “AHG9: Demonstration of AVC FGC SEI in real-time fixed-point grain blending process on smartphone”, JVET-W0096, Teleconference, 7-16 July, 2021.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).