| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0096-v2 |
| --- | --- |

| Title: | AHG9: Demonstration of AVC FGC SEI in real-time bit-accurate grain blending process on smartphone |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayakumar G R, Jay N Shingala, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam Ittiam Systems S. McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen Dolby Laboratories, Inc. | Email: | {vijayakumar.gr, jay.shingala}@ittiam.com {sean.mccarthy, pyin}@dolby.com |
| Source: | Ittiam Systems Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution describes an optimized software implementation of a real-time bit-accurate grain blending process and AVC decode on a smartphone where the grain blending parameters are signalled in the film grain characteristics SEI message in an AVC bitstream. The bit-accurate grain blending process is as described in JVET-W0095. On a CortexA57 based ARM processor [verified on mid-tier smartphone with Qualcomm SDM730 chipset] for 8-bit 1920x1080 YUV 4:2:0 video content, the single-threaded implementation of the grain blending process completes in 4.5 ms/picture for 8x8 grain blocks and 4 ms/picture for 16x16 grain blocks. For 4-thread implementation, the grain blending process completes in 1.2 ms/picture and 1.1 ms/picture for 8x8 and 16x16 grain blocks, respectively.

Version 2 of this contribution provides additional examples, clarifications, and some editorial corrections.

# Problem Statement

Previous contributions have demonstrated the grain blending processes for HEVC, VVC, and AVC [1][2][3]. None, however, have demonstrated bit-accurate grain blending operating in real time on a consumer-grade smartphone.

# Description of bit-accurate grain blending implementation

The real-time grain blending process described here was developed, optimized using ARM Neon SIMD instructions, and tested on ARMv8 processors in single-threaded and multi-threaded operation. The real-time grain blending process is an implementation of the bit-accurate grain blending process described in JVET-W0095 [4] but for AVC instead of VVC. Parameter values for the grain blending process are signalled in the AVC bitstream, parsed by the AVC decoder, and passed to the grain generator. (Note that parameter values may be set in the encoder configuration file manually or determined automatically in a manner similar to that described in JVET-V0093 [5] and JVET-W0072 [6], but that aspect is not part of this contribution.)

Figure 1 represents the block diagram of the blending process on the smart phone

AVC Stream with Grain Blending SEI Message

AVC Software Decoder

Grain blending Software Component

Screenshots of the grain blending application on smartphone are shown in Figure 2.

Figure 2. Screenshots of the grain blending application on smartphone

A video showing the application running on smartphone is available at this link.: [FGS_DEMO_VIDEO].(JVET_W0096_CameraShoot.mp4)

Additionally, lossless AVI files comparing the decoded output “with and without blending” are also available in the shared link.

Figure 3 shows sample images of side by side comparision of output.

Figure 3. Side by Side comparison of without and with blending decoded output

# Performance

The performance of the grain blending process is summarized in Table 1.

Processor Details:

Smartphone with Qualcomm SDM730 chipset

ARMv8 Cortex A57 equivalent quad core clocked at

Min frequency 345MHz

Max frequency 2GHz

Table 1 Performance results for 8x8 and 16x16 block sizes for single- and four-threaded operation

| Performance numbers on CortexA57 – 1080p resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit depth | 8x8 single thread | 8x8 Four thread | 16x16 single thread | 16x16 - Four thread |
| 8 bit | 4.5 ms/ frame | 1.2 ms / frame | 4 ms/ frame | 1.1 ms/frame |

- References

- Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen, “Illustration of the film grain characteristics SEI message in HEVC”, JCTVC-AM0023, Teleconference, 18-24 April, 2020.

- Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen. “AHG 17: Illustration of the film grain characteristics SEI message for VVC”, JVET-R0359, teleconference, 15-24 April 2020

- Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen, “Illustration of the film grain characteristics SEI message in AVC”, JCTVC-AN0022, teleconference, 24 June – 1 July, 2020.

- Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, Edouard François, Miloš Radosavljević, Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, “AHG9: Bit-accurate grain blending process for film grain characteristics SEI message”, JVET-W0095, Teleconference, 7-16 July, 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

- Miloš Radosavljević, Edouard François, “”, JVET-W0072, Teleconference, 7-16 July, 2021

# Patent rights declaration(s)

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).