| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0097-v2 |
| --- | --- |

| Title: | EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, C.-W. Kuo, Xianglin Wang Ru-Ling Liao, Yan Ye, Xinwei Li, Jie Chen Zhipin Deng, Kai Zhang, Li Zhang Na Zhang, Yang Wang Yao-Jen Chang, Han Huang, Vadim Seregin, Chun-Chi Chen, Marta Karczewicz | Tel: Email: | {xiaoyuxiu, cheweikuo, xianglinwang}@kwai.com {ruling.lrl, yan.ye, sid.lxw, jiechen.cj}@alibaba-inc.com {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com {yjchang, hanhuang, vseregin, martak}@qti.qualcomm.com |
| Source: | Kwai Inc., Alibaba group, Bytedance Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, one enhanced GPM is proposed based on the combination of EE2-3.3 on GPM with MMVD and EE2-3.4 and EE2-3.5 on GPM with template matching. Two encoder options are provided to trade-off the coding performance and encoding complexity. Compared to ECM-1.0 anchors, simulation results of the proposed combination are summarized as follows:

|  | Encoder option #1 |  |  |  |  | Encoder option #2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| RA | -0.44% | -0.75% | -0.90% | 115% | 99% | -0.40% | -0.75% | -0.80% | 109% | 99% |
| LD | -0.58% | -0.96% | -1.06% | 119% | 98% | -0.59% | -1.22% | -0.89% | 113% | 99% |

# Introduction

At the JVET-V meeting, several methods [1][2][3][4] were proposed to improve the coding efficiency of the GPM mode in the VVC. The methods were included in the exploration experiment (EE2) for further evaluations, the main technical aspects of which are described as follows:

EE2-3.3 on GPM with MMVD (GPM-MMVD): 1) additional MVDs are added to the existing GPM merge candidates; 2) the MVDs are signaled in the same manner as the MMVD in the VVC, i.e., one distance index plus one direction index; 3) two flags are signaled to separately control whether the MMVD is applied to each GPM partition or not.

EE2-3.4-3.5 on GPM with template matching (GPM-TM): 1) template matching is extended to the GPM mode by refining the GPM MVs based on the left and above neighboring samples of the current CU; 2) the template samples are selected dependent on the GPM split direction; 3) one single flag is signaled to jointly control whether the template matching is applied to the MVs of two GPM partitions or not.

# Proposal

This contribution proposes a combination of EE2-3.3, EE2-3.4 and EE2-3.5 to further improve the coding efficiency of the GPM mode. Specifically, in the proposed combination, the existing designs in EE2-3.3, EE2-3.4 and EE2-3.5 are kept unchanged while the following modifications are further applied for the harmonization of the two coding tools:

- The GPM-MMVD and GPM-TM are exclusively enabled to one GPM CU. This is done by firstly signaling the GPM-MMVD syntax. When both two GPM-MMVD control flags are equal to false (i.e., the GPM-MMVD are disabled for two GPM partitions), the GPM-TM flag is signaled to indicate whether the template matching is applied to the two GPM partitions. Otherwise (at least one GPM-MMVD flag is equal to true), the value of the GPM-TM flag is inferred to be false.

- The GPM merge candidate list generation methods in EE2-3.3 and EE2-3.4-3.5 are directly combined in a manner that the MV pruning scheme in EE2-3.4-3.5 (where the MV pruning threshold is adapted based on the current CU size) is applied to replace the default MV pruning scheme applied in EE2-3.3; additionally, as in EE2-3.4-3.5, multiple zero MVs are added until the GPM candidate list is fully filled.

# Simulation results

The proposed scheme was implemented on top of the ECM-1.0 [6] and tested using the common test conditions as defined in [7]. Table 1 and Table 2 illustrates the overall BD-rate performance of the proposed combination, where two encoder speed-up options are provided for different trade-offs between coding performance and encoder run-time.

Table 1 Performance of the combination of EE2-3.3, EE2-3.4 and EE2-3.5, Encoder option #1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.54% | -0.70% | 110% | 100% |
| Class A2 | -0.50% | -0.71% | -0.72% | 112% | 99% |
| Class B | -0.36% | -0.67% | -0.81% | 117% | 100% |
| Class C | -0.61% | -1.03% | -1.28% | 119% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.75% | -0.90% | 115% | 99% |
| Class D | -0.54% | -0.85% | -0.74% | 117% | 99% |
| Class F | -0.23% | -0.47% | -0.46% | 112% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.54% | -0.71% | 117% | 99% |
| Class C | -0.74% | -1.67% | -1.46% | 125% | 97% |
| Class E | -0.81% | -0.71% | -1.12% | 115% | 99% |
| Overall | -0.58% | -0.96% | -1.06% | 119% | 98% |
| Class D | -0.62% | -1.07% | -1.43% | 122% | 97% |
| Class F | -0.37% | -0.64% | -0.90% | 114% | 99% |

# Table 2 Performance of the combination of EE2-3.3, EE2-3.4 and EE2-3.5, Encoder option #2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.56% | -0.69% | 106% | 99% |
| Class A2 | -0.45% | -0.62% | -0.67% | 107% | 99% |
| Class B | -0.33% | -0.69% | -0.69% | 111% | 99% |
| Class C | -0.56% | -1.09% | -1.11% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.75% | -0.80% | 109% | 99% |
| Class D | -0.46% | -0.92% | -1.00% | 109% | 98% |
| Class F | -0.18% | -0.45% | -0.38% | 107% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.83% | -0.83% | 111% | 98% |
| Class C | -0.71% | -1.57% | -1.26% | 116% | 99% |
| Class E | -0.84% | -1.42% | -0.52% | 114% | 99% |
| Overall | -0.59% | -1.22% | -0.89% | 113% | 99% |
| Class D | -0.54% | -0.54% | -1.25% | 114% | 98% |
| Class F | -0.36% | -0.22% | -0.84% | 107% | 99% |

# References

- Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang, “AHG12: Geometric prediction mode with motion vector differences,” JVET-V0103, Apr. 2021.

- X. Xiu, H.-J. Jhu, C.-W. Kuo, W. Chen, Y.-W. Chen, X. Wang, “AHG12: Evaluation of GPM with MMVD for coding efficiency improvement over VVC”, JVET-V0125, Apr. 2021.

- R.-L. Liao, Y. Ye, X. Li, J. Chen, “EE2-related: Combination of GPM and template matching”, JVET-V0117, Apr. 2021.

- C.-C. Chen, V. Seregin, Y.-J. Chang, Z. Zhang, H. Huang, Y. Zhang, M. Karczewicz, “EE2-related: Extension of template matching to Affine, CIIP, GPM merge modes, and boundary sub-blocks”, JVET-V0118, Apr. 2021.

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-V2024, Apr. 2021.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).