| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0098-v1 |
| --- | --- |

| Title: | Non-EE2: Bilateral Inloop Filter on Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)999-7093 yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution extends Bilateral Inloop Filter (BIF) in ECM to chroma components. The BIF-chroma keeps the same design as BIF-luma in ECM. A diamond 5×5 filtering kernel and a CTU level on/off control mechanism are used. The filtering coefficients are retrained for chroma components and the filtering strength for chroma is decided based on the chroma block size as well as the corresponding luma block size.

On top of the EE code of ECM-1.0, simulation results of the proposed method are reported as below:

Tool-on test (VTM-11 with improved MCTF as the anchor):

AI: 0.02%, -0.82%, -0.89%, 101%, 104%;

RA: 0.03%, -0.65%, -0.49%, 101%, 102%;

LB: 0.03%, -1.07%, -1.45%, 100%, 101%.

Tool-off test (ECM-1.0 as the anchor):

AI: 0.00%, -0.77%, -1.01%, 101%, 101%;

RA:  %,  %,  %,  %,  %;

LB: -0.06%, -1.19%%, -1.30%, 99%, 99%.

Introduction

In ECM-1.0, a bilateral filter [1] is adopted to reduce artifacts for the luma component. It is carried out in parallel with the sample adaptive offset (SAO) loop-filter stage. Both BIF-luma and SAO use samples from deblocking filter as input and generates two offsets per luma sample in parallel. The two offsets are added to the input luma sample to generate an output luma sample, which is further fed to the adaptive loop filter (ALF) process. It should be noted that BIF is only performed on luma samples in ECM-1.0, without considering artifacts of chroma samples.

Proposed schemes

In this contribution, it is proposed to extend BIF to chroma components.

Same as BIF-luma, the proposed BIF-chroma is also performed in parallel with the SAO process as shown in Fig.1. The BIF-chroma and SAO use the same chroma samples that produced by deblocking filter as input and generate two offsets per chroma sample in parallel. Then these two offsets are both added to the input chroma sample to obtain a sum, which is then clipped to form the final output chroma sample value. The proposed BIF-chroma provides an on/off control mechanism on CTU level and slice level.

Fig. 1 Filtering stage of BIF-chroma

The filtering process of BIF-chroma is also similar to that of BIF-luma. For a chroma sample, a 5×5 diamond shape filter is used for generating the filtering offset. The difference between the central sample and each sourrding sample is calculated first. The coefficient for each reference sampel is extracted from a pre-defined look-up-table based on the calculated difference direclty. The coefficients used for chroma components are retrained, different from those for the luma component. In the BIF-luma design, the block-level filtering strength parameter c is determined based on luma TU size and CU mode. While in the BIF-chroma design, the parameter for chroma components is determined based the chroma TU size and mode when dual-tree partitioing is enabled for current slice, and based on corresponding luma TU size and mode when dual-tree partitioing is disabled.

Experimental results

The following tables show the overall results for AI, RA and LDB. The simulations were performed following the the common test conditions [2]. The tests were implemented on top of ECM-1.0 [3]. Compared to the VTM-11.0 with improved MCTF, the performance of the proposed filter is summarized in Table 1 (tool-on test). On top of the ECM-1.0, the performance of the proposed filter is summarized in Table 2 (tool-off test).

Table 1. Performance of the proposed filter on top of VTM 11.0 (tool-on test).

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.74% | -1.18% | 100% | 104% | 0.04% | -0.61% | -0.62% | 101% | 103% |  |  |  |  |  |
| Class A2 | 0.11% | -0.76% | -0.75% | 101% | 105% | 0.06% | -0.50% | -0.46% | 101% | 102% |  |  |  |  |  |
| Class B | 0.02% | -0.72% | -0.76% | 101% | 105% | 0.01% | -0.58% | -0.34% | 100% | 102% | 0.02% | -0.98% | -1.12% | 100% | 100% |
| Class C | -0.01% | -1.06% | -0.89% | 101% | 103% | 0.04% | -0.86% | -0.61% | 100% | 100% | 0.01% | -1.51% | -1.66% | 100% | 103% |
| Class E | 0.02% | -0.81% | -0.95% | 100% | 102% |  |  |  |  |  | 0.08% | -0.65% | -1.73% | 100% | 102% |
| Overall | 0.02% | -0.82% | -0.89% | 101% | 104% | 0.03% | -0.65% | -0.49% | 101% | 102% | 0.03% | -1.07% | -1.45% | 100% | 101% |
| Class D | -0.04% | -1.16% | -0.98% | 104% | 107% | 0.03% | -0.84% | -0.97% | 102% | 101% | 0.02% | -1.32% | -0.86% | 100% | 95% |

Table 2. Performance of the proposed filter on top of ECM 1.0 (tool-off test).

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.52% | -1.48% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.07% | -0.87% | -0.81% | 101% | 101% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.84% | -0.92% | 101% | 102% | -0.01% | -0.60% | -0.49% | 100% | 100% | 0.00% | -0.70% | -0.45% | 100% | 100% |
| Class C | -0.03% | -0.93% | -0.90% | 100% | 101% | -0.05% | -0.63% | -0.35% | 99% | 100% | -0.04% | -1.73% | -1.31% | 99% | 99% |
| Class E | 0.02% | -0.61% | -1.07% | 100% | 102% |  |  |  |  |  | -0.16% | -1.27% | -2.71% | 98% | 98% |
| Overall | 0.00% | -0.77% | -1.01% | 101% | 101% |  |  |  |  |  | -0.06% | -1.19% | -1.30% | 99% | 99% |
| Class D | -0.08% | -0.97% | -0.59% | 100% | 97% | 0.01% | -0.94% | -0.60% | 99% | 99% | 0.08% | -1.57% | -2.06% | 98% | 99% |

Conclusion

In this contribution, it is proposed to extend BIF to chroma components. It is asserted that the proposed filter achieves gains in terms of coding efficiency on chroma components and has almost no impact on encoder and decoder complexity. It is recommended to adopt it into ECM.

- References

[1] J. Ström, P. Wennersten, K. Andersson, R. Sjöberg, S. Ikonin and E. Alshina “EE2: Bilateral filter in VTM, EE2 and VVenC”, JVET-V0094, Apr 2021.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[3] ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).