| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0100-v2 |
| --- | --- |

| Title: | AHG11: Deep In-Loop Filter with Adaptive Model Selection and External Attention |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a convolutional neural network-based in-loop filtering method wherein adaptive model selection is introduced. The proposed Deep in-loop filter with Adaptive Model selection (DAM) method is developed from the prior contribution JVET-V0100, introducing a new network structure to the code base of VTM-11.0+NewMCTF. Compared with VTM-11.0+NewMCTF, the proposed method reportedly shows on average {9.12%, 22.39%, 22.60%}, {12.32%, 27.48%, and 27.22%}, and {10.61%, 28.19%, and 27.91%} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

# Introduction

In this contribution, convolutional neural networks (CNN) are utilized to construct in-loop filters for reducing the distortion incurred during compression. The network structure is revised from the one presented in our prior contribution JVET-V0100 [1]. Similar to JVET-V0100, residual blocks are utilized as the basic module and stacked several times to construct the final network. As a further improvement from JVET-V0100, external attention mechanism is introduced in this contribution, leading to an increased representation capability with a similar model size. In addition, to deal with different types of content, individual networks are trained for different types of slices and quality levels.

# Proposed method

In section 2.1, the new architecture equipped with an external attention mechanism is presented. Model selection, which is the same as our prior contribution JVET-V0100, is introduced in section 2.2.

## Architecture

The backbone of the proposed CNN filter is shown in Fig. 1. In addition to the attention, other parts are the same as those in JVET-V0100. The calculation process in the attention module can be written as

F_out = F_in × f (Rec, Pred) + F_in

where F_in and F_out denote the input and the output of the attention module, respectively. Rec and Pred stands for the reconstruction and the prediction respectively. f comprises 2 convolutional layers, where an activation function is applied after the first convolutional layer. The objective of f is to generate a spatial attention map from external information, which then recalibrates the feature maps F_in.

Fig. 1. (a) Architecture of the proposed CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a)

## Model Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which model from a candidate model list including three models could be further decided. To this end, individual models are trained with QPs in {17, 22, 27, 32, 37, 42}. Given a test QP for a sequence, denoted as q, the candidate model list includes models trained with QPs equal to {q, q-5, q-10}. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the model index, if needed, are signalled in the bitstream. Other details regarding the model selection are provided as below.

- The granularity of the filter determination and the model selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

- The candidate model list is different across different temporal layers. For a low temporal layer, the third candidate is replaced by the intra-slice NN filter model if the ratio of intra-coded samples is no smaller than a threshold. For a high temporal layer, the third candidate, i.e. the model corresponding to q-10, is replaced by the model corresponding to q+5.

- For all-intra configuration, model selection is disabled while on/off control is still preserved.

## Inference

PyTorch is used for performing the inference of the proposed CNN filters in VTM. The network information in the inference stage is provided in Table 1 as suggested by [2].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 5.53M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~22.1M/model，24 models in total |
|  | MAC (Giga) | 1.43M/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 69+64 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 32×32, 64×64, 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training

PyTorch is used as the training platform. The DIV2K [3] and BVI-DVC [4] datasets are adopted to train the CNN filters of I slices and B slices, respectively. Different CNN models are trained to adapt to different QP points. The network information in the training stage is provided in Table 2 as suggested by [2].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 + new MCTF [5] according to the common test conditions defined in [2], where the new MCTF patch is from [6]. Deblocking filtering and SAO are disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3 ~ Table 5. Under AI configurations, the proposed method can bring 9.12%, 22.39%, and 22.60% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 12.32%, 27.48%, and 27.22% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring 10.61%, 28.19%, and 27.91% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -14.27% | -11.23% | -21.85% | -22.49% | 316% | 106894% |
| Class A2 | -15.97% | -12.91% | -26.09% | -26.30% | 300% | 100508% |
| Class B | -15.87% | -11.87% | -31.12% | -28.84% | 308% | 100969% |
| Class C | -16.51% | -13.27% | -28.19% | -29.42% | 237% | 93822% |
| Class E |  |  |  |  |  |  |
| Overall | -15.74% | -12.32% | -27.48% | -27.22% | 287% | 100056% |
| Class D | -18.11% | -14.95% | -29.89% | -30.63% | 223% | 87872% |
| Class F | -8.69% | -6.31% | -17.37% | -17.62% | 436% | 39043% |

Table 4. Performance of the proposed method on top of VTM-11.0 + new MCTF (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -13.44% | -9.72% | -29.50% | -29.04% | 298% | 100552% |
| Class C | -15.61% | -12.20% | -29.61% | -30.35% | 241% | 99462% |
| Class E | -12.82% | -9.96% | -24.12% | -22.77% | 611% | 80736% |
| Overall | -14.01% | -10.61% | -28.19% | -27.91% | 333% | 94838% |
| Class D | -17.06% | -13.52% | -32.93% | -33.23% | 227% | 90645% |
| Class F | -9.31% | -6.87% | -19.80% | -18.48% | 442% | 47851% |

Table 5. Performance of the proposed method on top of VTM-11.0 + new MCTF (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -10.74% | -7.78% | -18.09% | -17.37% | 253% | 71308% |
| Class A2 | -11.27% | -8.14% | -22.53% | -22.32% | 182% | 58999% |
| Class B | -10.97% | -8.21% | -23.36% | -22.65% | 167% | 55609% |
| Class C | -12.33% | -9.71% | -21.56% | -24.03% | 134% | 37708% |
| Class E | -15.15% | -12.18% | -26.02% | -26.11% | 179% | 60865% |
| Overall | -11.98% | -9.12% | -22.39% | -22.60% | 175% | 54509% |
| Class D | -12.03% | -9.36% | -22.81% | -24.31% | 125% | 32717% |
| Class F | -6.35% | -4.82% | -13.56% | -11.69% | 132% | 21437% |

In addition, Table 6 and Table 7 provide the coding efficiency of a simplified version of the proposed method, where the number of residual blocks is reduced to 8 for the inter slice models.

Table 6. Performance of the simplified models on top of VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.94% | -9.82% | -20.99% | -21.30% | 183% | 42883% |
| Class A2 | -14.52% | -11.41% | -25.07% | -24.58% | 175% | 39914% |
| Class B | -14.48% | -10.41% | -30.32% | -27.68% | 178% | 41102% |
| Class C | -14.93% | -11.61% | -27.11% | -27.95% | 147% | 34987% |
| Class E |  |  |  |  |  |  |
| Overall | -14.30% | -10.81% | -26.55% | -25.85% | 170% | 39477% |
| Class D | -16.40% | -13.25% | -28.36% | -28.82% | 140% | 32219% |
| Class F | -8.01% | -5.62% | -16.92% | -16.78% | 229% | 23168% |

Table 7. Performance of the simplified models on top of VTM-11.0 + new MCTF (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -12.41% | -9.01% | -27.64% | -26.67% | 170% | 40321% |
| Class C | -14.08% | -10.92% | -27.61% | -27.68% | 146% | 35830% |
| Class E | -13.31% | -10.61% | -24.18% | -22.75% | 281% | 40248% |
| Overall | -13.19% | -10.05% | -26.77% | -26.03% | 184% | 38747% |
| Class D | -15.33% | -12.14% | -30.25% | -29.53% | 139% | 31649% |
| Class F | -8.38% | -6.31% | -17.63% | -15.89% | 224% | 28342% |

# Conclusions

This contribution presents a CNN-based in-loop filtering method. The proposed CNN-based filtering method shows very promising coding gains.

# References

- AHG11: Deep in-loop filter with adaptive model selection, JVET-V0100, Y. Li, L. Zhang, K. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).