| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0101 |
| --- | --- |

| Title: | AHG11 & AHG12: Deep In-Loop Filter with Adaptive Model Selection and External Attention for Enhanced Compression Beyond VVC Capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution evaluates the deep in-loop filter presented in JVET-W0100 on top of ECM-1.0. Compared with ECM-1.0, the proposed method reportedly shows on average {6.74%, 19.53%, 20.16%}, {%, %, %}, and {%, %, %} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively. Compared with VTM-11.0 + new MCTF, a combination of the NN-based coding methods and the non-NN-based coding methods beyond VVC reportedly shows on average {12.16%, 26.17%, 26.68%}, {%, %, %}, and {%, %, %} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

Introduction

ECM-1.0 [1] is the test model to investigate the non-NN-based compression methods beyond VVC. In this contribution, Deep in-loop filter with adaptive model selection and external attention proposed in JVET-W0100 [2] is incorporated in ECM-1.0, to demonstrate the coding efficiency of a combination of NN-based coding methods and non-NN based coding methods beyond VVC.

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of ECM-1.0, according to the common test conditions defined in [3]. Deblocking filtering and SAO are disabled while BIF, ALF, and CCALF are placed after the proposed CNN-based filtering. Test results of the proposed deep filtering compared to ECM-1.0 are shown in Table 1 ~ Table 3. Compared with ECM-1.0, the proposed method reportedly shows on average {6.74%, 19.53%, 20.16%}, {%, %, %}, and {%, %, %} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively. In addition, Table 4 ~ Table 5 provides the coding efficiency of a combination of the NN-based coding methods and the non-NN-based coding methods beyond VVC compared to VVC.

Table 1. Performance of the proposed method on top of ECM-1.0 (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -12.93% | -9.20% | -27.44% | -25.38% | 175% | 22331% |
| Class C | -12.90% | -10.01% | -23.70% | -24.45% | 145% | 22952% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -14.58% | -11.46% | -26.04% | -27.07% | 139% | 20006% |
| Class F | -6.95% | -4.94% | -14.84% | -14.15% | 239% | 13414% |

Table 2. Performance of the proposed method on top of ECM-1.0 (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -11.63% | -9.19% | -23.50% | -22.27% | 145% | 37049% |
| Class E | -12.48% | -9.60% | -23.81% | -22.86% | 335% | 40522% |
| Overall |  |  |  |  |  |  |
| Class D | -13.21% | -10.54% | -26.09% | -25.27% | 140% | 33736% |
| Class F | -8.33% | -6.38% | -17.65% | -14.96% | 252% | 25041% |

Table 3. Performance of the proposed method on top of ECM-1.0 (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -8.37% | -6.01% | -13.90% | -16.00% | 163% | 31464% |
| Class A2 | -8.54% | -6.09% | -18.20% | -19.80% | 135% | 28110% |
| Class B | -8.70% | -5.90% | -21.96% | -20.31% | 124% | 25173% |
| Class C | -9.30% | -6.81% | -18.42% | -20.28% | 110% | 19374% |
| Class E | -12.50% | -9.42% | -23.90% | -24.26% | 125% | 27798% |
| Overall | -9.39% | -6.74% | -19.53% | -20.16% | 128% | 25526% |
| Class D | -9.43% | -6.87% | -19.24% | -21.41% | 105% | 17599% |
| Class F | -4.63% | -3.24% | -11.59% | -9.02% | 115% | 8866% |

Table 4. Performance of a combination of the proposed method and non-NN-based tools presented in ECM-1.0 compared with VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -22.99% | -19.53% | -36.29% | -34.12% | 500% | 106620% |
| Class C | -24.23% | -21.60% | -33.88% | -34.71% | 420% | 103026% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -26.18% | -23.57% | -35.85% | -36.65% | 418% | 92685% |
| Class F | -18.67% | -16.59% | -26.30% | -26.21% | 609% | 43398% |

Table 5. Performance of a combination of the proposed method and non-NN-based tools presented in ECM-1.0 compared with VTM-11.0 + new MCTF (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -20.91% | -18.78% | -30.57% | -30.75% | 408% | 110210% |
| Class E | -20.41% | -17.68% | -31.59% | -29.25% | 727% | 107339% |
| Overall |  |  |  |  |  |  |
| Class D | -23.83% | -21.66% | -34.22% | -33.59% | 412% | 104910% |
| Class F | -18.27% | -16.59% | -26.16% | -23.82% | 617% | 60011% |

Table 6. Performance of a combination of the proposed method and non-NN-based tools presented in ECM-1.0 compared with VTM-11.0 + new MCTF (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -15.03% | -11.65% | -22.41% | -25.71% | 435% | 74093% |
| Class A2 | -14.02% | -11.36% | -24.95% | -24.79% | 344% | 61296% |
| Class B | -13.88% | -10.77% | -27.87% | -26.51% | 330% | 55797% |
| Class C | -15.26% | -12.62% | -24.89% | -26.80% | 289% | 37764% |
| Class E | -18.31% | -15.17% | -30.01% | -29.63% | 339% | 63723% |
| Overall | -15.14% | -12.16% | -26.17% | -26.68% | 339% | 55703% |
| Class D | -14.13% | -11.59% | -24.03% | -26.14% | 289% | 33188% |
| Class F | -15.15% | -13.50% | -22.70% | -20.87% | 245% | 23515% |

Conclusions

This contribution presents the test results of a combination of NN-based coding methods and non-NN based coding methods beyond VVC. These two kinds of methods show additive coding gains.

References

- https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tree/ECM-1.0.

- AHG11: Deep in-loop filter with adaptive model selection and external attention, JVET-W0100, Y. Li, K. Zhang, L. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).