| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0104-v1 |
| --- | --- |

| Title: | AHG9: Resampling SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier Gaëlle Martin-Cocher Fabrice Le Léannec Karam Naser | Tel: Email: | tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes the inclusion of resampling information in the VSEI specification. The resampling SEI message informs the decoder of the filters recommended to be applied for resampling prior to display.

# Introduction

VVC enables picture resolution change within a sequence at a position without encoding an IRAP picture.

During the development of VVC, the following test pipeline, as shown Figure 1 , has been used for performance evaluation. The first PSNR (PSNR1) in Figure 1 is computed between the resampled source and reconstructed sequence. The second PSNR (PSNR2) is computed between the original resolution source and the resampled version of the reconstructed sequence whose resolution is restored to the original resolution.

Figure 1: test pipeline

In order to downsample the source before encoding, SHM down-sampling 12-tap filter is used, for upscaling the decoded sequence, luma and chroma interpolation filters are used.

We have evaluated two categories of filters, some existing VVC filters and some edge aware filters such as Directional Cubic Convolution Interpolation (DCC)  and Iterative Curvature-based Interpolation (ICBI).

It is known that depending on the content, using other filters may be beneficial, as shown in Figure 2, especially for gaming sequences where sharp edges are prevalent and shorter tap filters give better performances.

Figure 2: VVC  luma (left) and VVC chroma (right) filters used for upsampling the luma component of HeroesOfStorm sequence

The proposed resampling SEI message enables to precisely describe the resampling filters used for both luma and chroma in both horizontal and vertical directions, whether they are existing VVC filters or not

# Description

## Resampling SEI message syntax

| resampling( payloadSize ) { |  |
| --- | --- |
| resampling_id | ue(v) |
| resampling_cancel_flag | u(1) |
| if( !resampling_cancel_flag ) { |  |
| resampling_persistence_flag | u(1) |
| if (pps_pic_width_in_luma_samples != sps_pic_width_max_in_luma_samples ¦¦ pps_pic_height_in_luma_samples !=sps_pic_height_max_in_luma_samples) { |  |
| num_resampling_filters_luma_hor | ue(v) |
| for( i = 0; i < num_resampling_filters_luma_hor; i++ ) { |  |
| resampling_tap_luma_hor_minus1[ i ] | ue(v) |
| for( j = 0; j <= resampling_tap_luma_hor_minus1; j++ ) { |  |
| resampling_luma_hor_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| use_alternative_filter_for_vertical_luma | u(1) |
| if( use_alternative_filter_for_vertical_luma ) { |  |
| num_resampling_filters_luma_ver | ue(v) |
| for( i = 0; i < num_resampling_filters_luma_ver; i++ ) { |  |
| resampling_tap_luma_ver_minus1[ i ] | ue(v) |
| for( j = 0; j <= resampling_tap_luma_ver_minus1; j++ ) { |  |
| resampling_luma_ver_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| use_alternative_filter_for_chroma | u(1) |
| if( use_alternative_filter_for_chroma ) { |  |
| num_resampling_filters_chroma_hor | ue(v) |
| for( i = 0; i < num_resampling_filters_chroma_hor; i++ ) { |  |
| resampling_tap_chroma_hor_minus1[ i ] | ue(v) |
| for( j = 0; j <= resampling_tap_chroma_hor_minus1; j++ ) { |  |
| resampling_chroma_hor_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| use_alternative_filter_for_vertical_chroma | u(1) |
| if( use_alternative_filter_for_vertical_chroma ) { |  |
| num_resampling_filters_chroma_ver | ue(v) |
| for( i = 0; i < num_resampling_filters_chroma_ver; i++ ) { |  |
| resampling_tap_chroma_ver_minus1[ i ] | ue(v) |
| for( i = 0; i <= resampling_tap_chroma_ver_minus1; i++ ) { |  |
| resampling_chroma_ver_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Resampling SEI message semantics

resampling_id contains an identifying number that may be used to identify the purpose of the resampling information. The value of resampling_id shall be in the range of 0 to 232 − 2, inclusive.

resampling_cancel_flag equal to 1 indicates that the resampling information SEI message cancels the persistence of any previous resampling information SEI message in output order that applies to the current layer. resampling_cancel_flag equal to 0 indicates that resampling information follows.

resampling_persistence_flag specifies the persistence of the resampling information SEI message.

resampling_persistence_flag equal to 0 specifies that the resampling information applies to the current decoded picture only.Let picA be the current picture. resampling_persistence_flag equal to 1 specifies that the resampling information persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	A picture picB in the current layer in an access unit containing a resampling information SEI message with the same value of resampling_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

num_resampling_filters_luma_hor specifies the number of filters signaled for luma resampling in the horizontal direction.

resampling_tap_luma_hor_minus1 specifies the size of the filter coefficient array that is applied to RPR frames to be resampled. The value of resampling_tap_luma_hor_minus1 shall be in the range of 1 to 15, inclusive.

resampling_luma_hor_coeff[ i ] specifies a filter coefficient for luma component with 16-bit precision that is applied to RPR frames to be resampled. The value of resampling_luma_hor_coeff[ i ] shall be in the range of −231 + 1 to 231 – 1.

use_alternative_filter_for_vertical_luma specifies if resampling information for vertical filtering for luma component is different from horizontal luma information and is coded.

num_resampling_filters_luma_ver specifies the number of filters signaled for luma resampling in the vertical direction.

resampling_tap_luma_ver_minus1 specifies the size of the filter coefficient array that is applied to RPR frames to be resampled. The value of resampling_tap_luma_ver_minus1 shall be in the range of 1 to 15, inclusive.

resampling_luma_ver_coeff[ i ] specifies a filter coefficient for luma component with 16-bit precision that is applied to RPR frames to be resampled. The value of resampling_luma_ver_coeff[ i ] shall be in the range of −231 + 1 to 231 – 1.

use_alternative_filter_for_chroma specifies if resampling information is coded for chroma.

It is a requirement of bitstream conformance that when use_alternative_filter_for_chroma is equal to 1, chroma_format_idc shall not be equal to 0.

num_resampling_filters_chroma_hor specifies the number of filters signaled for chroma resampling in the horizontal direction.

resampling_tap_chroma_hor_minus1 specifies the size of the filter coefficient array that is applied to RPR frames to be resampled. The value of resampling_tap_chroma_minus1 shall be in the range of 1 to 15, inclusive.

resampling_chroma_hor_coeff[ i ] specifies a filter coefficient for chroma component with 16-bit precision that is applied to RPR frames to be resampled. The value of resampling_chroma_coeff[ i ] shall be in the range of −231 + 1 to 231 – 1.

use_alternative_filter_for_vertical_chroma specifies if resampling information for vertical filtering for chroma component is different from horizontal chroma information and is coded.

resampling_tap_chroma_ver_minus1 specifies the size of the filter coefficient array that is applied to RPR frames to be resampled. The value of resampling_tap_chroma_ver_minus1 shall be in the range of 1 to 15, inclusive.

resampling_chroma_ver_coeff[ i ] specifies a filter coefficient for chroma component with 16-bit precision that is applied to RPR frames to be resampled. The value of resampling_chroma_ver_coeff[ i ] shall be in the range of −231 + 1 to 231 – 1.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).