| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0106-v3 |
| --- | --- |

| Title: | EE2-related: Bilateral matching AMVP-merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Han Huang Chun-Chi Chen Vadim Seregin Marta Karczewicz | Tel: Email: | zhizhang@qti.qualcomm.com hanhuang@qti.qualcomm.com chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, bilateral matching AMVP-merge mode is proposed to combine AMVP and merge inter predictors. The final motion vector of the merge inter predictor is selected from the merge candidate list and is refined based on the minimum bilateral matching error between a reference block that is generated by AMVP candidate and a reference block that is generated by the merge candidate. The proposed method was implemented on top of ECM-1.0, and it reports the BD-rate -0.11% (Y), -0.05% (U), -0.12% (V) for random access configuration with 106% encoding and 99% decoding run time. In addition, it reports the BD-rate -0.48% (Y), -0.44% (U), -0.38% (V) with template matching disabled in both the ECM-1.0 anchor and the test for random access configuration with 110% encoding and 100% decoding run time.

# Introduction

In ECM-1.0 software, bilateral matching (BM) motion vector (MV) refinement is applied to a regular merge mode when it satisfies all enabling conditions. The refined MVs in each prediction direction (L0 and L1) are derived as follows:

- applying BM to the whole coding block.

- applying BM to each 16 × 16 subblock within the coding block.

In this contribution, bilateral matching AMVP-merge mode, which is a combination of AMVP and merge modes, is proposed.

# Proposed method

In the proposed mode, the bi-directional predictor is composed of an AMVP predictor in one direction and a merge predictor in the other direction. The mode can be enabled to a coding block when there is at least one reference picture from the past and one reference picture from the future relatively to the current picture.

AMVP part of the proposed mode is signaled as a regular uni-directional AMVP, i.e. reference index and MVD are signaled, and it has a derived MVP index if template matching is used or MVP index is signaled when template matching is disabled. Let’s denote the AMVP direction as LX, X can be 0 or 1.

The merge part in the other direction (1 – LX) is implicitly derived by minimizing the bilateral matching cost between the AMVP predictor and a merge predictor, i.e. for a pair of the AMVP and a merge motion vectors. For every merge candidate in the merge candidate list which has that other direction (1 – LX) motion vector, the bilateral matching cost is calculated using the merge candidate MV and the AMVP MV. The merge candidate with the smallest cost is selected. The bilateral matching refinement is applied to the coding block with the selected merge candidate MV and the AMVP MV as a starting point.

The proposed mode is indicated by a flag, if the mode is enabled AMVP direction LX is further indicated by a flag.

In the proposed AMVP-merge mode, the values of AMVR and BCW parameters set to the default values IMV_OFF and BCW_DEFAULT, respectively, and the mode is not enabled together with affine and SMVD.

# Simulation results

The proposed method was implemented on top of ECM-1.0. The performance was evaluated using the common test conditions as defined in [2]. Two test results are reported:

- Test 1: results over ECM-1.0

- Test 2: results over ECM-1.0 with template matching disabled (both anchor and test set macros TM_MRG and TM_AMVP to 0)

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  | Over ECM-1.0 TM OFF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.02% | -0.19% | 106% | 99% | -0.36% | -0.21% | -0.27% | 110% | 100% |
| Class A2 | -0.10% | -0.12% | -0.06% | 106% | 99% | -0.50% | -0.51% | -0.49% | 109% | 99% |
| Class B | -0.11% | -0.02% | -0.04% | 106% | 99% | -0.46% | -0.41% | -0.32% | 110% | 100% |
| Class C | -0.14% | -0.07% | -0.22% | 106% | 99% | -0.56% | -0.58% | -0.46% | 110% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.11% | -0.05% | -0.12% | 106% | 99% | -0.48% | -0.44% | -0.38% | 110% | 100% |
| Class D | -0.11% | -0.11% | 0.06% | 106% | 99% | -0.65% | -0.69% | -0.91% | 110% | 100% |
| Class F | -0.08% | 0.00% | 0.02% | 105% | 100% | -0.26% | -0.31% | -0.25% | 109% | 101% |

# Conclusion

In this contribution, bilateral matching AMVP-merge mode is proposed, the gain of the method is higher when template matching is disabled. It is suggested to study the proposed method in the next EE.

# References

[1] ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).