| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0109-v2 |
| --- | --- |

| Title: | AHG8: Removal of a prevention mechanism of extended precision for low bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

In the latest WD for VVC ver.2, there is a prevention mechanism to deactivate extended precision even when sps-level enabled flag (extended_precision_processing_flag) is set to 1. This prevention mechanism loses an encoder flexibility to use extended precision for low bit-depth. In addition, extended precision provides minor gain (roughly -0.1% for AI, -0.1% for RA and -0.1% for LB) for YUV420 10-bit sequences at low QP. This contribution proposes to remove its prevention mechanism, which gives encoders flexibility to use extended precision for low bit-depth (proposal#1).

If necessary to disable extended precision for low bit-depth coding, rather than having a prevention mechanism, this contribution suggests to

- impose a bitstream constraint which indicates such a tool shall be disabled for bit-depth less than or equal to 10 in the profile like HEVC or semantics (proposal#2), or

- omit signaling of extended_precision_processing_flag depending on BitDepth to avoid unnecessary signaling (proposal#3).

A patch for disabling a prevention mechanism of extended precision relative to VTM-13.0 is attached.

In v2, the results were updated.

Introduction

At the last JVET-V meeting, several tools were adopted to improve coding performance for high bit-depth coding as below:

7.3.2.22 Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

Current WD [1] has a prevention mechanism to deactivate extended transform precision for bit-depth less than or equal to 1 even when its enabled flag is set to 1 as shown in Table 1 (related semantics are highlighted in green). This prevention mechanism loses an encoder flexibility to use extended precision for low bit-depth coding.

Table 1: Semantic of Extended precision

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

The variable Log2TransformRange is derived as follows:

Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min( 20, BitDepth + 6 ) ) : 15	(X)

To evaluate coding performance of operation range extension tools for low bit-depth, following two tests were conducted for YUV420 10-bit common test sequences at normal QPs (QP=22, 27, 32, 37) and low QPs (QP=2, 7, 12, 17). Note that first 100 frames were used for low QP test.

Test#1: VTM-13.0 vs VTM-13.0 w/ operation range extension tools including a prevention mechanism of extended precision

- Anchor: VTM-13.0 with a macro JVET_R0351_HIGH_BIT_DEPTH_ENABLED equal to 0

- Test: VTM-13.0 with a macro JVET_R0351_HIGH_BIT_DEPTH_ENABLED equal to 1

  - Encoder options for Test: --ExtendedPrecision=1 --TSRCRicePresent=1 --ExtendedRiceRRC=1 --GolombRiceParameterAdaptation=1 --Profile=none

Test#2: VTM-13.0 vs VTM-13.0 w/ operation range extension tools and excluding a prevention mechanism of extended precision

- Anchor: VTM-13.0 with a macro JVET_R0351_HIGH_BIT_DEPTH_ENABLED equal to 0

- Tested: VTM-13.0 with a macro JVET_R0351_HIGH_BIT_DEPTH_ENABLED equal to 1

  - Encoder options for Test: --ExtendedPrecision=1 --TSRCRicePresent=1 --ExtendedRiceRRC=1 --GolombRiceParameterAdaptation=1 --Profile=none

  - Enforcedly extended precision is enabled by removing its prevision mechanism.

Table 1 shows the summary of each test for AI, RA and LB, respectively. For more details, please refer to Table 2 through Table 5. Supplementally, Table 6 and Table 7 shows the coding performance of Test#2 Tested over Test#1 Tested at normal QPs and low QPs, respectively.

As can be seen, at low QPs extended precision provides minor gain (roughly -0.1% for AI, -0.1% for RA and -0.1% for LB) while less gain (roughly 0.0% for AI, 0.0% for RA and -0.0%) at normal QPs.

Table 1: Summary results of coding performance of each Test

| QP | Test | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| normal QP | Test#1 | -0.01% | -0.07% | -0.04% | 106% | 107% |
|  | Test#2 | -0.02% | -0.08% | -0.03% | 104% | 104% |
|  | delta = Test#2 - Test#1 | 0.00% | -0.01% | 0.02% | -2% | -3% |
| low QP | Test#1 | -0.28% | -0.32% | -0.33% | 109% | 105% |
|  | Test#2 | -0.37% | -0.48% | -0.50% | 107% | 102% |
|  | delta = Test#2 - Test#1 | -0.09% | -0.16% | -0.17% | -2% | -2% |

| QP | Test | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| normal QP | Test#1 | 0.02% | -0.12% | -0.10% | 108% | 111% |
|  | Test#2 | 0.02% | -0.05% | -0.10% | 106% | 108% |
|  | delta = Test#2 - Test#1 | 0.00% | 0.06% | 0.00% | -2% | -3% |
| low QP | Test#1 | -0.05% | -0.07% | -0.07% | 107% | 111% |
|  | Test#2 | -0.12% | -0.29% | -0.26% | 107% | 107% |
|  | delta = Test#2 - Test#1 | -0.08% | -0.22% | -0.20% | 0% | -4% |

| QP | Test | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| normal QP | Test#1 | 0.02% | 0.07% | 0.47% | 104% | 104% |
|  | Test#2 | -0.01% | 0.03% | 0.12% | 103% | 105% |
|  | delta = Test#2 - Test#1 | -0.04% | -0.04% | -0.35% | -1% | 1% |
| low QP | Test#1 | -0.01% | -0.05% | -0.05% | 104% | 105% |
|  | Test#2 | -0.09% | -0.26% | -0.27% | 102% | 105% |
|  | delta = Test#2 - Test#1 | -0.08% | -0.21% | -0.22% | -2% | 0% |

Test#1

Table 2: Coding performance of operation range extension tools including a prevention mechanism of extended precision over VTM-13.0 at normal QPs (Test#1 Normal QP)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.07% | 0.05% | 104% | 110% |
| Class A2 | 0.01% | 0.01% | -0.01% | 106% | 112% |
| Class B | -0.01% | -0.03% | -0.04% | 102% | 102% |
| Class C | 0.00% | -0.10% | -0.08% | 111% | 108% |
| Class E | -0.04% | -0.21% | -0.13% | 110% | 106% |
| Overall | -0.01% | -0.07% | -0.04% | 106% | 107% |
| Class D | 0.03% | -0.16% | 0.04% | 113% | 115% |
| Class F | -0.01% | -0.10% | -0.05% | 102% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.21% | -0.07% | 107% | 116% |
| Class A2 | 0.02% | 0.04% | -0.05% | 111% | 118% |
| Class B | 0.00% | -0.17% | -0.08% | 103% | 100% |
| Class C | 0.07% | -0.10% | -0.17% | 112% | 116% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.12% | -0.10% | 108% | 111% |
| Class D | 0.08% | -0.08% | 0.29% | 110% | 120% |
| Class F | -0.04% | -0.08% | 0.03% | 104% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.18% | 0.37% | 106% | 117% |
| Class C | 0.01% | -0.26% | 0.18% | 103% | 99% |
| Class E | 0.06% | 0.92% | 1.02% | 102% | 91% |
| Overall | 0.02% | 0.07% | 0.47% | 104% | 104% |
| Class D | 0.16% | 0.01% | 0.32% | 113% | 119% |
| Class F | 0.04% | 0.39% | -0.08% | 102% | 98% |

Table 3: Coding performance of operation range extension tools including a prevention mechanism of extended precision over VTM-13.0 at low QPs (Test#1 Low QP)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | -0.10% | 107% | 104% |
| Class A2 | -0.27% | -0.26% | -0.25% | 111% | 106% |
| Class B | -0.23% | -0.26% | -0.26% | 103% | 97% |
| Class C | -0.51% | -0.49% | -0.52% | 115% | 109% |
| Class E | -0.31% | -0.45% | -0.48% | 113% | 110% |
| Overall | -0.28% | -0.32% | -0.33% | 109% | 105% |
| Class D | -0.68% | -0.77% | -0.80% | 115% | 108% |
| Class F | -0.28% | -0.38% | -0.42% | 111% | 108% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.05% | 109% | 114% |
| Class A2 | -0.03% | -0.06% | -0.07% | 110% | 118% |
| Class B | -0.03% | -0.06% | -0.07% | 104% | 112% |
| Class C | -0.11% | -0.11% | -0.07% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.07% | -0.07% | 107% | 111% |
| Class D | -0.15% | -0.10% | -0.23% | 111% | 115% |
| Class F | -0.46% | -0.34% | -0.24% | 107% | 107% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.05% | -0.04% | 103% | 112% |
| Class C | -0.04% | -0.01% | -0.01% | 104% | 97% |
| Class E | 0.01% | -0.10% | -0.12% | 105% | 105% |
| Overall | -0.01% | -0.05% | -0.05% | 104% | 105% |
| Class D | -0.05% | -0.02% | -0.07% | 107% | 110% |
| Class F | -0.18% | -0.18% | -0.27% | 106% | 104% |

Test#2

Table 4: Coding performance of operation range extension tools and excluding a prevention mechanism of extended precision over VTM-13.0 at normal QPs (Test#2 Normal QP)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.15% | -0.02% | 104% | 111% |
| Class A2 | 0.00% | 0.04% | -0.01% | 104% | 111% |
| Class B | -0.02% | -0.04% | 0.00% | 102% | 101% |
| Class C | -0.01% | -0.03% | 0.07% | 108% | 100% |
| Class E | -0.02% | -0.28% | -0.23% | 104% | 99% |
| Overall | -0.02% | -0.08% | -0.03% | 104% | 104% |
| Class D | 0.04% | -0.19% | 0.02% | 113% | 117% |
| Class F | 0.00% | -0.12% | -0.03% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.10% | 107% | 117% |
| Class A2 | 0.01% | 0.01% | -0.08% | 110% | 123% |
| Class B | 0.00% | -0.10% | -0.04% | 103% | 100% |
| Class C | 0.07% | -0.04% | -0.18% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.10% | 106% | 108% |
| Class D | 0.09% | -0.07% | -0.11% | 112% | 123% |
| Class F | -0.01% | -0.09% | -0.03% | 100% | 92% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.45% | 107% | 121% |
| Class C | 0.03% | -0.16% | -0.08% | 101% | 100% |
| Class E | -0.08% | 0.32% | -0.18% | 98% | 88% |
| Overall | -0.01% | 0.03% | 0.12% | 103% | 105% |
| Class D | 0.15% | 0.53% | -0.48% | 107% | 108% |
| Class F | -0.02% | 0.92% | -0.04% | 101% | 98% |

Table 5: Coding performance of operation range extension tools and excluding a prevention mechanism of extended precision over VTM-13.0 at normal QPs (Test#2 Low QP)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.39% | -0.40% | 108% | 106% |
| Class A2 | -0.48% | -0.42% | -0.42% | 120% | 117% |
| Class B | -0.32% | -0.44% | -0.44% | 101% | 95% |
| Class C | -0.55% | -0.54% | -0.59% | 111% | 102% |
| Class E | -0.38% | -0.61% | -0.63% | 101% | 97% |
| Overall | -0.37% | -0.48% | -0.50% | 107% | 102% |
| Class D | -0.74% | -0.77% | -0.78% | 117% | 110% |
| Class F | -0.31% | -0.41% | -0.42% | 102% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.46% | -0.41% | 114% | 112% |
| Class A2 | -0.15% | -0.24% | -0.23% | 111% | 108% |
| Class B | -0.12% | -0.26% | -0.25% | 104% | 111% |
| Class C | -0.16% | -0.23% | -0.20% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.29% | -0.26% | 107% | 107% |
| Class D | -0.20% | -0.19% | -0.23% | 112% | 112% |
| Class F | -0.50% | -0.39% | -0.28% | 102% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.23% | -0.21% | 104% | 114% |
| Class C | -0.10% | -0.11% | -0.13% | 99% | 96% |
| Class E | -0.09% | -0.51% | -0.54% | 102% | 105% |
| Overall | -0.09% | -0.26% | -0.27% | 102% | 105% |
| Class D | -0.11% | -0.06% | -0.08% | 109% | 107% |
| Class F | -0.19% | -0.30% | -0.28% | 101% | 99% |

Supplement: Test2# Tested over Test#1 Tested

Table 6: Coding performance of Test#2 Tested over Test#1 Tested at normal QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.07% | 100% | 101% |
| Class A2 | -0.01% | 0.02% | -0.01% | 98% | 100% |
| Class B | 0.00% | -0.01% | 0.04% | 100% | 100% |
| Class C | -0.01% | 0.07% | 0.15% | 97% | 93% |
| Class E | 0.02% | -0.06% | -0.10% | 94% | 94% |
| Overall | 0.00% | -0.01% | 0.02% | 98% | 97% |
| Class D | 0.01% | -0.03% | -0.02% | 100% | 102% |
| Class F | 0.00% | -0.02% | 0.02% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | -0.03% | 100% | 101% |
| Class A2 | 0.00% | -0.03% | -0.03% | 99% | 104% |
| Class B | 0.00% | 0.07% | 0.04% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.00% | 94% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.00% | 98% | 98% |
| Class D | 0.01% | 0.01% | -0.39% | 101% | 103% |
| Class F | 0.03% | -0.01% | -0.06% | 96% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.18% | 0.08% | 101% | 103% |
| Class C | 0.03% | 0.11% | -0.26% | 98% | 101% |
| Class E | -0.14% | -0.59% | -1.19% | 97% | 97% |
| Overall | -0.04% | -0.04% | -0.35% | 99% | 101% |
| Class D | -0.01% | 0.52% | -0.80% | 95% | 91% |
| Class F | -0.07% | 0.53% | 0.04% | 98% | 100% |

Table 7: Coding performance of Test#2 Tested over Test#1 Tested at low QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.29% | -0.30% | 101% | 102% |
| Class A2 | -0.22% | -0.16% | -0.17% | 108% | 110% |
| Class B | -0.08% | -0.18% | -0.18% | 98% | 98% |
| Class C | -0.04% | -0.05% | -0.08% | 96% | 93% |
| Class E | -0.07% | -0.16% | -0.16% | 90% | 88% |
| Overall | -0.10% | -0.16% | -0.17% | 98% | 98% |
| Class D | -0.06% | -0.01% | 0.02% | 101% | 102% |
| Class F | -0.03% | -0.03% | 0.00% | 92% | 91% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.43% | -0.36% | 105% | 98% |
| Class A2 | -0.11% | -0.17% | -0.17% | 102% | 91% |
| Class B | -0.09% | -0.20% | -0.18% | 100% | 99% |
| Class C | -0.05% | -0.13% | -0.12% | 94% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.22% | -0.20% | 100% | 96% |
| Class D | -0.06% | -0.09% | 0.01% | 100% | 97% |
| Class F | -0.04% | -0.05% | -0.04% | 95% | 92% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test#1 Tested |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.18% | -0.17% | 100% | 102% |
| Class C | -0.06% | -0.11% | -0.12% | 95% | 99% |
| Class E | -0.11% | -0.41% | -0.42% | 97% | 99% |
| Overall | -0.08% | -0.21% | -0.22% | 98% | 100% |
| Class D | -0.06% | -0.04% | 0.00% | 102% | 97% |
| Class F | 0.00% | -0.11% | -0.01% | 95% | 95% |

Proposal

Since extended precision provides minor gain for YUV420 10-bit sequences at low QP, this contribution proposes to remove its prevention mechanism (proposal#1). It gives encoders flexibility to use extended precision for low bit-depth.

If necessary to disable extended precision for low bit-depth coding, rather than having a prevention mechanism, it is suggested to

- impose a bitstream constraint which indicates such a tool shall be disabled for bit-depth less than or equal to 10 in the profile like HEVC or semantics (proposal#2), or

- omit signaling of extended_precision_processing_flag depending on BitDepth to avoid unnecessary signaling (proposal#3).

Removal of a prevention mechanism of extended precision (proposal#1)

It is proposed to remove a prevention mechanism of extended precision. Proposed changes relative to JVET-V2005 are as below:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range is may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is set equal to extended_precision_processing_flag. derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

The variable Log2TransformRange is derived as follows:

Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min( 20, BitDepth + 6 ) ) : 15	(X)

Bitstream constraint (Proposal#2)

If necessary to disable extended precision for low bit-depth coding, it is suggested to impose a following bitstream constraint which indicates such a tool shall be disabled for bit-depth less than or equal to 10 in a profile definition like HEVC or its semantic on top of proposal#1.

[Case#1: Constraint in the semantic]

It is a requirement of bitstream conformance that the value of extended_precision_processing_flag shall be equal to 0 when BitDepth is less than or equal to 10.

[Case#2: Constraint in the profiles]

Bitstreams conforming to the “Operation Range Extension profiles” shall obey the following constraints:

– In bitstreams conforming to the “Operation Range Extension low bit-depth profile” (e.g. Main 10 Intra, Main 4:2:2 Intra or Main 4:4:4 10 Intra profiles in HEVC), referenced SPSs shall have extended_precision_processing_flag equal to 0.

– In bitstreams conforming to the “Operation Range Extension high bit-depth profile” (e.g. Main 12, Main 4:2:2 12, Main 4:4:4 12, or Main 4:4:4 16 Intra profiles in HEVC), referenced SPSs shall have extended_precision_processing_flag equal to 1 (or 0…1).

Omit signaling of sps enabled flag for low bit-depth (Proposal#3)

Alternatively, it is suggested to omit signaling of extended_precision_processing_flag depending on BitDepth. The benefit compared to proposal#2-1 is to avoid unnecessary signaling.

Proposed changes relative to JVET-V2005 are as below:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| if (BitDepth>10) |  |
| extended_precision_processing_flag | u(1) |
| } |  |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range is may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is set equal to extended_precision_processing_flag. derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

The variable Log2TransformRange is derived as follows:

Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min( 20, BitDepth + 6 ) ) : 15	(X)

Conclusion

Since extended precision provides minor gain for YUV420 10-bit sequences at low QP, it is proposed to remove the prevention mechanism of extended precision (proposal#1). It is recommended to adopt this cleanup for the next VTM.

If necessary to disable extended precision for low bit-depth coding, rather than having a prevention mechanism, it is suggested to

- impose a bitstream constraint which indicates such a tool shall be disabled for bit-depth less than or equal to 10 in a profile like HEVC or its semantic (proposal#2), or

- omit signaling extended_precision_processing_flag depending on BitDepth to avoid unnecessary signaling (proposal#3).

Reference

- F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, Y.-K. Wang, “VVC operation range extensions (Draft 3)”, JVET-V2005, Apr. 2021

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Nov. 2020.

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).