| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0110 |
| --- | --- |

| Title: | AHG12: GPM with inter and intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes geometric partitioning mode (GPM) with inter and intra prediction to enhance the coding performance beyond VVC. With the method, pre-defined intra prediction modes against geometric partitioning line can be selected in addition to motion vectors from a merge candidate list for each non-rectangular split region in the GPM-applied coding unit.

On top of the VTM-11.0+MCTF and ECM-1.0 which are defined as the anchor for tool-on and tool-off tests in EE2 of JVET-V2024, simulation results of the proposed method are reported as follows:

- Tool-on test (VTM-11.0+MCTF as the anchor):

  - RA: BD-rate YUV: -0.18%, -0.51%, -0.36%; EncT: 102%; DecT: 101%

  - LB: BD-rate YUV: -0.43%, -1.00%, -1.00%; EncT: 103%; DecT: 106%

  - LP: BD-rate YUV: -1.01%, -1.84%, -1.81%; EncT: 111%; DecT: 101%

- Tool-off test (ECM-1.0 as the anchor):

  - RA: BD-rate YUV: -0.04%, -0.29%, -0.18%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.20%, -0.87%, -0.48%; EncT: 101%; DecT: 100%

  - LP: BD-rate YUV: %, %, %; EncT: %; DecT: %

Introduction

In the current GPM of VVC, the prediction signal of one coding unit (CU) is generated from the sample-wise weighted combination of two uni-directional inter prediction signals for non-rectangular split regions [1]. The couple of uni-directional motion vectors (MVs) utilized to generate the two uni-prediction signals are obtained from a merge candidate list. However, GPM has room to improve coding performance if the two uni-directional prediction signals can be generated from not only inter prediction pixels with the MVs but also intra prediction pixels since it can directly fetch the prediction signals from the neighboring reference pixels against the geometric partitioning line.

Proposal

In this contribution, GPM with inter and intra prediction is proposed. With the method, pre-defined intra prediction modes against geometric partitioning line can be selected in addition to merge candidates for each non-rectangular split region in the GPM-applied CU. In the proposed method, whether intra or inter prediction mode is determined for each GPM-separated region with a flag from the encoder. When the inter prediction mode, a uni-prediction signal is generated by MVs from the merge candidate list. On the other hand, when the intra prediction mode, a uni-prediction signal is generated from the neighboring pixels for the intra prediction mode specified by an index from the encoder. The variation of the possible intra prediction modes is restricted by the geometric shapes. Finally, the two uni-prediction signals are blended with the same way of ordinary GPM.

Simulation Results

The proposed method was implemented on the top of VTM-11.0+MCTF [2] and ECM-1.0 [3], which are defined as the anchor software in EE2 for tool-on and tool-off tests [4]. Simulation conditions are followed by the common test condition (CTC) [5]. In the simulations, the following two tests are conducted to study the effect from the variation of possible intra prediction modes.

- Test 1: Parallel and perpendicular modes against the geometric partitioning line.

- Test 2: Test 1 plus Planar mode.

Tables 1 and 2 show the Test 1 results over VTM-11.0+MCTF and ECM-1.0.

Table 2: Simulation results of Test 1 over VTM-11.0+MCTF.

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.07% | -0.37% | -0.28% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.06% | -0.15% | -0.21% | 102% | 101% |  |  |  |  |  |  |  |  |  |  |
| B | -0.12% | -0.53% | -0.34% | 101% | 101% | -0.37% | -1.03% | -1.05% | 103% | 104% | -0.74% | -1.83% | -1.42% | 109% | 101% |
| C | -0.33% | -0.61% | -0.40% | 101% | 102% | -0.57% | -0.86% | -1.31% | 103% | 107% | -1.39% | -2.87% | -2.85% | 110% | 101% |
| E |  |  |  |  |  | -0.06% | -0.57% | -0.64% | 103% | 107% | -0.54% | -0.47% | -0.87% | 108% | 103% |
| Overall | -0.15% | -0.45% | -0.32% | 101% | 101% | -0.36% | -0.86% | -1.04% | 103% | 106% | -0.91% | -1.84% | -1.76% | 109% | 101% |
| D | -0.19% | -0.46% | -0.36% | 102% | 101% | -0.43% | -1.27% | -1.51% | 102% | 104% | -0.80% | -3.40% | -1.55% | 109% | 101% |
| F | -0.12% | -0.31% | -0.37% | 100% | 101% | -0.42% | -0.42% | -0.74% | 102% | 107% | -0.60% | -0.75% | -1.29% | 106% | 102% |

Table 2: Simulation results of Test 1 over ECM-1.0.

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.04% | -0.39% | -0.10% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.06% | -0.18% | -0.06% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.04% | -0.31% | -0.35% | 101% | 100% | -0.13% | -0.36% | -0.07% | 100% | 100% | % | % | % | % | % |
| C | -0.07% | -0.26% | -0.29% | 101% | 100% | -0.19% | -1.01% | -0.82% | 100% | 99% | % | % | % | % | % |
| E |  |  |  |  |  | -0.12% | -0.47% | -0.32% | 101% | 101% | % | % | % | % | % |
| Overall | -0.05% | -0.29% | -0.23% | 101% | 100% | -0.15% | -0.60% | -0.39% | 101% | 100% | % | % | % | % | % |
| D | -0.04% | -0.23% | 0.00% | 101% | 101% | -0.02% | -0.21% | -0.32% | 100% | 101% | % | % | % | % | % |
| F | -0.01% | -0.16% | -0.04% | 100% | 101% | 0.00% | -0.19% | 0.33% | 100% | 101% | % | % | % | % | % |

Tables 3 and 4 show the Test 2 results over VTM-11.0+MCTF and ECM-1.0.

Table 2: Simulation results of Test 2 over VTM-11.0+MCTF.

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.05% | -0.57% | -0.24% | 102% | 101% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.09% | -0.27% | -0.21% | 102% | 101% |  |  |  |  |  |  |  |  |  |  |
| B | -0.15% | -0.55% | -0.50% | 102% | 101% | -0.41% | -1.27% | -1.11% | 103% | 104% | -0.82% | -1.78% | -1.48% | 110% | 101% |
| C | -0.38% | -0.58% | -0.41% | 102% | 102% | -0.65% | -0.92% | -1.59% | 104% | 107% | -1.57% | -2.73% | -2.56% | 112% | 101% |
| E |  |  |  |  |  | -0.19% | -0.65% | -0.02% | 103% | 108% | -0.59% | -0.74% | -1.38% | 110% | 102% |
| Overall | -0.18% | -0.51% | -0.36% | 102% | 101% | -0.43% | -1.00% | -1.00% | 103% | 106% | -1.01% | -1.84% | -1.81% | 111% | 101% |
| D | -0.21% | -0.37% | -0.43% | 102% | 101% | -0.35% | -1.40% | -1.13% | 103% | 106% | -0.94% | -2.91% | -1.48% | 109% | 101% |
| F | -0.15% | -0.40% | -0.36% | 101% | 101% | -0.46% | -0.51% | -0.47% | 103% | 107% | -0.69% | -1.09% | -1.18% | 108% | 102% |

Table 2: Simulation results of Test 2 over ECM-1.0.

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.03% | -0.32% | -0.11% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.01% | -0.13% | -0.03% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.02% | -0.33% | -0.31% | 102% | 101% | -0.15% | -0.85% | -0.41% | 101% | 100% | % | % | % | % | % |
| C | -0.10% | -0.33% | -0.18% | 101% | 101% | -0.28% | -1.17% | -0.80% | 101% | 100% | % | % | % | % | % |
| E |  |  |  |  |  | -0.19% | -0.50% | -0.16% | 103% | 101% | % | % | % | % | % |
| Overall | -0.04% | -0.29% | -0.18% | 101% | 100% | -0.20% | -0.87% | -0.48% | 101% | 100% | % | % | % | % | % |
| D | -0.05% | 0.08% | 0.00% | 101% | 100% | -0.08% | -0.23% | -1.00% | 101% | 100% | % | % | % | % | % |
| F | 0.01% | 0.00% | 0.05% | 101% | 101% | 0.09% | -0.02% | -0.02% | 101% | 101% | % | % | % | % | % |

Conclusion

In this contribution, GPM with intra prediction was proposed to improve coding efficiency beyond VVC. Simulation results reportedly show the proposed methods provide potentially a -0.18%/-0.43%/-1.01% and -0.04%/-0.20%/% BD-rate savings of luma component with almost no increment encoding and decoding runtime over VTM-11.0+MCTF and ECM-1.0 in the RA/LB/LP conditions. Therefore, we recommend it will be included in the next EE2.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Recommendation ITU-T H.266, "Versatile video coding," in Edition 1.0, August 2020.

- VTM-11.0+MCTF: https://vcgit.hhi.fraunhofer.de/chollmann/VVCSoftware_VTM/-/tree/MCTF_VTM11

- ECM-1.0: https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-V2024, by teleconference, 20–28 Apr. 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).