| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0111-v1 |
| --- | --- |

| Title: | AHG11: neural network based cross-component prediction model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang, Sheng Lin, Renjie Chang, Xiaozhong Xu and Shan Liu | Tel: Email: | {liqiangwang, barneylin, xiaozhongxu} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution presents a neural network based cross-component prediction (NNCCP) model. As an alternative mode of intra chroma prediction, NNCCP can further reduce the cross-component redundancy.  Based on the AHG11 anchor (VTM11+V0056), it is reported that -0.34%, -0.78% and -0.35% BD-rate savings are achieved for Y, U and V components with the AI configuration, respectively.

# Introduction

Cross-component linear model prediction (CCLM) is adopted in VVC. The chroma samples are predicted based on the reconstructed luma samples by using a linear model, which is built by the neighbouring samples. In this contribution, an alternative model is built, but by the neural network.

# Proposed method

In this contribution, the neighbouring reconstructed samples and the reconstructed luma samples in the co-located area of the current block are fed into the network. Then the predcition samples of two chroma blocks are generated. The output of NNCCP is processd similar to other intra chorma predition modes, and the best mode is decided based on the RDO cost.

## Network architecture

The pre-processing unit takes the YUV 420 formatted input patches. In the Fig. 1, the pre-processing unit takes YUV and neighboring reconstructed samples as the input. The luma component down-sampling is performed by the Bicubic interpolation filter. The height/width of neighboring samples is decided by n, where n is set to 4 in our pre-processing unit. The neighboring samples (n, H) or (W, n) is transformed to the size (W, H). The transformation is performed by the W/n or H/n times duplication. The luma channel provides a (W, H, 3) patch, both U and V provide (W, H, 2) patch. Then all are concatenated into a (W, H, 7) as the input for neural network.

Fig. 1: Pre-processing unit

Fig. 2 shows the network structure of cross-component prediction models. The first and last layers are regular convolutional layers with kernel size 3. The skip layer is a regular convolutional layer with kernel size 5. The structure of residual block (ResBlock) consists of 3 regular convolutional layers, where N denotes the number of ResBlock. N is set to 8 in this contribution. In the ResBlock, the first layer is a 1x1 convolutional layer followed by a ReLU activation function, the second layer is a 1x1 convolutional layer and the third layer is 3x3 convolutional layer. As for the internal convolutional layers, the number of feature maps is set as 32.

Fig. 2: Network architecture

## Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: V100 |
|  | Framework: | PyTorch v1.5 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 500 |
|  | Batch size: | 16 |
|  | Loss function: | L1 |
|  | Training time: | 50h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 32*32 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | PyTorch v1.5 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 150122 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 0.600488 |
|  | Multiplay Accumulate (MAC) | 153724928 (32* 32 pixels) |
| Optional |  |  |
|  | Total Conv. Layers | 27 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

The anchor is VTM11+V0056 [][], and the 2s results are provided. Two tests are launched as follows:

Test 1: NNCCP is only enabled for 8x8, 16x16 and 32x32 chroma blocks.

Test 2: NNCCP is enabled for all chroma blocks, which is same as CCLM.

Table 3. Performance of the proposed method (AI, Test 1)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.41% | -0.98% | -0.56% | -0.65% | -1.07% | -1.02% | 382% | 2823% |
| Class A2 | -0.41% | -1.36% | 0.06% | -0.46% | -1.62% | 0.04% | 443% | 4822% |
| Class B | -0.34% | -0.49% | -0.49% | -0.45% | -0.91% | -0.33% | 318% | 4133% |
| Class C | -0.29% | -0.41% | 0.26% | -0.45% | -0.80% | 0.54% | 395% | 4911% |
| Class E | -0.27% | -0.96% | -1.11% | -0.41% | -1.04% | -0.58% | 308% | 3690% |
| Overall | -0.34% | -0.78% | -0.35% | -0.48% | -1.05% | -0.23% | 362% | 4058% |
| Class D | -0.24% | -0.96% | -0.72% | -0.30% | -2.57% | -0.52% | 360% | 5748% |
| Class F | -0.35% | -0.09% | -0.43% | -0.32% | -0.97% | -0.56% | 235% | 3007% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4. Performance of the proposed method (AI, Test 2)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.38% | -1.28% | -0.88% | -0.52% | -1.14% | -1.18% | 948% | 4446% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.38% | -1.65% | -1.47% | -0.45% | -2.34% | -1.70% | 1082% | 7506% |
| Class C | -0.33% | -1.51% | -0.47% | -0.47% | -2.80% | -1.02% | 1879% | 10713% |
| Class E | -0.36% | -1.46% | -2.20% | -0.57% | -1.76% | -2.08% | 905% | 5833% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.34% | -1.73% | -1.57% | -0.52% | -3.92% | -2.53% | 1594% | 12779% |
| Class F | -0.35% | -0.70% | -0.36% | -0.26% | -3.15% | -1.41% | 821% | 6822% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution presents a neural network based cross-component prediction model. By applying the proposed method, additional BD rate saving can be achieved.

# References

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-V2016, 22nd Meeting, by teleconference, 20–28 Apr. 2021.